IEEE P802.11
Wireless LANs

| LB271 CR for 11be D3.0 Section 36.3.12.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.03.14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following CIDs on P802.11be D3.0:

15720, 16347, 16362, 17222, 17919

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 15720

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.60 | 36.3.12.11.1 | For an EHT TB PPDU, the information on preamble puncturing is not described. | Add information on preamble puncturing of EHT TB PPDU | REVISED Instructions to the Editor: Place the following text at the end of section 36.3.12.11.1 (823.60): “There is no indication about the preamble puncturing in an EHT TB PPDU” Remove the text “There is no indication about preamble puncturing in the EHT TB PPDU” from 36.3.12.11.4 |

## CID 16347

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 824.11 | 36.3.12.11.1 | The sub-clause 36.3.12.11 does not contain any description about restricting subchannels that are not indicated in the Disabled Subchannel Bitmap from being punctured. Such a description exists in the corresponding MAC sub-clause on preamble puncturing so it seems reasonable to at least have a reference for it here. | (Updated in the edit notes in the comments collection) Add the following text at the end of subclause 36.3.12.11.1: "Puncturing of subchannels that are not indicated in the Disabled Subchannel Bitmap subfield in 9.4.2.311 shall follow the restrictions described in 35.15.2 (Preamble puncturing operation)" | ACCEPTED Note to the Editor: Same resolution for the following CIDs: 16347, 16362 |

## CID 16362

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.46 | 36.3.12.11.1 | It is beneficial to include a reference to 35.15.2 in 36.3.12.11.1 or 36.3.12.11.2, so that the reader understands where more restrictions on preamble puncturing in an EHT MU PPDU are defined. | Add reference to Clause 35.15.2 in either 36.3.12.11.1 or 36.3.12.11.2, and explain that the restictions on the use of preamble puncturing in an EHT MU PPDU are defined in that clause (35.15.2). | REVISED Note to the Editor: Please apply the resolution of CID 16347 Same resolution for the following CIDs: 16347, 16362 |

## CID 17222

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.62 | 36.3.12.11.2 | The paragraph starting at line 62 talks about OFDMA and non-OFDMA specific specifications. Sections 36.3.12.11.2 and 36.3.12.11.5 are about preamble puncturing for OFDMA and non-OFDMA sepcifically. It may make more sense to move the requirements to these respective sections, rather than having them in the "General" section 36.3.12.11.1. | Move OFDMA and non-OFDMA specific requirements to 36.3.12.11.2 and 36.3.12.11.3 respectively. | REVISED Instructions to the Editor: - Add the following text after the first paragraph of 36.3.12.11.2 (824.17): "The preamble puncturing resolution shall be 20 MHz for an EHT MU PPDU using OFDMA transmission for a bandwidth larger than 40 MHz. In other words, puncturing a subchannel smaller than a 242-tone RU is not allowed in PPDU bandwidths larger than 40 MHz." - Add the following text to the beginning of 36.3.12.11.3 (824.40): “The preamble puncturing resolution shall be 20 MHz for an EHT MU PPDU with 80 MHz and 160 MHz PPDU bandwidths. In other words, puncturing a subchannel smaller than a 242-tone RU is not allowed in 80 MHz and 160 MHz PPDU bandwidths. The preamble puncturing resolution shall be 40 MHz for an EHT MU PPDU with a 320 MHz PPDU bandwidth. In other words, puncturing a subchannel smaller than a 484-tone RU is not allowed in a 320 MHz PPDU bandwidth” - Remove the paragraph starting in 823.62 and ending in 824.4 |

## CID 17919

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 824.26 | 36.3.12.11.2 | A period is missing at the end of this sentence. | As in comment. | ACCEPTED |