IEEE P802.11
Wireless LANs

| LB271 CR for 11be D3.0 Section 36.3.12.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.03.14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolutions to the following CIDs on P802.11be D3.0:

15720, 16347, 16362, 17222, 17919

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Added text to show the changes |

## CID 15720

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.60 | 36.3.12.11.1 | For an EHT TB PPDU, the information on preamble puncturing is not described. | Add information on preamble puncturing of EHT TB PPDU | REVISED Agree with the commenter. Move a line from 36.3.12.11.4 to 36.3.12.11.1 Instructions to the Editor: Please follow the changes indicated in https://mentor.ieee.org/802.11/dcn/23/11-23-0434-01-00be-lb271-cr-for-p802-11be-d3-0-section-36-3-12-11.doc |

Instructions to the Editor:

Place the following text at the end of section 36.3.12.11.1 (823.60):

36.3.12.11.1 General

Preamble puncturing refers to the transmission of a PPDU in which no signal is present in at least one of the 20 MHz subchannels within the PPDU bandwidth.

Preamble puncturing might be the result of the unavailability of 20 MHz subchannel(s) within the PPDU bandwidth, such as a busy channel indicated by the CCA or the setting of the Disabled Subchannel Bitmap subfield in the EHT Operations element (see 9.4.2.311 (EHT Operation element)).

Preamble puncturing may exist in an EHT MU PPDU transmitted in the DL or the UL and in an EHT TB PPDU transmitted by a non-AP STA in the UL. For an EHT MU PPDU, the U-SIG and the EHT-SIG fields include information on preamble puncturing. There is no indication about the preamble puncturing in an EHT TB PPDU.

Instructions to the Editor:

Remove the text “There is no indication about preamble puncturing in the EHT TB PPDU” from 36.3.12.11.4

36.3.12.11.4 Preamble puncturing for EHT TB PPDUs

A non-AP EHT STA transmitting an EHT TB PPDU occupies the 20 MHz subchannels that are overlapped with its assigned RU or MRU and has no knowledge on whether the unassigned 20 MHz subchannels are due to puncturing or not, unless indicated as inactive in the Disabled Subchannel Bitmap subfield in 9.4.2.311 (EHT Operation element).

## CID 16347

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 824.11 | 36.3.12.11.1 | The sub-clause 36.3.12.11 does not contain any description about restricting subchannels that are not indicated in the Disabled Subchannel Bitmap from being punctured. Such a description exists in the corresponding MAC sub-clause on preamble puncturing so it seems reasonable to at least have a reference for it here. | (Updated in the edit notes in the comments collection) Add the following text at the end of subclause 36.3.12.11.1: "Puncturing of subchannels that are not indicated in the Disabled Subchannel Bitmap subfield in 9.4.2.311 shall follow the restrictions described in 35.15.2 (Preamble puncturing operation)." | ACCEPTED Note to the Editor: Same resolution for the following CIDs: 16347, 16362 |

## CID 16362

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.46 | 36.3.12.11.1 | It is beneficial to include a reference to 35.15.2 in 36.3.12.11.1 or 36.3.12.11.2, so that the reader understands where more restrictions on preamble puncturing in an EHT MU PPDU are defined. | Add reference to Clause 35.15.2 in either 36.3.12.11.1 or 36.3.12.11.2, and explain that the restictions on the use of preamble puncturing in an EHT MU PPDU are defined in that clause (35.15.2). | REVISED Note to the Editor: Please apply the resolution of CID 16347 Same resolution for the following CIDs: 16347, 16362 |

## CID 17222

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 823.62 | 36.3.12.11.2 | The paragraph starting at line 62 talks about OFDMA and non-OFDMA specific specifications. Sections 36.3.12.11.2 and 36.3.12.11.5 are about preamble puncturing for OFDMA and non-OFDMA sepcifically. It may make more sense to move the requirements to these respective sections, rather than having them in the "General" section 36.3.12.11.1. | Move OFDMA and non-OFDMA specific requirements to 36.3.12.11.2 and 36.3.12.11.3 respectively. | REVISED Agree with the commenter: split a text from the general sub-clause to the sub-clauses of the OFDMA transmission and the non-OFDMA transmission. Instructions to the Editor: Please follow the changes indicated in https://mentor.ieee.org/802.11/dcn/23/11-23-0434-01-00be-lb271-cr-for-p802-11be-d3-0-section-36-3-12-11.doc |

Instructions to the Editor:

Add the following text after the first paragraph of 36.3.12.11.2 (824.17):

36.3.12.11.2 Preamble puncturing for EHT MU PPDUs in an OFDMA transmission

Preamble puncturing may exist in PPDUs transmitted to one or more users using OFDMA transmission.‌ﾠThe U-SIG and EHT-SIG fields include information on the preamble puncturing of the PPDU.

The preamble puncturing resolution shall be 20 MHz for an EHT MU PPDU using OFDMA transmission for a bandwidth larger than 40 MHz. In other words, puncturing a subchannel smaller than a 242-tone RU is not allowed in PPDU bandwidths larger than 40 MHz.

The U-SIG field contains signaling of the punctured 20 MHz subchannel(s) in the 80 MHz frequency subblock where it is transmitted (see Table 36-28 (U-SIG field of an EHT MU PPDU)).

Instructions to the Editor:

Add the following text to the beginning of 36.3.12.11.3 (824.40):

36.3.12.11.3 Preamble puncturing for EHT MU PPDUs in a non-OFDMA transmission

The preamble puncturing resolution shall be 20 MHz for an EHT MU PPDU with 80 MHz and 160 MHz PPDU bandwidths. In other words, puncturing a subchannel smaller than a 242-tone RU is not allowed in 80 MHz and 160 MHz PPDU bandwidths.
The preamble puncturing resolution shall be 40 MHz for an EHT MU PPDU with a 320 MHz PPDU bandwidth. In other words, puncturing a subchannel smaller than a 484-tone RU is not allowed in a 320 MHz PPDU bandwidth.

In the preamble puncturing of a non-OFDMA transmission, at least one 20 MHz subchannel, in the pre-EHT modulated fields, is not occupied and the EHT modulated fields consist of a single large size MRU that

Instructions to the Editor:

In sub-clause 36.3.12.11.1, remove the paragraph starting in 823.62 and ending in 824.4

36.3.12.11.1 General

Preamble puncturing refers to the transmission of a PPDU in which no signal is present in at least one of the 20 MHz subchannels within the PPDU bandwidth.

Preamble puncturing might be the result of the unavailability of 20 MHz subchannel(s) within the PPDU bandwidth, such as a busy channel indicated by the CCA or the setting of the Disabled Subchannel Bitmap subfield in the EHT Operations element (see 9.4.2.311 (EHT Operation element)).

Preamble puncturing may exist in an EHT MU PPDU transmitted in the DL or the UL and in an EHT TB PPDU transmitted by a non-AP STA in the UL. For an EHT MU PPDU, the U-SIG and the EHT-SIG fields include information on preamble puncturing.

## CID 17919

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 824.26 | 36.3.12.11.2 | A period is missing at the end of this sentence. | As in comment. | ACCEPTED |