IEEE P802.11
Wireless LANs

| LB271 CR for for EHT-SIG Part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.03.27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F3 building, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 |  | ross.yujian@huawei.com |
| Ming Gan |  |  |  |  |
| Edward Au |  |  |  |  |

Abstract

This submission contains proposed comment resolutions to comments on P802.11be D3.0.

Three comments are resolved.

CID 15268, 16641and 16642

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Changes on the proposed resolution based on offline discussion |

## CID 15268, 16641and 16642

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 15268 | 779.39 | 36.3.12.8.2 | What is the relationship between RU allocation subfield and RU Allocation-A/B subfield? It should be clarified. | As in the comment | REVISED Agree with the commenter, reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/23-0542r1 tagged with #CID15268. |
| 16641 | 792.05 | 36.3.12.8.3 | In figures 31-33 and table 36-33, there are RU Allocation-A and RU Allocation-B. Need to mention Table 37-34 is for both RU Allocation-A and RU Allocation-B. Also, there are many places uses just RU Allocation subfield rather than RU Allocation-A and RU Allocation-B. Need to mention the relationship, especially need to update table 36-35. | As in the comment | REVISED Agree with the commenter, reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/23-0542r1 tagged with #CID15268. |
| 16642 | 786.13 | 36.3.12.8.2 | Change the title of the Table 36-34 such as "RU Allocation-A and RU Allocation-B subfield". There is no RU Allocation subfield. There are only RU Allocation-A or RU Allocation-B subfield. | As in the comment | REVISED Agree with the commenter, reflect the detail changes. Instructions to the editor: Please make the changes as shown in 11/23-0542r1 tagged with #CID15268. |

Discussion

In 11ax, there is only one type of RU Allocation subfield. In 11be, there are two types of RU allocation subfield, located in different common encoding blocks, named as RU Allocation-A subfield and RU Allocation-B subfield. Propose to add a praragraph to explain the relationship.

End of discussion

TGbe editor, please add the following paragraph to P788, line 65 of P802.11be D3.1 as shown below:

(#15268) Both RU Allocation-A subfield and RU Allocation-B subfield refer to an RU Allocation subfield, located in different common encoding blocks.