IEEE P802.11
Wireless LANs

| CR for Section 9.3.1.19 on P802.11be – part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 14.03.2023 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Genadiy Tsodik | Huawei Technologies |  |  |  |
| Mahmoud Kamel | InterDigital |  |  |  |

Abstract

This submission contains proposed comment resolution to comment on P802.11be D3.0.

4 comments (15249, 15250, 16124, 16127) under subclause 9.3.1.19 are resolved.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Minor changes |

## CID 15249

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 159.63 | 9.3.1.19.1 | Why does VHT NDPA frame format include in General description and HE NDPA frame format inclued in HE NPDA frame section? Move VHT NDPAt format to VHT NDPA section or generalize the format in General description | As in comment | REVISED Agree with the comment. Several changes are introduced to organize the text in this subclause as shown below. Note to the Editor: please change the text as specified in 11/23-0669r1 under CID #16127 |

## CID 15250

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 163.07 | 9.3.1.19.1 | Some NDPA frame format and the explanations are in General section and some are in HE or VHT section even the explanations are same. Unify the NDPA format of VHT NDPA and HE NDPA (with 'STA Info List (nx2 or 4)' subfield) and the explanation of the fields except STA Info field in General description. | As in comment | REVISED Agree with the comment. Several changes are introduced to organize the text in this subclause as shown below. Note to the Editor: please change the text as specified in 11/23-0669r1 under CID #16127 |

## CID 16124

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 159.64 | 9.3.1.19.1 | Instead of mentioning VHT NDPA frame in the general description subclause, should provide a general figure for NDPA frame covering four cases. Replacing n x 2 by n x 2 or n x 4 in the current VHT NDPA frame format, and remove VHT in the title. | Mention the frame format of VHT NDPA frame in VHT NDPA frame format subclause.Provide a frame format for NDPA frame here. | REVISED Agree with the comment. Several changes are introduced to organize the text in this subclause as shown below. Note to the Editor: please change the text as specified in 11/23-0669r1 under CID #16127 |

## CID 16127

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 165.23 | 9.3.1.19.4 | Move the Table to the general description subclasue or to the VHT NDPA subclause where AID11 is first mentioned. Add reference to the table for the following subclauses. | As in comment | REVISED Note to the Editor: please change the text as specified in 11/23-0669r1 under CID #16127 |

Instructions to the editor: please do the following changes in subclause 9.3.1.19.1

please change the text starting from page 159, L50-55 as following:

9.3.1.19.1 General description

Move the first to ninth paragraphs of 9.3.1.19 to this new child subclause 9.3.1.19.1 and changes as follows:

The NDP Announcement frame has four variants, the VHT NDP Announcement frame, the HE NDP Announcement frame, the Ranging NDP Announcement frame, and the EHT NDP Announcement frame. The four formats are distinguished by the setting of the NDP Announcement Variant subfield in the Sounding Dialog Token field.

The frame format of the NDP Announcement frame is shown in Figure 9-XX (NDP Announcement frame format).

Add a new figure 9-XX – NDP Announcement frame format:

|  | Frame Control | Duration | RA | TA | Sounding Dialog Token | STA Info List | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 6 | 1 | n ×2 or n ×4 (Variant Dependent) | 4 |

Figure 9-XX— NDP Announcement frame format

Instructions to the editor: please change the text starting from page 160, L4 as following:

An NDP Announcement frame contains at most one STA Info field per STA.

The NDP Announcement frame contains at least one STA Info field with AID11, AID12 or AID13 subfield.

The AID11 subfield encoding in the NDP Announcement frame is defined in Table 9-45a (AID11 subfield encoding in an NDP Announcement frame).

The encoding of AID12 and AID13 subfields is defined in 9.3.1.19.2 (VHT NDP Announcement frame format).

Please move Table 9-45a (AID11 subfield encoding in an NDP Announcement frame) here

If the NDP Announcement frame contains only one STA Info field with a value less than 2008 in the AID11, AID12 or AID13 subfield, then in the case of VHT, HE or EHT NDP Announcement frames the RA field is set to the address of the STA addressed in the only STA Info field of this NDP Announcement frame (see 10.37.5.2 (Rules for VHT sounding protocol sequences), 26.7 (HE sounding operation), 35.7 (EHT sounding operation)). In the case of Ranging NDP Announcement frames, the RA address is set to the address of the RSTA or ISTA that is the intended recipient of the frame. If the VHT/HE/Ranging NDP Announcement frame contains more than one STA Info field with a value less than 2008 in the AID11 subfield, then the RA field is set to the broadcast address.

Instructions to the editor: please do the following changes in subclause 9.3.1.19.2

Please change the text starting from page 161, L10-15 as following:

9.3.1.19.2 VHT NDP Announcement frame format

Copy 2nd paragraph and move 10th to 12th paragraphs, Figure 9-76 (VHT NDP Announcement frame) [not shown here], Figure 9-78 (STA Info field format in a non-S1G STA) [not shown here], Table 9-43 (STA Info subfields) [not shown here], and Figure 9-79 (STA Info field format in an S1G STA) [not shown here] of 9.3.1.19 to this new child subclause 9.3.1.19.2.

Instructions to the editor: please make the following changes in subclause 9.3.1.19.3

Please change the text starting from page 161, L56-57 as following:

The AID11 subfield encoding in the HE NDP Announcement frame is defined in subcaluse 9.3.1.19.1, Table 9-45a (AID11 subfield encoding in an NDP Announcement frame).

The format of the STA Info field in an HE NDP Announcement frame if the AID11 subfield is not set to 2047 is defined in Figure 9-81 (STA Info field format in an HE NDP Announcement frame if the AID11 subfield is not 2047).

Instructions to the editor: please do the following changes in subclause 9.3.1.19.4

Please move Table 9-45 (AID11 subfield encoding in an NDP Announcement frame) to subclause 9.3.1.19.1 as specified above

Pleas change the text on P165 L16 as following: