IEEE P802.11
Wireless LANs

| LB271 CR for 36.3.10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023.06.20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ying Li | Huawei |  |  | liying246@huawei.com |
| Lei Huang | Huawei |  |  | lei.huang1@huawei.com |
| Ross Jian Yu | Huawei |  |  | ross.yujian@huawei.com |

Abstract

This submission contains proposed comment resolutions to 2 comments related to clause 36.3.10 of P802.11be D3.0.

CID 17196, 17197

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# #17196

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 17196 | Sigurd Schelstraete | 36.3.10 | 746.15 | Table 36-20, column "Description": N_SR is described as the "highest data subcarrier index". This is not correct if the punctured 20/40/80 MHz is at the high end of the bandwidth. | Correct description or add NOTE | Revised It is very clear from (36-9) and (36-10) that the value of N_SR only depends on the channel bandwidth, and does not depend on whether the highest 20MHz subchannel is punctured or not. For clarification, we add a note here. TGbe editor: please incorporate the changes shown in 11-23/1029r0 under tag #17196. |

Background (P746 of D3.0):

Background (P753~756 of D3.0):

Proposed Changes:

Instruction to TGbe Editor:

Change Table 36-20 (P802.11be D3.0 P746) as follows:

Table 36-20—Subcarrier allocation related constants for the EHT-modulated fields in a punctured non-OFDMA EHT PPDU (continued)

| Parameter | CBW80 with 20 MHz puncturi ng 484+ 242- tone MRU | CBW160 with 40 MHz puncturi ng 996+ 484- tone MRU | CBW160 with 20 MHz puncturi ng 996+484+ 242-tone MRU | CBW320 with 120 MHz puncturi ng 2996+ 484-tone MRU | CBW320 with 80 MHz puncturi ng 3996- tone MRU | CBW320 with 40 MHz puncturi ng 3996+ 484-tone MRU | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NSR | 500 | 1 012 | 1 012 | 2 036 | 2 036 | 2 036 | Highest data subcarrier index (#17196)NOTE-The value of N_SR only depends on the channel bandwidth, and does not depend on whether the highest 20MHz subchannel is punctured or not. |
| NDC | 23 | 23 | 23 | 23 | 23 | 23 | Number of null subcarriers at DC |
| NGuard,Left | 12 | 12 | 12 | 12 | 12 | 12 | Number of low frequency guard subcarriers |
| NGuard,Right | 11 | 11 | 11 | 11 | 11 | 11 | Number of high frequency guard subcarriers |

# #17197

| CID | Commenter | Clause Number(C) | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 17197 | Sigurd Schelstraete | 36.3.10 | 748.16 | "NOTE--For LDPC coding, this is the number of data bits per OFDM symbol based on the LDPC coding rate for the MCS.". I have no idea what the purpose of this note is. How is this different when BCC is used? This is a table with definitions. THis is not the place to go into the derivation of N_DBPS. | Delete NOTE | Accepted |

Background (P746 of D3.0):