IEEE P802.11
Wireless LANs

| IEEE 802.11 AIML TIG Technical Report Draft Executive Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023-11-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Xiaofei Wang | InterDigital Inc. | 111 West 33rd Street New York, NY 10120 USA | +1-607-592-2727 | Xiaofei.wang@interdigital.com |
| Ming Gan | Huawei |  |  | Ming.gan@huawei.com |
| Zinan Lin | InterDigital |  |  |  |
| Rui Yang | InterDigital |  |  |  |
| Aiguo Yan | Zeku |  |  |  |
| Junghoon Suh | Huawei |  |  |  |
| Ziyang Guo | Huawei |  |  |  |
| Marco Hernadez | YRP-IAI, Japan; CWC Oulu Univ. Finland, NICT, Japan |  |  |  |
| Liangxiao Xin | Zeku |  |  |  |
| Peng Liu | Huawei |  |  |  |
| Federico Lovison | Cisco |  |  |  |
| Jerome Henry | Cisco |  |  |  |
| Juan Carlos Zuniga | Cisco |  |  |  |
| Sukrit Dasgupta | Cisco |  |  |  |
| Eunsung Jeon | Samsung |  |  |  |
| Ziming He | Samsung Cambridge Solution Centre |  |  |  |
| Stephen McCann | Huawei |  |  |  |
| Jing Ma | Toyota Motor Corporation |  |  |  |
| Lei Zhong | Toyota Motor Corporation |  |  |  |
| Szymon Szott | AGH University of Science and Technology |  |  |  |
| Katarzyna Kosek-Szott | AGH University of Science and Technology |  |  |  |
| Boris Bellalta | UPF Barcelona |  |  |  |

Abstract

This document contains the executive summary of the technical report of the IEEE 802.11 AIML TIG.

R0: initial outline

- Table of Contents

Executive Summary	4

1	Introduction	5

1.1	Terminology	5

1.2	Background information	5

1.3	Technical Report Overview	5

2	AIML use cases for IEEE 802.11	7

2.1	General	7

2.2	Use case that enables AIML in WLAN	7

2.2.1	Use case 1: Enabling Efficient AIML Model Sharing	7

2.3	Use cases that leverage AIML to enhance WLAN performance	9

2.3.1	Use case 2: CSI feedback compression	9

2.3.2	Use case 3: Distributed channel access	13

2.3.3	Use case 4: Roaming Enhancements	15

2.3.4	Use case 5: Multi-AP Coordination	19

3	Conclusions and Recommendations	23

References	24

# Executive Summary

In recent years, significant strides have been made in the advancement of Artificial Intelligence/Machine Learning (AIML) technologies. Their widely applicable algorihtms have influenced numerous other application domains, including wireless communications.

The AIML Technical Interest Group (TIG) has been exploring AIML use cases and their feasibility for IEEE 802.11 networks and devices. These AIML use cases can be divided into two categories. In the first category, one use case, namely efficient AIML model distribution, has been explored as to how WLAN can efficiently enable and support AIML algorithms that need data across the whole or part of the network and make AIML models available to the IEEE 802.11 devices within the network. In the second category, four use cases have been studied in which AIML-based methods can be leveraged to improve the performance of various operations or layers in WLANs. These four use cases include Channel State Information (CSI) feedback, distributed channel access, AIML-based roaming and multi-AP coordination. They are targeted to utilize AIML techniques to reduce overhead, improve throughput, reduce delay and jitter, improve roaming smoothness and fairness, and enhance user experience. For each use case, the AIML TIG has extensively studied its feasibility. In addition, the AIML TIG has studied requirements, Key Performance Indicators (KPIs) and standards impact to the IEEE 802.11 specifications for any features that address that use case to provide an estimate of the scope of work required.  The use cases and the results of the AIML TIG study for each use case has been provided in Section 2.

The  conclusions  based on the AIML TIG’s study are:

In order to enable and support AIML algorithms in IEEE 802.11 network and devices and in order to leverage AIML techniques to enhance IEEE 802.11 network performance, features that are needed to address the AIML use cases identified in this technical report can be achieved with a reasonable scope of work. In addition, in order to support generic AIML algorithms in IEEE 802.11 networks and devices, general management and support features, such as capability indication, control and model distribution of AIML technology deployment are needed for all AIML-based use cases.

With the fast progress of AIML techniques and the integration of AIML methods in many areas of technology, IEEE 802.11 networks and devices are expected to support AIML related technology in the near term. In addition, it is also expected that IEEE 802.11 would benefit from AIML-based algorithms for better performance and enhanced user experience. Other standard development organizations are progressing quickly with enabling and integrating AIML algorithms and features.

Hence, the AIML TIG recommends that:

IEEE 802.11 should commence further study and work related to AIML technologies. In particular, efforts and standards changes to make the IEEE 802.11 standards deployment-ready (i.e., general support and management) for AIML techniques should be defined. IEEE 802.11 should also consider defining features and capabilities in which AIML techniques are leveraged to enhance network performance and user experience when using IEEE 802.11 networks and devices. Privacy considerations should also be studied during further AIML related work.

# Introduction

## Terminology

AIML	Artificial Intelligence/Machine Learning

CSI	Channel State Information

DL	Downlink

UHR	Ultra High Reliability

UL	Uplink

## Background information

Artificial Intelligence/Machine Learning (AIML) algorithms have made significant progress and are being applied in many domains, including medical diagnosis, speech recognition, computer vision, and the integration of vision and control for robotics. In addition, AIML algorithms are emerging as important components in many applications such as autonomous driving, language translation and human-machine interactions.

Traditional AIML techniques are based on a centralized model which requires the exchange of a large amount of data between data sources and a centralized server. More recently, distributed AIML algorithms such as federated learning have been developed that will allow more analysis at the source, reducing the amount of data that need to be exchanged, though the expected amount of exchanged data remains significant. With the prevalence of wireless networks and communications, much of the exchanged data is expected to be carried through wireless networks, such as IEEE 802.11 WLANs.

Studies have shown that AIML algorithms can help improve the performance of wireless communication networks, by providing better resource usage, lower energy consumption, higher reliability and more robustness within a changing environment. As these algorithms become more mature and cost effective, WLAN may leverage AIML for enhanced network performance and user experience.

In May 2022, the IEEE 802.11 Working Group (WG) approved the forming of the AIML Task Interest Group (TIG) with the following motion [1]:

Motion 5: TIG Re: AIML use in 802.11

Approve formation of a Topic Interest Group (TIG) to:

(a) describe use cases for Artificial Intelligence/Machine Learning (AIML) applicability in 802.11 systems and

(b) investigate the technical feasibility of features enabling support of AIML.

The TIG is to complete a report on this topic at or before the March 2023 session.

## Technical Report Overview

This technical report is the final report of the AIML TIG to the IEEE 802.11 working group, detailing various AIML use cases discussed by the AIML TIG.

The AIML TIG has identified two categories of uses cases. The first category is about how the WLAN can efficiently enable and support AIML algorithms that need data across the whole or part of the network and make AIML models available to the IEEE 802.11 devices within the network. Distributing AIML models is a specific use case identified for this category. The second category is about how the AIML-based methods may be leveraged to improve the performance of various operations or layers in WLANs. Four use cases have been identified for this category.

For each identified use case, a number of Key Performance Indicators (KPIs) have been identified. In addition requirements and technical feasibility analyses have been provided. Standards impact to the IEEE 802.11 specifications is also identified for each use case to provide an estimate of the scope of needed work to address the use case.

Conclusions and recommendations are provided as well for further studying and working on AIML related topics in IEEE 802.11.

# AIML use cases for IEEE 802.11

## General

The AIML TIG has identified two categories of use cases for WLAN in the area of AIML. In the first category, it is explored how WLAN can efficiently enable and support AIML algorithms that need data across the whole or part of the network and make AIML models available to the IEEE 802.11 devices within the network. One use case has been identified for this category. In the second category, it is explored how AIML-based methods may be leveraged to improve the performance of various operations or layers in WLANs. Four use cases have been identified for this category. These use cases are described in details below.

## Use case that enables AIML in WLAN

### Use case 1: Enabling Efficient AIML Model Sharing

#### Use case description

AIML models are mathematical algorithms replicating a reasoning and decision process to enable automation and understanding. They may include specific or application specific learning algorithms, including but not limited to, supervised, unsupervised and reinforcement learning, and their parameters.

The generation and refinement of AIML models require a large amount of information exchange depending on a number of factors, such as AIML model users (such as APs and non-AP STAs), network deployment, device computation power, and the AIML model generation schemes.

Traditionally, AIML models as well as the data required for AIML model training are exchanged as application data and may severely burden wireless networks on which such information exchange is carried, such as the IEEE 802.11 WLANs, which can cause significant descrease in QoS performance and undermine the user experience, contradictory to the ever increasing demand of higher throughput and better QoS. However, existing wireless network protocols, including those in WLAN MAC and PHY layers, may not be able to support some applications, due to their stringent latency and reliability requirements, therefore  need to rely on AIML models for performance gain.

Assuming that devices participating in AIML-based operations can be both non-AP STAs and APs, efficient AIML model distribution is essential for the success of AIML-based operations as well as for the performance and user experience of an IEEE 802.11 WLAN in the scenarios below:

- Centralized learning or centralized AIML model generation/refinement

In this scenario, STAs provide training data to a central point, e.g., an AP, or a server, where the global AIML model is generated and refined. The central point, which may be the AP, subsequently distributes the global AIML model to all STAs, in the DL direction.

- Distribued learning or distributed AIML model generation/refinement

In this scenario, all participating devices, such as STAs or APs, are capable of computing AIML models and share data to all participating devices so that each device can train the global model at the device. This scenario, without much optimization,  is considered to be especially taxing for an IEEE 802.11 network, since it requires a large amount of data to be exchanged over the air and also requires high computation power at each device.

- Federated learning or hybrid AIML model training/refinement

In this scenario, all participating devices, such as STAs or APs, are capable of computing AIML models and generating their local models and subsequently:

    - All or a subset of participating devices share their local models to a central point, such as the AP or a central server, likely in the UL direction, and the AP or central server generates a global model using the received local models as input and distributes the global model to all or a subset of participating devices, likely in the DL direction;

    - All or a subset of participating devices share their local models to all participating devices, likely in the UL direction and peer-to-peer, and all participating devices generate the global model using the received local models as input.

Additionally, with expected increased use of AIML techniques, IEEE 802.11 WLANs are expected to carry a significant amount of traffic that distributes AIML models and their parameters.These models are not limited to only those AIML models that are used to enhance performance and user experience for WLANs. There can also be AIML models that are used for other applications such as autonomous driving, video compression, human-machine interface.

Wireless transmissions to distribute information are by nature broadcast and IEEE 802.11 WLANs should leverage this for the more efficient distribution of AIML models. IEEE 802.11bc-2023 has specified Enhanced Broadcast Services (EBCS), which includes both UL and DL broadcast services for the cases in which APs and STAs are associated and for the case in which they are not, provides a good set of baseline technologies for efficient AIML model distribution.

In this use case, additional WLAN procedures and signaling are expected to support efficient distribution of AIML models and their parameters, including AIML models that are used to enhance performance and user experience for WLANs as well as AIML models for other applications are expected to be carried by the WLANs anyway.

#### KPIs

KPIs considered in this use case include:

- AIML model distribution airtime reduction: the medium occupation time consumed using newly designed AIML model distribution protocols, as compared to traditional designs that distribute AIML models through the application layer data.

- additional overhead used for AIML model distribution: the additional medium occupation time consumed, which is needed to enable and manage the new AIML model distribution protocols.

#### Requirements

The performance of newly designed AIML model distribution protocols shall provide medium occupation time savings, including any medium occupation time required for overhead signalings, compared to traditional designs that may distribute AIML models through the application layer data.

#### Technical Feasibility Analysis

##### Standards Impact

The following standards impacts are expected:

- Architecture that enables AL/ML model sharing: architecture changes are expected to allow AIML models to be shared at the MAC Layer.

- Signaling and protocols related to AIML model sharing support/capability indication.

- Signaling and protocol related to AIML model sharing management.

##### Technical feasibility

IEEE 802.11bc has defined UL and DL EBCS services for STAs and APs, either associated or unassociated, which provides a good set of tools for supporting efficient AIML model sharing and distribution in IEEE 802.11 WLANs. Given the standards impact provided by IEEE 802.11bc-2023, as described in the previous section, it is expected that specification for a new design of efficient AIML model distribution protocols should be accomplished with a reasonable timeline.

Additionally, neural network quantization is a well-developed AI technique that significantly reduces the required data for model parameters, such as post-training quantization and quantization-aware training [38] . This technique can be utilized for AIML model distribution airtime reduction [39] .

## Use cases that leverage AIML to enhance WLAN performance

### Use case 2: CSI feedback compression

#### Use case description for Use Case 2a: system throughput improvement

In 802.11ax [1] and the draft of 802.11be [2], the AP initiates a sounding sequence by transmitting the NDPA frame followed by a NDP, which is used for the generation of a precoding matrix, V, at the beamformee. Upon the receipt of the NDP from the beamformer, the beamformee applies a compression scheme (i.e., a Givens rotation) on the V matrix and feeds back the angles in a beamforming report frame.

It is indicated in [4]  that a higher number of spatial streams has been an inevitable trend in WLANs for more than a decade. The preliminary results [4] [5] [6] show that MIMO with a large number of transmitter antennas and a large number of spatial streams (e.g., 16 spatial streams) offer remarkable system performance gains in both SU-MIMO and MU-MIMO cases. Coordinated multi AP (MAP) may be one potential feature in the next 802.11 generation, e.g. UHR [7] - [10] . A large number of spatial streams combined with coordinated MAP features may further increase the sounding feedback airtime overhead if coordination between APs (e.g., joint transmission/reception, coordinated beamforming) is applied.  Large amounts of overhead or prolonged sounding procedures may negatively impact the latency and limit the system performance. Therefore, there is a need to reduce the CSI feedback overhead, especially when the number of transmitter antennas increases or multiple APs perform joint or coordinated transmissions.

Some studies (e.g., [11] [12] [13] [14] [30] [31] Error: Reference source not found have shown that AIML can efficiently reduce the CSI feedback size and improve the system throughput. For example, motivated by the fact that the CSI may fall into different clusters due to the similarity of nearby STA channels, the iFOR algorithm [11] and algorithm-I in [31] apply the unsupervised learning, K-means, to the CSI compression to classify the angle vectors which are derived from the V matrix.

Algorithm-II in [31] applies K-means to classify the V matrix directly. Simulation results in [11] show that for an 8x2 SU-MIMO, iFOR uses around 8% of the number of bits required by the existing feedback mechanism (802.11ax) and boosts the system throughput by up to 52%. Moreover, simulation results in [31]  have shown that the proposed algorithms boost the system throughput by up to 20%.

In [30] , the dual CSI compression combines a codebook and a Givens rotation. It uses the fact that any untiary matrix can be expressed by multiplication of two unitary matrices, i.e., V = V1 V2. By using this property, the dual CSI compression decomposes a large sized CSI into a subband CSI as a function of V1, CSI (V1) , and a subcarrier CSI as a function of V2, CSI (V2), giving lower a feedback rate for the slow-varying subband CSI (V1) using the codebook, while allocating a higher feeback rate for the frequency-selective subcarrier CSI (V2) using the Givens rotation.

In order to improve the reliability for a subband CSI (V1), the K-means algorithm is exploited for the codebook generation. The simulation results show that for an 8x2 SU-MIMO, the AIML aided dual CSI compression can reduce the feedback overhead by more than 50% compared with the conventional scheme. In addition, the throughput improvement from the reduced overhead is about 20%.

In [12] , another unsupervised learning technique, Deep Neural Network Autoencoder (DNN-AE) is applied to CSI angle vectors and further compresses the derived angles by leveraging the compression capability of DNNs. Experimental results show that the algorithm reduces the feedback overhead by 73% and increases the network throughput by 69%, on average.

In [32], a vector quantized variational autoencoder (VQVAE), which learns the compression and quantization schemes together, is applied to compress the V matrix. Simulation results show that compared to the existing feedback mechanism (802.11ax), the implementation of VQVAE can reduce the feedback overhead by 97% and increase the system throughput by up to 223%. In addition, a lightweight encoder and uniform quantization are adopted to reduce the model deployment overhead and inference complexity while maintaining the throughput improvement.

Hence, in this use case, applying AIML techniques to the CSI feedback process are expected to reduce CSI overhead with minimum loss PER performance, resulting in an increase in network throughput.

#### Use case description for Use Case 2b: compression complexity reduction

A large number of transmitter antennas and spatial streams will significantly increase the computational complexity of the CSI feedback compression. For example, the complexity comes from the Givens rotations in the V matrix compression. A highly complex compression scheme may negatively impact the processing time (introduces latency) and the power consumption at non-AP STAs [33] .

A study [33] has shown that AIML can reduce the computational complexity of the CSI feedback compression by nearly 60% without degrading the system throughput. The idea is to compress the V matrix directly by avoiding the computation in the Givens rotations, using a low complexity Autoencoder.

This use case proposes to apply AIML technique to CSI feedback schemes to reduce the computational complexity of the feedback compression without loss of the system throughput.

#### KPIs

KPIs considered in this use case include:

- Number of feedback bits per subcarrier group

- Achieved PER

  - Both SU-MIMO and MU-MIMO cases need to be considered

- Additional AIML overhead compared with additional compression gains

  - One example is the ratio between the number of additional bits required by an AIML process (including data used for model training/inference [15] , the AIML model parameters, CSI codebook information, the additional signaling, etc.) and the number of bits saved by the CSI feedback scheme. In this example, if the data used for model training that is performed by the AP fully relies on the legacy CSI report, then the additional AIML overhead used for model training/inference may be negligible.

- Computation complexity/Latency:

  - Additional delay or computation is introduced by AIML processing. An evaluation methodology needs to be established.

#### Requirements

- Support backward compatibility and coexistence with legacy 802.11 CSI report schemes

- Performance should follow the guidance below:

  - CSI airtime reduction: achieve airtime reduction of CSI feedback over 802.11be for a given Nr x Nc MIMO, where Nr is the number of rows in the compressed beamforming feedback matrix, Nc is the number of columns in the compressed beamforming feedback matrix.

  - Additional AIML overhead: minimize the additional overhead used for AIML process. Additional AIML overhead may include the data used for an AIML model training/inference [15] , the model parameters and additional signaling. The data used for AIML model training/inference [15] can reuse legacy CSI report data.

  - Packet Error rate (PER): guarantee minimum SNR loss, compared with 802.11be, to achieve the target PER (e.g., 1% and/or 10%) at a given MCS in all types of channels [16] .

  - Computation complexity/Latency: minimize the additional computation complexity or latency required by an AIML process.

  - AIML model generalization: improve the generalization capability of AIML models. Generalization refers to the ability of an AIML model to be used in various scenarios, such as different channel models, bandwidths or number of spatial streams. For example, one or few number of  codebooks are required in different scenarios for K-means based CSI feedback schemes (e.g., [11], [30], [31]); a single neural network model (including both architecture and parameters) can be used in different scenarios for autoencoder based CSI feedback schemes (e.g., [32]).

#### Technical Feasibility Analysis

##### Standard Impact

The standard impact may include:

- Additional signaling (e.g., between AP and non-AP STAs) required by an AIML process, , such as codebook sharing, AIML model (encoder or/and decoder) sharing, etc.

##### Technical feasibility

The following metrics will be studied:

- Backward compatibility: The STAs that supports AIML enabled CSI feedback compression shall support the legacy 802.11 CSI report scheme. This compatibility is expected to be supported since AIML capable STAs are expected to support legacy CSI report scheme.

- Data availability and accessibility: There are some STAs that are able to use data to perform AIML model training and/or inference [15] . The data used for model training and/or inference shall be accessible for these STAs.

      - AP/edge computing based AIML: Data may be collected from non-AP STAs. The legacy 802.11 CSI reports may be used as training data.

      - Device computing based AIML: Data should be available at all STAs that support an AIML process.

- Hardware/software capability: The STAs that use AIML to generate CSI feedback compression shall have the hardware and software capability to support the AIML algorithm(s).

      - AP/edge computing based AIML[17] : Extra data and model (e.g., model parameters) exchange may be required to support AP/edge computing based AIML.

      - Device computing based AIML [17] : STAs that support AIML may be required to have extra computation capability. Extra data and model (e.g., model parameters) exchange between STAs may also be required to support device computing based AIML.

### Use case 3: Distributed channel access

#### Use case description

In 802.11, the fundamental access method of the MAC used by STAs is a Distributed Coordination Function (DCF) known as carrier sense multiple access with collision avoidance (CSMA/CA).

A STA is required to sense the wireless medium before transmitting. If it finds that the medium is continuously idle for a minimum specified duration, e.g., DIFS, the STA is permitted to start the transmission of a frame after waiting for an additional random backoff duration. The range of the generated random backoff counter is bounded by the

Contention Window. The initial range is set from 0 to the minimum Contention Window size (CWmin). If the medium is occupied, the contention window is doubled until it reaches the maximum Contention Window size (CWmax) in order to further reduce collisions.

Requirement for ever increasing throughput and stringent latency have been an inevitable trend in WLAN for more than a decade as mentioned in [18] [19] [20] [21] . An efficient channel access protocol is fundamental to achieve these goals. However, the analysis in [22] shows that current contention-based access protocol suffers large performance degradation in dense deployments. The binary exponential backoff leads to short-term unfairness [23] , i.e., consecutive unsuccessful transmissions lead to worse latency performance. Therefore, more efficient channel access schemes are needed to increase throughput and to reduce latency and jitter.

Some studies [24] [25] [26] [27] have shown that AIML based channel access schemes can efficiently improve throughput/channel efficiency and additionally reduce latency and jitter. For example, Centralized Contention window Optimization with Deep reinforcement learning (CCOD) [24]  applied Deep Q Network (DQN) or Deep Deterministic Policy Gradient (DDPG) contribute to contention window optimization. Simulation results show that the throughput improvement of CCOD over standard 802.11 ranges from 1.5% (for 5 stations) to 40% (for 50 stations). In [25] , QMIX-advanced Listen-Before-Talk (QLBT) is proposed for distributed channel access by leveraging the powerful learning capability of deep neural networks. Experimental results show that QLBT increases the channel efficiency by 18% and reduces the latency and jitter by 25% and 90% on average. In addition, other factors such as coexistence with conventional schemes [35] , model deployment overhead [36] , and dynamic environment [37] , are also considered to enhance the overall AIML  algorithm performance.

In this use case, applying AIML techniques to distributed channel access is expected to improve throughput and reduce latency and jitter.

#### KPIs

KPIs considered in this use case include:

- Throughput

- Latency and jitter

- Fair coexistence

#### Requirements

- Fair coexistence with legacy 802.11

  - An AIML process shall support fair coexistence with legacy 802.11 channel access schemes. Utilization of an AIML-based channel access scheme by one or more STAs in a network shall not degrade the performance of STAs utilizing conventional channel access schemes, regardless of whether these STAs are operating in the same BSS or in OBSS. Performance metrics include, but are not limited to, throughput and latency.

- Performance should follow the guidance below:

  - Throughput improvement measured at a MAC data service access point (SAP): Achieve throughput improvement compared to 802.11be for different scenarios.

  - Latency and jitter reduction: Achieve latency and jitter reduction compared to 802.11be for different scenarios. Latency is the time from when a packet enters the queue to when it is successfully received. Jitter is the standard deviation of the latency.

  - Additional complexity introduced by AIML: Minimize the additional complexity introduced by an AIML process. The complexity includes various factors, such as the computation complexity required for AIML decision making, the additional storage for an AIML model, and the overhead associated with the AIML model distribution. These factors can be evaluated by considering metrics such as the number of FLOPs required per AIML inference and the number of model parameters (e.g., [34] [36] [37] ).

  - Performance evaluation needs to consider both single BSS and OBSS scenarios.

#### Technical Feasibility Analysis

##### Standard Impact

The standard impact may include:

Signaling and protocols related to parameter exchange between AP and non-AP STAs. These may include:

      - capability indication

      - data report to facilitate training

      - neural network model deployment (according to Section 2.3)

      - management, e.g., neural network model operation and maintenance, abnormal event report [37] .

##### Technical feasibility

The following metrics will be studied:

- Hardware/software capability: The STAs that support AIML based channel access shall have the hardware and software capability to support AIML algorithm(s). The STAs that support model training may require higher computation capabilities.

- Data availability and accessibility: There are some STAs that are able to use data to perform AIML model training and/or inference [28] . The data used for model training and/or inference shall be accessible for these STAs. It is expected that signaling and protocols related to data accessibility will be specified.

- Model distribution: There may exist AIML model exchange between AP and non-AP STAs during model training and/or inference [28] . It is expected that signaling and protocols related to AIML model distribution will be specified.

### Use case 4: Roaming Enhancements

#### Use case description

Multi-AP networks commonly see a high level of roaming events. In dense AP deployments, the high number of roaming candidate APs and channels in use can cause the STA scanning process to take a long time, affecting the device battery lifetime and the user experience.

The 802.11k neighbor report, sent by the AP upon STA request, indicates information about neighboring APs BSSID, channel, PHY type and optional elements (see 802.11-2020 9.4.2.36) that are useful to assist STA roaming decisions, mainly by providing a subset of channels and pre-scanned AP candidate list.

However, not all the neighboring APs are equally as relevant for the STA to make optimal roaming decisions. Each AP can have tens of neighbors, close and far, and there is no easy mechanism for the STA to determine which is likely the best next AP to roam to.

The roaming enhancement use case proposes to apply AIML techniques to determine the probability of a STA to roam to a specific AP, based on the learned client roaming patterns.

In particular, the output of the AIML algorithm can be used to:

- Augment the 802.11k neighbor report, assigning a weight to each candidate, to help the STA prioritize scanning towards APs that are the most probable roaming targets.

While roaming algorithms implemented on STAs often already assign a weight to each neighbor, based for instance on a QBSS load, the proposed weight represents the probability of the STA to roam to a given AP based on roaming patterns; therefore it would help STAs prioritize scanning on channels that are likely served by APs located in the STA’s direction of movement, which in some scenarios will be more relevant than the load information (e.g., if the least loaded AP is located in the opposite direction to the client’s moving direction, associating to such an AP would probably require the STA to roam again within a short time).

The proposed enhancements aim at providing the STA’s roaming algorithms with additional information, in order to improve the quality of roaming decisions. Likewise, the STAs can share information about the roaming decisions or its local conditions, which can then be used to further enhance future weight reports (e.g., providing a reason code, in case the STA decides not to follow the recommendations received by the AP, etc.).

- Assist the STA to find the mid-point between APs.

The proposed solution aims at applying AIML techniques to predict the optimal RSSI threshold, for a given STA to start the scanning; the prediction can include the target RSSI value as well as the predicted time that a given STA would reach such an RSSI level (scanning delay) in order to help the STA to know in advance when to expect the scanning to start.
This allows the STA to optimize the time of scan and roam, to maintain the highest possible connection speed, without the need to wait for a usability threshold failure (e.g., increased retry/drop rate, when reaching the edge of the AP’s radio coverage).

The goal is to achieve overall faster roaming with reduced latency and improved reliability.

The proposed solution supports multiple AIML processes, from simple models to more sophisticated deep-learning techniques, allowing for distributed or centralized training and inference, adapting to different deployment scenarios. A sample implementation of the neighbor report weight calculation with an estimation of the benefits achieved, as compared to a flat neighbor list is described in [29] .

How the AP determines the weights for each candidate AP or the target RSSI level is implementation specific. The AP can use AIML techniques or non-AIML techniques to determine these values.

In this use case, applying AIML techniques to determine the probability of a STA to be able to roam to a specific AP, based on the learned client roaming patterns, is expected to improve roaming performance in terms of latency and reliability.

#### KPIs

KPIs considered in this use case are proposed as follows:

- Scanning / Roaming time:

  - Both scanning and roaming time are expected to decrease by having the STAs to follow the scanning priority suggested by the weighted neighbor reports as compared to the traditional method (flat neighbor report).

- Roaming failure rate:

  - Roaming failure rate is expected to decrease because of the recommendations about the best target neighbors, as compared to the traditional method.

- MCS distribution during a BSS transition:

  - The AP mid-point prediction is expected to allow the STAs to maintain higher MCS rates during the roaming, hence having a distribution whose mode is shifted towards higher data-rates, as compared to the traditional method.

  - The measurement is referred to MCS rates for successful transmission, for both uplink and downlink.
While data rates for retransmissions wouldn’t be relevant to this KPI, retry rate is proposed as a separate KPI.

  - The KPI would be primarily aimed at measuring the MCS rate improvement for data frames, as those would have the stronger impact on user experience; moreover, as management frames are often transmitted at basic rates, they’re less reflective of the STA conditions.

- Retry rate during a BSS transition:

  - The AP mid-point prediction is expected to result in a reduction in retransmissions, by allowing the STAs to roam before usability threshold failures are hit.

  - The retry rate reduction and the MCS rate improvement metrics are correlated and they’re both meant to measure the link quality enhancement obtained thanks to STA making use of the mid-point prediction, even though they should be measured separately due to the implementation differences for MCS selection and retry algorithms.

- RSSI at reattach time:

  - The AP mid-point prediction is expected to improve the overall roaming, and the STA RSSI at re-association time would be an indication of whether the STA followed the recommended scanning time and RSSI threshold.

#### Requirements and Potential features analysis

The following potential features can be analyzed in this use case:

- Roaming neighbor list

- Roaming mean-point

#### Technical feasibility analysis

##### Standards impact

Notifying STAs about the weight of each roaming neighbor candidate, requires the enhancement of the existing 802.11k neighbor report, using new optional elements. Using optional elements to distribute this information would allow backwards compatibility.

Similarly, helping the STAs to trigger scanning at the mid-point between APs may require the enhancement of the 802.11v BTM request frame, for instance to include a scanning delay parameter.

The 802.11v BTM response frame can also be enhanced to let the STAs provide detailed feedback on the roaming recommendations, in order to allow the AP to improve the quality of the recommendations over time.

##### Technical feasibility

Depending on the specific implementation, the AIML capabilities can be either handled on each individual AP or managed by a central controller; the entity managing the model training would require sufficient computation capabilities to train and execute the implemented model.

#### Privacy considerations

Learning the roaming paths to provide the STAs with recommendations on the next roaming candidate AP or the RSSI mid-point requires collecting and processing STA association and connection information, which naturally raises privacy concerns.

The proposed AIML-based roaming enhancement use case is designed to only leverage information already available today at the network infrastructure level, and often already widely used to implement other use cases (e.g., location-based services, AIML-less roaming recommendations, etc.), therefore we expect no negative impact to the privacy exposure.

Just like various implementations fulfill today roaming recommendations based on different ways of understanding the STA BSS location and path, there are several possible ways to implement the computation of the roaming predictions when leveraging AIML; in the simplest scenarios (i.e., the method presented in [29]), there is no need to track the STAs full roaming path, and the STAs identifiers are also not important, as learning can be based only on the aggregate count of the transitions observed between any given AP and each of its neighbors.

More sophisticated implementations may record longer roaming vectors to increase the prediction accuracy; in this case, tracking a STA through more than one BSS is needed, but there is no requirement to know the real identity of the STAs, as learning can still be effectively achieved using anonymized identifiers. For instance, the proposed method is applicable in case the STAs make use of randomized and changing MAC addresses (e.g., as defined in IEEE 802.11aq).

MAC randomization, just like anonymization, is not expected to affect the quality of the learning and predictions. On the other hand, active MAC address rotation (e.g., as anticipated with 802.11bh and 802.11bi) can protect the STA privacy and limit the tracking of the STA roaming paths (with and without AIML). In this use case, such rotation still permits the generation of recommendations, with a prediction accuracy that matches the STA willingness to share its path information.

### Use case 5: Multi-AP Coordination

#### Use case description

In dense multi-AP networks, co-channel interference becomes a problem. Hence, methods for multi-AP coordination (MAPC) become necessary to improve the utilization of the limited radio resources, typically measured as an increase of the overall network throughput [43] . An example method is to create groups of APs, able to leverage spatial reuse opportunities, and have stations transmit in parallel during transmission opportunities (TXOPs) shared among coordinated APs [44] [50] . Several coordination methods (sometimes called coordination subtypes), which are referred to as MAPC transmission strategies, have been proposed in the literature and discussed in the 802.11 WG including coordinated TDMA [49] , coordinated OFDMA, coordinated spatial reuse, coordinated beamforming, and joint transmissions [42] . In addition to the strategies used for transmissions, strategies for MAPC coordination level (centralized, distributed, etc.) also need to be studied. Various coordination levels have been proposed, e.g., two levels (loose/light coordination and tight coordination) [40]  or even three levels (centralized, directly between APs and between APs through a common non-AP STA) [41] . The Ultra High Reliability (UHR) SG will be “considering scenarios with multiple overlapping networks” [48]  and, based on discussions in the group, MAPC may be included as one of the features in UHR [42] ; at this stage specifics of the MAPC features (transmission strategies and coordination levels) are still left open.

Analyzing the state of the art, MAPC is clearly a novel functionality with as yet few ML-based solutions [14] . A related method was studied in [45] , where distributed MIMO (D-MIMO) for WLANs was proposed under the assumption that APs are replaced in a multi-AP network by radio heads (RHs). Deep reinforcement learning (DRL) agents were able to learn which groups of RHs to select to perform D-MIMO, following dynamic client distribution, leading to a 20% overall throughput improvement. The authors of [46]  propose a centralized channel allocation architecture which supports federated learning of channel access. The method is shown to outperform TXOP sharing, but is not strictly an MAPC transmission strategy as considered by UHR. However, coordinated OFDMA is a considered MAPC transmission strategy and its performance is closely related to regular (uncoordinated) OFDMA. The difference between coordinated and uncoordinated OFDMA is that in the latter, resource units (RUs) are allocated among stations within a single BSS, while in the former – within multiple BSSs. There has been much research done on optimizing RU allocation in OFDMA networks [51] [52] [53] . In particular, deep reinforcement learning is used in [54]  to improve RU selection in IEEE 802.11ax OFDMA networks: average throughput is increased by over 100%, while latency is reduced by over 70%. In our opinion, the same ML methods proposed for single-BSS OFDMA can be applied to the coordinated OFDMA case and therefore, similar performance gains will be obtained. Furthermore, for the case of coexistence between coordinated TDMA APs with legacy APs, the authors of [55]  propose a reinforcement learning method to avoid interference from disruptive, uncoordinated MAPC transmissions and thus improve the aggregate network throughput. In summary, even though a literature review has revealed few AIML solutions to MAPC so far, further research in this area is expected, as evidenced by the “Machine Learning for Throughput Prediction in Coordinated Wi-Fi Networks” problem statement of the ITU AI Challenge [47]  and the recently released survey [56] .

To illustrate AIML-enhanced MAPC operation, Figure 1 shows a simple scenario with 2 BSSs. AP 1 of BSS 1 is able to execute AIML models, which may be trained in AP 1, in other APs such as AP 2, or outside the 802.11 network. It collects channel state information (CSI), buffer state reports (BSRs) and any other suitable information from AP 2 of BSS 2 and all stations (STAs 1, 2, and 3), either directly or through AP 2. Using the collected information as the input for the AIML model, AP 1 is then able to a) determine if AP 1 and AP 2 can form an MAPC group, and b) select the most adequate transmission strategy depending on which AP-STA pairs are scheduled (including SU or MU, DL or UL). For example, while STA 1 and STA 3 can transmit simultaneously by leveraging coordinated spatial reuse, transmissions from STA 1 and STA 2 require coordinated OFDMA.

Figure 1. MAPC example scenario

The MAPC use case proposes to apply AIML methods to MAPC with the goal of improving utilization of available resources while maintaining minimum inter-BSS interference. In particular, AIML can be used to learn and make decisions for the following subtasks of MAPC:

- Select which devices (APs/STAs) to involve (e.g., find the AP-STA pairs that can leverage coordinated transmissions).

- Configure the selected transmission strategies and the associated parameters.

- Perform prediction-based scheduling decisions following traffic patterns and channel conditions.

- Reduce the overhead of statistical information exchange required for MAPC.

Since UHR will define a subset of all possible coordination levels and transmission strategies, we note that AIML can choose the best strategies among the available options given the state of the environment. The AIML model selection and training will need to consider the specifics of the available transmission strategies (e.g., coordinated beamforming may require different training than coordinated OFDMA).

Figure 2 presents an example message exchange for an AIML-enhanced multi-AP coordinated transmission. (This is an illustrative example only, the exact exchange of MAPC messages will be determined by UHR.) The MAPC exchange is triggered by a sharing AP (AP 1) with an MAPC-RTS/MAPC-CTS exchange between all APs involved in the upcoming coordinated transmission. This RTS/CTS may be optional. Then, in an initial phase —indicated by (1) in Figure 2— each AP collects information from its associated stations. Note that this data collection and report phase may not be required if AIML can predict measurement reports, or in the case that some measurements are quasi-static. Moreover, principal component analysis (PCA) can be used to reduce the amount of data to exchange (i.e., exchanging only a fraction of the total data, and reconstructing the missing data at the destination). However, since the reconstructed data may incur information loss, the accuracy-overhead trade-off needs to be considered. In the second phase —indicated by (2) in Figure 2— these reports are communicated to the sharing AP which uses AIML to select and configure the MAPC transmission strategy to be used in the final phase of transmission. It is expected that the pre-trained AIML model provides either a near-instantaneous response for immediately scheduling the coordinated transmission or schedules coordinated transmissions in the near future.

Figure 2. Example of AIML-enhanced MAPC message exchange.

In summary, the reported use case of applying AIML for multi-AP coordination is important because MAPC is a strong candidate for adoption in UHR and can benefit from extensions with AIML support.

#### KPIs

The KPIs considered in this use case are proposed as follows:

- Network performance metrics (throughput, latency and jitter, power efficiency) measured at the BSS level but also aggregated over the whole multi-AP network.

- Fairness – to ensure that all users are fairly served.

- AIML overhead – additional signaling, computational complexity, and learning latency, i.e., any additional delay which is introduced by AIML exploration.

#### Requirements

The requirements of this use case are the following:

- Compatibility with UHR amendment (as it becomes drafted).

- Performance evaluation proving the gain from AIML (improved network performance metrics, high fairness, low overhead).

#### Potential features analysis

The following potential features can be analyzed within the scope of this use case. First, applying AIML in scenarios where the MAPC transmission strategy must be selected and configured. The set of transmission strategies will be defined in the UHR amendment and may include TXOP sharing (coordinated TDMA), coordinated OFDMA, coordinated spatial reuse, coordinated beamforming and joint transmissions. Second, applying AIML to predict measurement reports to limit the MAPC signaling overhead.

#### Technical feasibility analysis

##### Standard impact

The signaling and protocols related to parameter exchange between APs as well as between APs and non-AP STAs may be impacted. Signaling may be required to facilitate AIML model training, e.g., an AP may request additional data reports (the same format as standardized but requested more frequently). Additionally, trained AIML models may be shared between APs (according to Use case #3) so that any AP can be the sharing AP in case they are the TXOP holders. Conversely, signaling can also be reduced if an AIML model is able to predict network and traffic conditions so that the exchange of reports, indicated as (1) and (2) in Figure 2, is not required.

##### Technical feasibility

Hardware/software capability: The APs that support AIML-based multi-AP coordination shall have the hardware and software capability to support AIML algorithm(s). The APs that support model training may require higher computation capabilities.

# Conclusions and Recommendations

# References

- 11-22/597r3: May 2022 Working Group Motions, May 18, 2022

- IEEE 802.11-REVme D2.0, October 2022

- IEEE P802.11be D2.2, October 2022

- 802.11-18/0818r3, 16 Spatial Stream Support in Next Generation WLAN

- 802.11-20/1877r1, 16 Spatial Stream Support

- 802.11-20/1535r66, Compendium of straw polls and potential changes to the Specification Framework Document Part 2

- 802.11-22/1515, A candidate feature: Multi-AP

- 802.11-22/1394, Virtual BSS And Multi AP Transmissions

- 802.11-22/1516 Considerations on Multi-AP Coordination

- 802.11-22/1512 Multi-AP Coordination for UHR

- M. Deshmukh, Z. Lin, H. Lou, M. Kamel, R. Yang, I. Güvenç, “Intelligent Feedback Overhead Reduction (iFOR) in Wi-Fi 7 and Beyond,” in Proceedings of 2022 VTC-Spring

- P. K. Sangdeh, H. Pirayesh, A. Mobiny, H. Zeng, “LB-SciFi: Online Learning-Based Channel Feedback for MU-MIMO in Wireless LANs, ” in Proceedings of 2020 IEEE 28th ICNP

- P. K. Sangdeh and H. Zeng, “DeepMux: Deep-Learning-Based Channel Sounding and Resource Allocation for IEEE 802.11ax,” IEEE Journal on Selected Areas in Communications, Vol. 39, No. 9, Aug. 2021

- S. Szott, K. Kosek-Szott, P. Gawłoicz, J. T. Gómez, B. Bellalta, A. Zubow, F. Dressler, “WiFi Meets ML: A Survey on Improving IEEE 802.11 Performance with Machine Learning,” IEEE Communication Surveys & Tutorials, Vol.24, Issue 3, Juen 2022

- 802.11-22/0723r1, Further discussion on next generation WLAN

- 802.11-19/0719r1, IEEE 802.11be Channel Model Document

- 802.11-22/1443r0, Wi-Fi Meets ML: Re-thinking Next-generation Wi-Fi Networks

- 11-14-0165-01-0hew-802-11-hew-sg-proposed-par

- 11-18-1231-06-0eht-eht-draft-proposed-par

- 11-22-0030-01-0wng-look-ahead-to-next-generation

- 11-22-0046-01-0wng-next-generation-after-802-11be

- F. Abinader et al., “Performance evaluation of IEEE 802.11n WLAN in dense deployment scenarios”, 2014 IEEE 80th vehicular technology conference (VTC2014-Fall)

- A. Bhattacharya and A. Kumar, “Analytical Modeling of IEEE 802.11-Type CSMA/CA Networks with Short Term Unfairness”, IEEE/ACM Transactions on Networking, Vol. 25, Issue 6, Dec 2017

- W. Wydmański and S. Szott, “Contention Window Optimization in IEEE 802.11ax Networks with Deep Reinforcement Learning”, 2021 IEEE Wireless Communications and Networking Conference (WCNC)

- Z. Guo, Z. Chen, P. Liu, J. Luo, X. Yang and X. Sun, “Multi-agent reinforcement learning-based distributed channel access for next generation wireless networks”, IEEE Journal on Selected Areas in Communications, Vol. 40, Issue 5, May 2022

- Y. Ding, S. Liew and T. Wang, “Non-Uniform Time-Step Deep Q-Network for Carrier-Sense Multiple Access in Heterogeneous Wireless Networks”, IEEE Transactions on Mobile Computing, Vol. 20, Issue 9, Sept 2021

- L. Zhang, H. Yin, Z. Zhou, S. Roy and Y. Sun, “Enhancing Wi-Fi multiple access performance with federated deep reinforcement learning”, 2020 IEEE 92nd Vehicular Technology Conference (VTC2020-Fall)

- 11-22-0723-01-0wng-further-discussion-on-next-generation-wlan

- IEEE 802.11-23/0433r2 - AIML-based Roaming Enhancements Use Case

- 802.11-23/0280r0, ML aided Dual CSI Feedback for Next Generation WLANs

- 802.11-23/0275r2, Improved AIML Enabled Index Based Beamforming CSI Feedback Schemes

- 802.11-23/0290r2, Study on AI CSI Compression

- 802.11-23/0755r0, AIML Assisted Complexity Reduction For Beamforming CSI Feedback Using Autoencoder

- 11-22-1522-01-aiml-drl-based-channel-access

- 11-22-1942-00-aiml-follow-up-on-drl-based-channel-access

- 11-22-1931-01-0uhr-follow-up-on-latency-reduction-with-machine-learning-techniques

- 11-23-0075-01-0uhr-more-discussions-on-deep-learning-for-wlan

- M. Nagel, et al., “A White Paper on Neural Network Quantization”, Qualcomm AI Research, Jun. 2021.

- 802.11-23/0756r1, Neural Network Quantization for Overhead Reduction in AIML Model Sharing

- 11-22/1899 "Multi-AP Operation for Low Latency Traffic Delivery - Follow up"

- 11-22/1512 "Multi AP Coordination and Residential Wi-Fi"

- 11-22/1516 "Considerations on Multi-AP Coordination"

- C. Deng et al., "IEEE 802.11be Wi-Fi 7: New Challenges and Opportunities," in IEEE Communications Surveys & Tutorials, vol. 22, no. 4, pp. 2136-2166, Fourthquarter 2020, doi: 10.1109/COMST.2020.3012715.

- D. Nunez, F. Wilhelmi, S. Avallone, M. Smith and B. Bellalta, "TXOP sharing with Coordinated Spatial Reuse in Multi-AP Cooperative IEEE 802.11be WLANs," 2022 IEEE 19th Annual Consumer Communications & Networking Conference (CCNC), Las Vegas, NV, USA, 2022, pp. 864-870.

- N. Nurani Krishnan, E. Torkildson, N. B. Mandayam, D. Raychaudhuri, E. -H. Rantala and K. Doppler, "Optimizing Throughput Performance in Distributed MIMO Wi-Fi Networks Using Deep Reinforcement Learning," in IEEE Transactions on Cognitive Communications and Networking, vol. 6, no. 1, pp. 135-150, March 2020.

- L. Zhang, H. Yin, S. Roy and L. Cao, "Multiaccess Point Coordination for Next-Gen Wi-Fi Networks Aided by Deep Reinforcement Learning," in IEEE Systems Journal, 2022.

- UPF/ITU-T AI Challenge 2022 edition, https://www.upf.edu/web/wnrg/2022-edition (accessed on 2023-02-21).

- 802.11 UHR Draft Proposed PAR, doc.: IEEE 802.11-22/0078r3.

- P. Imputato, S. Avallone and D. Magrin, "Multi-AP coordination in Wi-Fi 7 exploiting time resources sharing," 2022 IEEE International Mediterranean Conference on Communications and Networking (MeditCom), Athens, Greece, 2022, pp. 166-171, doi: 10.1109/MeditCom55741.2022.9928663.

- Nunez, David, Malcom Smith, and Boris Bellalta. "Multi-AP Coordinated Spatial Reuse for Wi-Fi 8: Group Creation and Scheduling." MedComNet 2023. arXiv preprint arXiv:2305.04846 (2023).

- I. -S. Comşa, S. Zhang, M. Aydin, P. Kuonen, R. Trestian and G. Ghinea, "Enhancing User Fairness in OFDMA Radio Access Networks Through Machine Learning," 2019 Wireless Days (WD), Manchester, UK, 2019, pp. 1-8, doi: 10.1109/WD.2019.8734262.

- I.-S. Comsa, S. Zhang, M. Aydin, P. Kuonen, R. Trestian, and G. Ghinea, “A Comparison of Reinforcement Learning Algorithms in Fairness-Oriented OFDMA Schedulers,” Information, vol. 10, no. 10, p. 315, Oct. 2019, doi: 10.3390/info10100315.

- M. K. Tefera, S. Zhang, and Z. Jin, “Deep Reinforcement Learning-Assisted Optimization for Resource Allocation in Downlink OFDMA Cooperative Systems,” Entropy, vol. 25, no. 3, p. 413, Feb. 2023, doi: 10.3390/e25030413.

- D. Kotagiri, K. Nihei and T. Li, "Distributed Convolutional Deep Reinforcement Learning based OFDMA MAC for 802.11ax," ICC 2021 - IEEE International Conference on Communications, Montreal, QC, Canada, 2021, pp. 1-6, doi: 10.1109/ICC42927.2021.9500628.

- Y. Kihira, Y. Koda, K. Yamamoto, T. Nishio and M. Morikura, "Adversarial Reinforcement Learning-based Robust Access Point Coordination Against Uncoordinated Interference," 2020 IEEE 92nd Vehicular Technology Conference (VTC2020-Fall), Victoria, BC, Canada, 2020, pp. 1-5, doi: 10.1109/VTC2020-Fall49728.2020.9348462.

- S. Verma, T. K. Rodrigues, Y. Kawamoto, and N. Kato. "A Survey on Multi-AP Coordination Approaches over Emerging WLANs: Future Directions and Open Challenges." arXiv preprint arXiv:2306.04164 (2023).