IEEE P802.11
Wireless LANs

| Proposed 802.11bn Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2023-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ming Gan | Huawei Technologies | F3, Huawei Base, Bantian, Longgang District, Shenzhen, China | +8615889743667 | ming.gan@huawei.com |
| David Xun Yang |  |  |  |  |
| Edward Au |  |  |  |  |
| Yan Xin |  |  |  |  |

### Abstract

This submission proposes the 802.11bn Functional Requirements as derived from the 802.11bn PAR [Ref-1] and CSD [Ref-2].

-Rev 0: initial draft

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the 802.11bn amendment, referred to as the TGbn Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is set by the P802.11bn PAR [Ref-1] and CSD [Ref-2].

The functional requirements as stated in this document cover the following aspects of 802.11bn:

- System Performance

- Bands of Operation

- Backward Compatibility and Coexistence

- Compliance to PAR and CSD

  - Notation

Requirements are identified by a preceding unique number in the format of “TGbn Rn”, where n is an integer number representing the ID of the requirements.

- Functional Requirements

  - System Performance

    - Reliability of transmission

- The 802.11bn amendment shall enable at least one mode of operation capable of increasing throughput by 25%, as measured at the MAC data service access point (SAP), in at least one Signal to Interference and Noise Ratio (SINR) level (Rate-vs-Range), compared to the Extremely High Throughput MAC/PHY operation.

- The 802.11bn amendment shall enable at least one mode of operation capable of reducing latency by 25% for the 95th percentile of the latency distribution compared to the Extremely High Throughput MAC/PHY operation.

- The 802.11bn amendment shall enable at least one mode of operation capable of reducing MAC Protocol Data Unit (MPDU) loss by 25% compared to the Extremely High Throughput MAC/PHY operation for a given scenario, especially for transitions between BSSsError: Reference source not found.

    - Legacy Performance

- The legacy performance shall not be significantly degraded by operation in or in proximity of an 802.11bn network.

    - Power Efficiency

- The 802.11bn amendment should improve the STA (including AP) power efficiency measured as average power consumption, compared to the existing IEEE 802.11 standard and its amendments operating in the same deployment scenario.

  - Bands of Operation

- The 802.11bn amendment shall define operations in unlicensed frequency bands between 1 and 7.250 GHz.

  - Backward Compatibility and Coexistence

- The 802.11bn amendment shall enable backward compatibility and coexistence with legacy IEEE802.11 devices in the 2.4 and 5 GHz bands.

- The 802.11bn amendment shall enable backward compatibility and coexistence with IEEE802.11ax and IEEE IEEE802.11be devices in the 6 GHz band.

  - Compliance to PAR and CSD

- The 802.11bn amendment shall comply with the PAR Error: Reference source not found and the CSD Error: Reference source not found.

- References

- 11-23-0480-03-0uhr-uhr-proposed-par

- 11-23-0079-10-0uhr-uhr-draft-proposed-csd