IEEE P802.11
Wireless LANs

| CR for Section 36.3.11.12 on P802.11be D5.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2024.03.10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Oded Redlich | Huawei Technologies |  |  |  |

Abstract

This submission contains proposed comment resolution to CID 22264 on P802.11be D5.0.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

## CID 22264

| Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- |
| 831.59 | 36.3.12.11.1 | The sentence "For an EHT MU PPDU, the U-SIG and the EHT-SIG fields include information on preamble puncturing." is not very accurate because EHT-SIG field does not include information on preamble puncturing in a non-OFDMA transmission. | Replace the sentence with "For an EHT MU PPDU in an OFDMA transmission, the U-SIG and the EHT-SIG fields include information on preamble puncturing. For an EHT MU PPDU in a non-OFDMA transmission, the U-SIG includes information on preamble puncturing." | ACCEPTED |