IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE 802.15.3 MAC primitives for synchronization support |  |
| Date Submitted | 31 October, 2003 |  |
| Source | [Peter Johansson] [Congruent Software, Inc.] [98 Colorado Avenue] | Voice: [(510) 527-3926] Fax: [(510) 527-3856] E-mail: [PJohansson@ACM.org] |
| Re: | Extensions to the IEEE 802.15.3 MAC |  |
| Abstract | Some applications, for example, the transport and rendering of audio or video streams, require synchronized timers shared among different DEVs. An additional requirement might be greater accuracy (in terms of jitter bounds) or finer timer granularity than that provided by DEVs’ local clocks synchronized to the beacon’s Time Token. In support of such applications, this submission proposes a MAC service that enables layers above the MAC to accurately synchronize application-dependent timers shared among DEVs. The service is usable by more than one application within a DEV at a time. |  |
| Purpose | New MAC primitives are proposed for inclusion in a future amendment to or revision of IEEE Std 802.15.3-2003. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

6.3.x Application-dependent timer synchronization

Although timer synchronization methods and accuracy requirements are application-dependent and beyond the scope of IEEE Std 802.15.3-2003, they rely on an indication from each DEV’s MAC that is provided essentially simultaneously to all DEVs within a piconet. The MAC accomplishes this by indicating the occurrence of the beginning of each beacon’s data preamble. The beginning of the data preamble is observed on the air simultaneously by all DEVs within a piconet while the delay between the observation and the delivery of the indication is known within a MAC by design (and communicated to the application by implementation-dependent means). The common reference point in time provided by the beginning of data preamble indication is the essential building block upon which a variety of application-dependent timer synchronization methods may be based.

6.3.x.1 MLME-BEACON-SYNC.request

This primitive requests activation of the MAC synchronization support facility; its semantics are:

MLME-BEACON-SYNC.request	()

The primitive has no parameters.

6.3.x.1.1 When generated

This primitive is generated by the DME when a higher-layer application requires synchronization support from the MAC.

6.3.x.1.2 Effect of receipt

This request activates the MAC synchronization support facility at the DEV. The MLME subsequently generates an MLME-BEACON-SYNC.confirm that reports the results of the activation request. If the request succeeds and the synchronization support facility is activated, the MLME shall generate an MLME-BEACON-SYNC.indication whenever a beacon is received or transmitted.

6.3.x.2 MLME-BEACON-SYNC.confirm

This primitive confirms the activation of the MAC synchronization support facility; its semantics are:

MLME-BEACON-SYNC.confirm	(ResultCode)

The table below defines the primitive’s parameter:

| 6.3.x.2.1 When generated Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, NOT SUPPORTED | Reports the result of the preceding MLME-BEACON-SYNC.request |

This primitive is generated by the MLME as a result of an MLME-BEACON-SYNC.request to activate the MAC synchronization support facility.

6.3.x.2.2 Effect of receipt

The DME is notified whether or not the MAC synchronization support facility is active. The NOT_SUPPORTED result code is returned if the MAC does not support the facility.

6.3.x.3 MLME-BEACON-SYNC.indication

This primitive indicates the beginning of a beacon’s data preamble, whether the beacon is transmitted or received by the MAC; its semantics are:

MLME-BEACON-SYNC.indication	(BeaconNumber)

The table below defines the primitive’s parameter:

| 6.3.x.3.1 When generated Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| BeaconNumber | Integer | 0 – 65535 | The least significant 16 bits of the received or transmitted beacon’s Time Token |

This primitive is generated by the MLME when the beginning of a beacon’s data preamble is detected, as indicated to the MAC by primitives generated by the physical layer. When generated by the PNC’s MAC, the MLME-BEACON-SYNC.indication shall occur when the first symbol of the beacon’s data preamble is transmitted. Otherwise, the indication shall occur when the first symbol of the beacon’s data preamble is received. In both cases, the MLME-BEACON-SYNC.indication provided is simultaneous (within implementation-dependent delay bounds) with the indication provided to other DEVs within the piconet for the same beacon.

6.3.x.3.2 Effect of receipt

The DME is notified of the beginning of data preamble for the beacon identified by BeaconNumber.