IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG4a Technical Requirements |  |
| Date Submitted | 04/03/30 |  |
| Source | [Philippe Rouzet, technical editor] [STMicroelectronics] [Jason Ellis-] [Stacatto] | Voice: [+41 22 929 58 66] E-mail: [philippe.rouzet@st.com] Voice: [] E-mail: [] |
| Re: |  |  |
| Abstract | [Definitions for the proposal evaluation for Study Group 4a] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG4.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.	Introduction	3

2.	Alt-PHY Layer Technical Characteristics summary	4

3.	Topology	4

4.	Bit Rate	5

5.	Range	5

6.	Coexistence and Interference Resistance	5

7.	Channel Model	5

8.	Power Consumption	6

9.	Quality of service	6

10.	Form Factor	6

11.	Antenna	6

12.	Complexity	6

13.	Location awareness	6

14.	Mobility	7

15.	Compliance and or Supplements to 802.15.4 functionality	7

16.	Regulatory matters	7

# Introduction

This document provides the technical content for the project to develop an alternate low bit rate physical layer (alt-PHY). This alt-PHY shall be a amendment to the proposed IEEE 802.15.4 Standard, intended for low bit rate WPAN communication systems. The reference model used  for this alternate phy layer is the following:

Figure 1 – Reference partitioning

This document serves two purposes. It is first a summary of the application presentations from the Study Group 4a call for applications. Second it defines the fundamental requirements implied by applications and summarized in the documents

- ref. 15-03-0489-04-004a-application-requirement-analysis.xls

- ref. 15-03-0442-01-004a-SG4a Technical Requirements Response.doc

- This document associated with the corresponding selection criteria document will guide proposers on how to respond to a call for proposals.

# Alt-PHY Layer Technical Characteristics summary

This project will define an alternative PHY clause for a precision location, extended range, robustness and mobility amendment to standard 802.15.4.]

The intended Alt-PHY aims to support a low complexity, low cost, low power consumption WPAN communication system with precision location, extended range, robustness and mobility. The precision ranging capability, range, robustness and mobility will be improved enough to satisfy an evolutionary set of industrial and consumer needs. The project will address the requirements to support sensor, control, logistic and peripheral networks in multiple compliant co-located systems and also coexistence .

Anticipated high-level characteristics of the alt-PHY layer are summarized as follows.

- Location-awareness is a very important feature of the communication system and precision ranging must be provided by the PHY itself without support by external features.

- The link throughput shall not exceed some tens of Kbps in most of the cases.

- The form factor shall be compatible with the needs of sensor networks or RF tags applications.

- A crucial requirement is that power consumption must allow for self powered operating time without intervention during several months to several years.

- Robustness and interference resistance are key features.

This list of requirements dictates that a new physical layer must be specified.

# Topology

The alt-PHY shall support all types of topologies defined by IEEE 802.15.4 standard in its MAC section.

This includes capability of relaying messages, coordinating cells or aggregated cells or quasi simultaneously concentrating data issued from multiple nodes.

The network configuration has to be highly dynamic. Thus the alt-PHY must be workable without requiring complex static set up procedure and must comply with dynamic insertion and de-insertion of nodes into a network.

Typical applications imply data collection by a unique or a set of coordinated data collectors. Thus the corresponding alt_PHY component may have to sustain a much higher throughput than the ones of the other nodes. The alt-PHY layer must be able to maintain bidirectional links (half duplex)..

# Bit Rate

The bit rate is categorized the following way:

- individual link bit rate. This is related to a peer to peer link, typically between a sensor device and an information collector or between two devices (relaying of information, synchronization, mutual positioning etc…). The effective data bit rate can be very low (less than 1Kbps), is generally a few Kbps and in exceptional situation about 1 Mbps.

- aggregated bit rate. This is typically the bit rate concentrated from many sensor devices to a data collector during a short period of time (can be during specific situations when many devices need to update their information at the same time like alarm or emergency situations). The data collector must be capable of acquiring several Mbps of effective data.

Typical selected figures:

Link bit rate: at least 1 Kbps at PHY-SAP.

Aggregated bit rate (data collector only):at least 1 Mbps at PHY -SAP.

. The transmit EIRP is fixed by regulatory emission limits

# Range

The maximum distance between communicating nodes is generally 0 to 30 m. In some cases, mainly assets tracking, the range has to be extended to several hundreds of meters. Possibly relaying of messages could be used in such situations. In most of the cases the link data rate can be limited to a few Kbps where the range is very large, however the number of nodes is very large (up to thousands) and the data collector needs to absorb large aggregated data rate ( in sustained mode, and particularly in burst mode).

# Coexistence and Interference Resistance

The devices must be able to operate in high noise and high multipath environment (e.g. harsh factory environments).

The alt-PHY must be able to sustain co-channel and out-of-band interference from legal transmitters without significant performance impairment.

Both indoor and outdoor applications have to be considered.

# Channel Model

It is anticipated that the channel environment may be different from the current ones (i.e. those established for 2.4Ghz, 5Ghz, High Bit Rate UWB), due to the high specificities of the considered applications in term of range, environments,.

Outdoor environment has to be taken into account, not necessarily restricted to LOS. Large range is a common characteristic to most of the applications, specific harsh environments need to be considered, e.g. factory environment or large containers, with strong multipath effects.

The alt-PHY must exhibit high rejection to in-channel, co-channel, and out of band interference.

# Power Consumption

The device (complete communication system including alt-PHY and MAC) must operate  while supporting a battery life of months or years without intervention.

Therefore very efficient power saving modes are desirable, in particular for devices that transmit sporadically. In addition the coordination of nodes must not induce frequent wake up of nodes. These mechanisms must be supported by the alt-PHY layer.

# Quality of service

The critical factor is the reliability of the transmission, meaning that strong error correction methods need to be provided at PHY level.

Other Quality of Service parameters have a strong impact on PHY layer:

- Real time communication is required,

- latency may exist but must be controlled (jitter elimination for localization).

- synchronization of nodes (mainly for localization),

-    capability to provide fast reaction in emergency situations.

# Form Factor

The alt-PHY components should be capable of fitting into a form factor consistent with sensor and RF tag applications. The critical point is that this includes the battery and the antenna parts.

# Antenna

Omnidirectional antenna is the nominal requirement. This is due to the environment and the impossibility to predict the position of a device antenna. The antenna must be very robust.

# Complexity

Complexity should be minimal to enable mass commercial adoption for a variety of cost sensitive products. Complexity (gate count, die size) and BOM shall be minimized. In a number of applications, the components are to be considered as throwaway after use.

# Location awareness

This is a mandatory function in most applications.

It can be related to precise (tens of centimeters) localization in some cases, but is generally limited to about one meter.

Localization awareness may result in different application such as precise positioning, localization aided routing, motion tracking (simple detection of an object in a determined area, or moving outside of this area).

This functionality must be built-in the node with basic functions embedded into the alt-PHY and capable of being serviced in a simple and automatic way by higher layers.

It is anticipated that the physical layer must be capable of providing adequate time resolution and jitter elimination to properly exercise the localization awareness functionality, for example by providing services based on message transit time measurement.

# Mobility

This is a mandatory feature related to intra-cell mobility, not to roaming or handover. Nodes shall be capable of reliable communication when in the move, at least for tracking. It is admitted that limited communication performance (e.g. data rate) can be tolerated in such cases. The considered applications may involve pedestrian, industrial vehicle or higher speed mobility

# Compliance and or Supplements to 802.15.4 functionality

It is envisioned that the alt-PHY project will allow supplements to 802.15.4. In addition to the alt-PHY itself, the project may include MAC enhancements, only if those enhancements are necessary to support the selected alt-PHY (see 14 above as an example of possible specific requirement on the MAC).

# Regulatory matters

The alt-PHY standard will comply with necessary geopolitical or regional regulations.