IEEE P802.15

Wireless Personal Area Networks™

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks |  |
| --- | --- | --- |
| Title | P802.15. TG4a Alt PHY Selection Criteria |  |
| Date Submitted | November 03, 2004 |  |
| Source | Jason Ellis Philippe Rouzet | [Jason.Ellis@staccatocommunications.com] philippe.rouzet@st.com |
| Re: |  |  |
| Abstract | 04-0232-12-004a is the repository for the requirements to be used in the selection process for a PHY Draft Standard for P802.15.4a. … |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG P802.15.4a.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1.	Introduction	4

2.	References	5

3.	General Solution Criteria	6

3.1.	Unit Manufacturing Cost/Complexity (UMC)	6

3.1.1.	Definition	6

3.1.2.	Values	7

3.2.	General Definitions	7

3.2.1.	Payload bit rate and throughput	7

3.2.2.	Error rate	8

3.2.3.	Receiver sensitivity	8

3.2.4.	Antenna gain	8

3.2.5.	Band in use	8

3.3.	Signal Robustness	8

3.3.1.	Coexistence and interference mitigation techniques.	9

3.3.2.	Interference Susceptibility	9

3.3.3.	Coexistence	11

3.4.	Technical Feasibility	12

3.4.1.	Manufacturability	12

3.4.2.	Time to Market	12

3.4.3.	Regulatory Impact	13

3.5.	Scalability	13

3.5.1.	Definition	13

3.5.2.	Values	13

3.5.3.	Mobility Values	14

4.	MAC Protocol Supplement	15

4.1.	Alternate PHY Required MAC Enhancements and Modifications	15

4.1.1.	Definition	15

4.1.2.	Values	15

5.	PHY Layer Criteria	16

5.1.	Channel models and payload data to be used in the simulations	16

5.2.	Size and Form Factor	16

5.2.1.	Definition	16

5.2.2.	Values	17

5.3.	PHY-SAP Payload Bit Rate and Data Throughput	17

5.3.1.	Payload Bit Rates	17

5.4.	Simultaneously Operating Piconets	18

5.4.1.	Definition	18

5.4.2.	Values	18

5.5.	Signal Acquisition	22

5.5.1.	Definition	22

5.5.2.	Values	22

5.5.3.	Clear Channel Assesment Values	22

5.6.	System Performance	22

5.6.1.	Definition	22

5.6.2.	Values	23

5.7.	Ranging	23

5.7.1.	Definition	23

5.7.2.	Values	23

5.8.	Link Budget	24

5.8.1.	Definition	24

5.8.2.	Values	24

5.9.	Sensitivity	26

5.9.1.	Definition	26

5.9.2.	Values	26

5.10.	Power Management Modes	26

5.10.1.	Definition	26

5.10.2.	Values	26

5.11.	Power Consumption	26

5.11.1.	Definition	26

5.11.2.	Value	27

5.12.	Antenna Practicality	28

5.12.1.	Definition	28

5.12.2.	Value	28

ANNEX A

1.	Interferers definition	29

1.1.	Microwave Oven	29

1.2.	Bluetooth™ and IEEE 802.15.1 Interferer	29

1.3.	IEEE 802.11b and IEEE 802.15.3 Interferer	30

1.4.	IEEE 802.11a Interferer	30

1.5.	Generic In-band Modulated Interferer	31

1.6.	Generic In-band Tone Interferer	31

2.	Victim receivers definition	32

2.1.	Bluetooth™ (IEEE 802.15.1) Devices	32

2.2.	IEEE P802.15.3 Devices	33

2.3.	IEEE 802.11b Devices	33

2.4.	IEEE 802.11a Device	33

# Introduction

This is the criteria for the selection of the alternate PHY Draft Proposals. In order to accurately and consistently judge the submitted proposals, technical requirements are needed that reflect the application scenarios that were contributed in response to the call for applications.

This working document will become the repository for the requirements to be used in the selection process for a PHY Draft Standard for P802.15.4a.

The document is divided into three main sections: General Solution Criteria, MAC Protocol Supplements Criteria, PHY Layer Criteria.

Document [xxx] provides the down selection process.

This document and the Requirements document [IEEE 15-04-0198-02-004a] provide the technical content for the project to develop an alternate physical layer (alt-PHY). This alt-PHY shall be a supplement to the proposed IEEE 802.15.4 Standard. Revision 0 of this Selection Criteria Document references draft xxx of the IEEE 802.15.4 Standard.

In this document, as per 04/198, the reader will see reference to 1kbps and 1Mbps  at MAC-PHY interface. The associated distance for these data rates are, respectively, 30 and 10 m and a distance given by the presenter. The mentioned data rates are minimums and data rates in the actual proposals may be higher than the minimums.

# References

| Ref. | [date(yy/mm/dd)] | DCN | Title |
| --- | --- | --- | --- |
| [1] |  |  | IEEE 802.15.4 -2003 Standard |
| [2] | [date] | IEEE 15-04-198-02-004a, | TG4a Technical Requirements |
| [3] | [date] | [xxx] | TG4a Down Selection Process |
| [4] | [date] | IEEE 15-04-0535-00-004a, | TG4a Channel Model Final Report |
| [5] | [04/02/10] | IEEE 15-03-0489-05-004a, | application-requirement-analysis.xls |
| [6] | [03/11/] | IEEE 15-03-0442-01-004a | TG4a Alt PHY Selection Criteria Response |
| [7] | [03/09/08] | IEEE 15-03-0537-00-004a | Formal Submission of the 802.15.4IGa informal CFA Response] |
| [8] | [03/ 05/18] | IEEE.15-03-0031-11 | P802.15. TG4a Alt PHY Selection Criteria PAR 5C |

# General Solution Criteria

This section defines the technical and marketing system level concerns of the proposals.

## Unit Manufacturing Cost/Complexity (UMC)

### Definition

The cost/complexity of the device must be as minimal as possible for use in the personal area space, see [03/530]. Fig. 1 illustrates the logical blocks in the transceiver PHY layer. Not all blocks are required to implement a communications system. However, if the functionality is used (even optionally) in the specification, then the complexity for implementing the functionality must be included in the estimate. The order and contents of the blocks may vary, for example, the frequency spreading may be a part of the modulate/demodulate portion, and the encode/decode operations might split out to ‘source encode/decode’ and ‘channel encode/decode’. In addition to the communication blocks, the altPhy devices should be equipped with the functions needed for ranging.

Figure 1: Logical blocks in the transceiver PHY layer

- Encode/Decode – packet formation including headers, data interleaving, error correction/detection (FEC, CRC, etc.), compression/decompression, bias suppression, symbol spreading/de-spreading (DSSS), data whitening/de-whitening (or scrambling).

- Modulate/Demodulate – convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Frequency Spreading/De-spreading – can include techniques to increase or decrease, respectively, the Hz/bit of the analog signal in the channel.

- Transmit/Receive – transition the signal to/from the channel.

### Values

Complexity estimates should be provided in terms of both analog and digital die size estimates, semiconductor processes, specified year for process technologies, gate count estimates, and major external components. Similar considerations should be made with regard to MAC enhancements. Reasonable and conservative values should be given. Relative comparisons to existing technologies are acceptable. Complexity estimates should include the means needed to implement the ranging, including, if needed, synchronization means.

## General Definitions

The general definitions below are applicable to all sections unless a specific one is specified.

### Payload bit rate and throughput

The PHY-SAP payload bit rate is the instantaneous bit rate at PHY-SAP level, during PSDU transfer (net amount of data, after removing the effect of channel coding or other form of redundancy) during apeer to peer transfer. Each proposer is responsible for specifying its nominal PHY-SAP payload bit rate. This value is called X0, is expressed in kb/s, and will used for all assessments defined in this document unless specified differently,

Unless specified differently, all data packets are using a 32 byte PSDU and the nominal PHY preamble which allows to meet the performance requirement as defined in .section 5.4.

In addition the proposer will specify and characterize, when applicable:

- optional PHY-SAP payload bit rates, called Xi

- (if different from X0 or Xi), the the aggregated data rate Y0 and Yi that a specific device could absorb from a number of devices (e.g. a collector device gathering all information from the cell devices transmitting X0 or Xi kb/s),

- The PHY-SAP peer-to-peer data th roughput. This is the net amount of data (PSDU) that is transferred from one PHY-SAP to another over at least 200 packets (PPDU) sent using the above defined conditions. The throughput should include the normal overhead associated with a packet transmission, (id est PPDU preamble and header, interframe spacing or IFS, and if needed associated control frames such as ACK frames). The connection is assumed to have already been established and in progress. Throughput is in kb/sand is called T0 (nominal) and Ti (optional),

- The duty cycle factor which is considered in normal operating mode and the impact on useable peer to peer data throughput.

### Error rate

The error rate criterion is the maximum packet error rate (PER) for a specified packet length. The error ratio should be determined at the PHY-SAP interface, after any error correction methods required in the proposed device have been applied.

Unless specified, the packet error rate is for 32 octet PSDU.

The proposer will be asked to indicate the PER.

### Receiver sensitivity

The receiver sensitivity is the power level of a signal in dBm present at the input of the receiver for which the error rate criteria are achieved in the AWGN environment at a specified bit rate.

The receiver sensitivity is calculated in clause 5.6.3. The proposer should include all the calculations used to determine the receiver sensitivity. The power level should be specified at the receiver antenna connection (that is, 0 dBi antenna gain assumed, with a loss factor of 3 dB).

The minimum required receiver sensitivity used for this requirement is that sensitivity which produces PER less than 1% for 32 octet PSDU when receiving a transmitted signal compliant with regulatory emission levels and producing the above specified mandatory bit rates of X0 kb/s, and optionally the bit rate of Xi kb/s over the respective free space distance of 30, 10 meters and optionally other presenter specified distances.

Devices may exceed the minimum required sensitivity performance; however, the measurements are taken relative to the proposed system receiver sensitivity.

### Antenna gain

Unless otherwise noted, the P802.15.4a transceivers are assumed to use 0 dBi antennas.

### Band in use

The proposers will specify the frequency band to be used for the considered system (both the full band and if it applies, the proposed splitting in frequency channels). The proposers will state if their intention is to use bands available for license exempt systems or not

## Signal Robustness

### Coexistence and interference mitigation techniques.

The alternate PHY needs to operate in an interference environment and may have PHY level attributes that can be adjusted by higher layer management to mitigate interference ingress (interference coming into the alternate PHY) and interference egress (interference caused by the alternate PHY).  The proposers should show what attributes of their proposal can be adjusted to mitigate interference ingress and what attributes of their proposal can be adjusted to mitigate interference egress.  Supporting analysis to indicate the level of ingress/egress mitigation should be provided.  The actual algorithms for making these adjustments is beyond the scope of the alternate PHY effort.

### Interference Susceptibility

#### Definition

Interference susceptibility refers to the impact that other co-located intentional and unintentional radiators may have on a proposed alt-PHY. This section is mainly concerned with the interference coming from other non-P802.15.4a devices. Although there may be a number of systems radiating RF energy in the environments envisioned for P802.15.4a systems, the interference from WLANs (2.4 GHz and 5 GHz), other WPANs (such as 802.15.1, 802.15.3, and 802.15.4), cordless phones (2.4 GHz and 5 GHz), cellular phones, and microwave ovens will be the primary cases considered. Interference from a generic UWB device (FCC compliant) must also be specified if it is applicable.

#### Interference Model

The following interferers will be considered:

- Microwave Oven

- IEEE 802.15.1 (Bluetooth)

- IEEE 802.11b,g

- IEEE 802.15.3

- IEEE 802.11a

- IEEE 802.15.4

- Interference from intentional or unintentional radiators

Although other wireless systems may be present, the above systems represent a broad representative set of interferers whose impact has been determined to be sufficient for the evaluation of the proposed alt-PHY solutions based upon the IEEE P802.15.SG4a target applications. Since this document is concerned only with evaluating the capabilities, complexities, and performance implications of proposed physical layers, it is sufficient to use generic models of the above systems in order to ease the burden on the proposers.

Representative models as described in annex 1 are suggested.

      - Evaluation Method and Minimum Criteria

The following subsections describe how the above models can be used for evaluating the performance impact on the proposal. Since the performance of these systems may depend on particular receiver designs, and it is not the intent to standardize receiver designs, the proposer should describe any special circuits that were needed to obtain these results (e.g., interference suppression algorithms, notch filters, steep roll-off filters, etc.). Also, all of the following tests should be done using the nominal system configuration which provides X0 Kb/s payload bit rate as specified in section 3.2.1. and a PER < 1% for packets with 32 byte PSDU

- Microwave Oven,IEEE 802.15.1,IEEE 802.11b,g, IEEE 802.15.3,IEEE 802.11a, IEEE 802.15.4

Using simulation results, analysis, or technical explanations, describes the impact of each of the above mentioned ISM band interferers on the proposed system performance when operating at 6 dB above the proposed systems receiver sensitivity level. This impact should either be a reduction in data throughput or rise in the PER. Interferers are considered separately.

Minimum criteria: Proposed system should be able to meet the PER criteria, when the interference is present at a distance separation of 1 meters from the receiver. If this criteria cannot be met, proposers should define the minimum separation distance between the interferer and the proposed system at which the PER criteria of 1% can be achieved when operating at 6 dB above receiver sensitivity.

In addition proposers can also provide results using different distance separation.

- Generic In-band Modulated Interferer and Generic In-band Tone Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average received interference power, PI, that can be tolerated by the receiver, after it has executed any interference mitigation algorithms, while still maintaining the PER criteria. Proposer should show results for a number of different center frequencies or describe how the performance changes as the center frequency changes.

Minimum criteria: PI−Pd > 3 dB (Pd is the received power which is defined here as 6 dB above the receiver sensitivity level) If this criteria cannot be met, proposers should specify the necessary PI−Pd to meet the PER criteria

- Out-of-Band Interference from Intentional or Unintentional Radiators

Proposers should report the minimum out-of-band rejection in dB provided by the proposed system. This will provide a minimum standard for out-of-band interferer immunity.

### Coexistence

#### Definition

Coexistence, in this context, refers to the co-location of IEEE P802.15.4a devices with other, non-P802.15.4a devices. The criteria described in this section focuses only on the impact the P802.15.4a devices have on other non-P802.15.4a devices that may be sharing the same frequency bands. The impact of the non-P802.15.4a devices on a P802.15.4a receiver is addressed in clause 3.2.2.

#### Coexistence Model

The following victim receivers which may be co-located with P802.15.4a devices, will be considered here:

- Bluetooth™ (IEEE 802.15.1)

- P802.15.3

- IEEE 802.11b,g

- IEEE 802.11a

- IEEE 802.15.4

Although other wireless systems may be present, the above systems represent a broad representative set of systems whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE P802.15.SG4a target applications.

Each of the victim receivers listed above operates in unlicensed spectrum and, according to FCC, 47 C.F.R. Sec. 15.5(b), may not cause and must accept harmful interference. For this reason these systems have been specified to operate in the presence of other devices sharing the same spectrum. The P802.15.4a coexistence model is consistent with this principle, limited to devices sharing the same frequency band of operation.

For example, proposers using the 5 GHz ISM band are required to show coexistence with 802.11a, not with 802.11b; proposers using the 2.4 GHz ISM band are required to show coexistence with 802.11b, not with 802.11a; proposers using UWB in the 3.1-10.6 GHz bands are required to show coexistence with 802.11a if their system intentionally emits power in the 5 GHz U-NII (Unlicensed National Information Infrastructure) band, not with 802.11b.

The coexistence model, evaluation method and criteria are based on victim receiver’s performance in presence of P802.15.4a transmitters partially or totally sharing the same frequency of operation, not on P802.15.4a transmit power. This model is consistent with FCC interference recommendations, described in Spectrum Policy Task Force report, ET Docket No. 02-135, Nov 2002.

Document ref. [04] describes in more detail the reference systems that must be considered by each PHY proposal.

#### Evaluation Method and Minimum Criteria

In order to simplify the criteria, the Interfering Average Power generated by the 802.15.4a transmitter and measured in the relevant bandwidth of the victim receiver at any frequency at which that receiver operates should be used as a parameter to evaluate the coexistence capability of the proposed PHY. This power received by a 0 dBi antenna at the victim receiver frequency should be calculated at 1 m and 0.3 m distance separation between 802.15.4a transmitter and victim receiver.

## Technical Feasibility

This is intended to determine if the proposal is technically achievable or academic. Any proposal may be submitted, but demonstrated feasibility and manufacturability should receive favor over equal but untested proposals. Proposers will be asked to comment on criteria listed in the following sections.

### Manufacturability

#### Definition

Manufacturability is defined in terms of the use of mature, cost effective manufacturing processes with evidence of effective mass production capability.

#### Values

The proposers are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations. Globally accepted concepts that will be quick to market, with little risk will be favored.

### Time to Market

#### Definition

Time to Market addresses the question of when the proposed technology will be ready for integration.

#### Values

The proposal shall include an estimate of a schedule for when the PHY would be available for integration.

### Regulatory Impact

#### Definition

The proposal should specify to which geopolitical regions it applies and identify any applicable requirements with which it conflicts. Merit will be awarded for proposals with regulatory compliance of wider geopolitical scope.

#### Values

The proposer may state in which regions the proposal is in regulatory compliance, and if local regulation permits license exempt use of the considered spectrum.

Specific conflicts and potential derogations may be detailed.

## Scalability

### Definition

Scalability refers to the ability to adjust important parameters, such as those mentioned below, (if they are required by the applications) without rewriting the standard. Scalability should address evolutionary extensions to this proposal and lower or higher throughput modes of operation.

The proposers should describe "PHY level hooks" that can be used by a cognitive upper layer to modify the emissions (Cognitive Radio). The MAC should be able to support the scaling of the PHY (for example: 1 Mb/s at the PHY-SAP, with possibility to scale the payload bit rate down to 1kb/s with performance benefits such as power consumption etc.). Anticipated PHY mechanisms that will allow use of the scalability must be detailed. Criteria such as TPC (Transmit power control) or DFS (Dynamic Frequency Selection) –or more generally dynamic channel selection- may also be considered.

### Values

Scalability parameters may include, amongst others: power consumption, payload bit rate and data throughput both measured at the PHY-SAP, channelization (physical or coding), complexity, range, frequencies of operation, occupied bandwidth of operation, and other functions deemed appropriate. Proposers are encouraged to show power consumption levels scaling with reduced or extended ranges and reduced or increased bit rates. Proposers are further encouraged to show scalability up to 1 Mb/s and beyond, as well as 1 kb/s and below, when applicable.

There are a wide variety of applications presently being considered by the 802.15.4a standards committee; some of which can greatly benefit from very specific optimizations; such as very low power consumption, long range, higher speed of mobility, etc. It is requested that the proposers bear in mind the applications the technology is intended to serve. When preparing your contribution, please be aware that a proposal that is flexible for optimization for a number of different applications will likely be very well received by the committee. The 802.15.4a PAR specifies improved performance on a number of different parameters, though it is not expected that all performance improvements will happen within the same mode of operation, results should be offered for all the modes, hence include performance improvements for all parameters. These applications are categorized in great detail in document 03/0442r1 as well as in the applications requirements analysis spreadsheet, document 03/489r4; and include the following applications: Safety / Health Monitoring, Personnel Security, Logistics, Industrial Inventory Control, Industrial Process Control and Maintenance. Home Sensing, Control and Media Delivery, and other communication systems.

### Mobility Values

Proposals should determine the maximum relative speed that the proposed PHY will accommodate for both ranging and data communication.

For ranging, a possible process is described below at end of 3.5.2. For data communication, the proposer should explicit the conditions under which such relative speed is given (PER, channel environment, …)

# MAC Protocol Supplement

## Alternate PHY Required MAC Enhancements and Modifications

### Definition

Supplements and modifications to the MAC may be required to accommodate the alternate PHY. It is preferred that the supplements be additions which expand the solution capability as opposed to changes in the MAC that represent an alternative way to do a particular function.

### Values

Proposals should justify and explain the supplements that may be necessary in support of additional features for the alternate PHY.

Proposals should justify and explain the modifications that may be necessary to support or enhance operation of the alternate PHY.

# PHY Layer Criteria

## Channel models and payload data to be used in the simulations

Pre-specified channel models and channel realizations will be provided for all simulations requiring the use of channel models (simulation with interfering links, ranging). They are defined in doc. ref.[4].

Five different models will have to be used:

- X1 (one sentence to characterize)

- X2 (one sentence to characterize)

- X3(one sentence to characterize)

- X4 (one sentence to characterize)

- X5 (one sentence to characterize)

However, if all simulations cannot be performed for the first round of presentation, this list has to be considered as a prioritized list, with maximum priority given to channel model X1.

Conversely to the channel models, packet payload drawings used in the simulations have to be made in a pseudo random fashion by each proposer. PPDU preamble and header must correspond to the definition given by the proposers. The drawings used for each simulations leading to exposition of results must be kept available by proposers for further simulation replays.

## Size and Form Factor

### Definition

Devices specified under 802.15 TG4a are envisioned to be small, simple, low power transceivers. These transceivers are capable of forming mobile, low power communications networks with precise range determination between network nodes. These devices should be designed with sufficient features that an application can send information to any node, receive information from any node and be able to determine the location of any node. These devices should be designed that they can be installed by relatively low-skilled technicians and require little operator intervention post installation. They should operate with as flexible and reliable radio channel as possible to enable applications in more environments and ease of installation.

Applications that are suited to these devices are those that generally require very long battery life (low-power consumption), location awareness, true wireless operation, and true autonomous operation (no operator intervention). These applications include sensor networks, location devices for personnel, control elements for machines and location/identification devices for objects.

Applications that are not suited to these devices are those that require very long range (in excess of 1km), high data rates (in excess of 1Mbit/sec) and frequent operator intervention.

### Values

Proposers shall provide a time line estimate of when their proposed PHY and the P802.15.4 MAC will fit into the proposed form factors such as SD Memory.

## PHY-SAP Payload Bit Rate and Data Throughput

### Payload Bit Rates

#### Definition

- SIFS, LIFS and Packet Overhead

For each of the proposed rates the proposer should provide the SIFS, LIFS and packet overhead (PHY Preamble + PHY header) as illustrated in Fig. 3:

PPDU

P

PHY HEADER

MPDU

T preamble                   T header

Figure 3 Packet overhead parameters for data throughput comparison

Time values for SIFS, LIFS and Packet Overhead should be stated in microseconds. Those values are used in the throughput determination as defined in section 3.2. They must be provided for the nominal payload bit rate (X0) and if applicable for optional payload bit rates of Xi. The PHY preamble is the one specified for meeting the performance requirement of section 5.4

## Simultaneously Operating Piconets

### Definition

The proposed PHY should operate in the close proximity of multiple uncoordinated piconets, at specific error rates. For the purpose of this clause, the term "uncoordinated" will refer to the fact that the PAN coordinators do not coordinate their beacons, and/or their GTS allocations, and/or their macSuperframeOrder."

Proposers will show how their proposal supports nearly orthogonal channels and evaluate the corresponding degradation at RX side during simultaneous transmission. It is recommended that a minimum of 4 nearly orthogonal channels be supported.

### Values

Proposers should evaluate the effect of simultaneously operating piconets for the following specified parameters:

- Packet defined as in 3.2.1

- Nominal PHY-SAP bit rates (X0 kb/s)

- Interference over the whole reference packet duration and uniformly random initial symbol alignment between reference link and interferers

- Meet the baseline performance as indicated in clause 5.5.

- A 0 dBi antenna gain is assumed unless otherwise stated in the channel model document ref. [4].

- Piconets are uncoordinated.

Using the dref distance as defined later in this section (see Single Co-channel separation distance test procedure step one)

- the proposer should indicate the values of dint that cause the PER to degrade to 1%

Figure 4 Test geometry for simultaneously operating piconets

Evaluation geometry and procedure

An interfering transmitter is an uncoordinated transmitter operating at the same power as the reference transmitter. There are two cases to be considered: (1) a co-channel interferer, occupying the same channel and (2) channel interferer, occupying worst case non co-channel, (e.g code channels with the highest cross correlation by code, or adjacent frequency channels). For simplification these channels will be called adjacent channels in the present document.

Single co-channel interferer separation distance is defined as the threshold distance separation (dint) of an interfering co-channel transmitter from the test receiver such that the test receiver PER degrades to a specified error rate.

Multiple adjacent channel interferers separation distance is defined as the threshold distance separation (dint) of multiple interfering transmitters on different adjacent channels equidistant from the test receiver such that the test receiver PER degrades to a specified error rate.

Single Co-channel separation distance test procedure

1. 	Establish a test link with a test receiver at a fixed distance from the reference transmitter, such that the receiver power is 6 dB above the receiver sensitivity level (this gives dref). Continue by sending packets to the test receiver using the reference channel realizations as defined in doc. XXX (channel reference doc). Each channel realization provided by doc. Ref [4] is normalized to unity multipath energy.

2. 	Verify PER at the test receiver.

3. 	Begin transmitting with a single co-channel interfering alt-PHY transmitter at a large distance from the test receiver. The pre-specified channel models and channel realizations from doc. ref.[4] will be used for the interfering links. The simultaneous piconet operation shall be assessed for each of the specified interference channels.

4. 	Continue PER verification at the test receiver.

5. 	Incrementally move the co-channel interfering alt-PHY transmitter closer to the test receiver until the PER exceed 1%.

6. 	Record the distance associated with the last acceptable PER as the single-channel separation distance (dint) for the selected test receiver.

Multi-channel separation distance test procedure

1. 	Use the same procedure as defined above (single co-channel step 1) except that for the N=2 and 3 case the interferers are free space and the reference link is to use the previously mentioned channels.

2. 	Verify PER at the test receiver.

3. 	Begin transmitting with N different adjacent channel interfering alt-PHY transmitters at a large distance from the test receiver.  The proposer should consider the cases N equal 1, 2, and 3.  Channel realizations are given in document XXX.  The energy of each realization is normalized to unity.  For the N=2 and 3 case the interferers are free space and the reference link is to use the previously mentioned first 4 normalized channels.

4. 	Continue PER verification at the test receiver.

5. 	Incrementally move the N different adjacent channel interfering alt-PHY transmitters closer to the test receiver uniformly until the PER exceed 1%.

6. 	Record the distance associated with the last acceptable PER as the multi-channel separation distance (dint) for the selected test receiver.

[The channel model report may suggest some minor wording changes to this section.]

## Signal Acquisition

### Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### Values

The proposer should provide the false alarm probability and the miss detect probability for the proposed preamble design in both AWGN and the environment specified by the channel model document ref. [4]. The proposer should consider both the single piconet and multiple uncoordinated piconet environment. The proposer should indicate a time-line showing the overall acquisition process, according to the preamble resources devoted to acquisition as specified in this document, at the payload bit rates and ranges specified in the TG4a Technical Requirements document ref. [2] subject to the channel model provisions as described in the channel model document ref. [4]. Additional information concerning how well the acquisition process scales with payload bit rate would be beneficial.

### Clear Channel Assesment Values

The proposers should provide the the false alarm probability and the miss detect probability for the Clear Channel Assesment mechanism consistent with the requirements of the IEEE 802.15.4 in both AWGN and the environment specified by the channel model document ref. [4]. The proposer should indicate the time required for Clear Channel Assesment.

## System Performance

### Definition

System performance refers to the ability of the system to successfully acquire and demodulate data packets at the required data rates and bit and packet error rates, both in the free space AWGN channel and in the multipath channels specified by the channel model document ref.[4].

Performance of the proposed system as measured in various multipath environments defined in the latest revision of the channel model document ref.[4]. The multipath performance can be used to evaluate the losses incurred due to multipath. The simulations will be conducted with the following parameters:

- For each of the 5 channels models defined in the channel model document ref. [4], the proposer will be asked for average PER over a minimum of 1000 sent packets, as defined here after, , with length defined in clause 3.2.1.

- For each of those 5 channel models, 100 different, normalized, pseudo-random channel realization are to be extracted from a fixed list as defined in the channel model document ref. [4].

- For each pseudo-random channel realization, a new set of 10 different packet random drawings have to be made by the proposer.

- No shadowing is required for this simulation.

[The channel model report may suggest some minor wording changes to this section.]

### Values

The proposer should provide the probability of average PER (the ability to acquire and pass data with the specified packet length at minimum payload bit rates for the PHY-SAP for both AWGN and the channel model specified in document ref. [4], relative to distance). The proposer should further indicate the range at which the proposed PHY can acquire and meet the bit rate packet length and PER requirements of [04/198] for the channel model specified in document ref. [4]. The proposer should indicate PER and acquisition performance as a function of the distance. The acquisition parameters (signaling and duration) should be noted for all scenarios. [The channel model report may suggest some minor wording changes to this section.]

## Ranging

### Definition

Ranging is a feature performed at the PHY level in order to give information to the upper layers for delivering location awareness related information. Location awareness is the ability to determine information about the relative location of one device with respect to another or set of others.

The purpose is to improve usability of portable devices. This data can be used to locate, identify and discriminate amongst users, possibly on the move, in crowded environments and to simplify device registration in constantly changing network topology. Provisions must be made to ropagate ranging information to a suitable management entity

### Values

1-D location

Proposers should show that they have the capability to estimate the 1-D location of devices (i.e. ranging) and the level of accuracy that can be achieved, and the time and energy required to achieve that accuracy, using all specified channel models. Results should show maximum error for 90% of the node placements and the time needed to get the result. Proposers should indicate if they can handle only static scenarios or if they can handle both static and mobile scenarios. If mobile location awareness is handled (i.e. ranging) then the proposer should indicate the maximum speed that can be accommodated and the location accuracy at that speed.

The following procedure should be followed to determine ranging performance. Depending on the method selected by the proposers, deviations from this method can be necessary. It is expected that the proposers will justify their specific ranging method and provide the adapted procedure:

- Static nodes

1. Place a reference node (or a set of reference nodes if more than one is required)

2. For each channel model as defined in the channel model document ref. [4]:

3. Place a node at the range at which the proposed PHY can acquire and meet the bit rate packet length and PER requirements of [04/198] for the channel model specified in document ref. [4], as previously defined in clause 5.5.2.

4. Estimate the range to the reference node using the proposer's ranging technique, using the PPDU format and content according to the selected ranging method.

5. Repeat the test for 100 channel realizations as defined in the channel model document ref.[4].

6. Make a histogram of the estimations. Report the range error which is larger than 90% of the range errors (i.e. larger than the range estimation errors in 90 of the channels). Report the time or number of packets spent during range estimation in each of the channel realizations. During this time the channel stays constant.

7. Decrease the distance by factor of 2 and repeat steps 3-7. Stop when the ranging error reaches the error floor (i.e. not dependent on SNR anymore).

- Moving Nodes

It is left to the proposer to specify the maximum node speed, however the speed of moving the device can be deduced from the time needed to do the measurement in real time.

## Link Budget

### Definition

Link budget is used to determine proposal capabilities under certain operating conditions for the standards specified bit rates, ranges, and bit error rate.

### Values

The table below, see [02/490], identifies the necessary parameters and equations that should be used to compute the final link margin. Proposers should complete this link budget table and identify and explain all assumptions. Although the proposers may need to make minor alterations to this table to more adequately reflect their proposal, the table identifies the minimum expected level of thoroughness, detail, and justification.

| Parameter | Value | (optional) Value |
| --- | --- | --- |
| peak payload bit rate (Rb) | X0 kb/s | Xi Kb/s |
| Average Tx power (PT) | dBm | dBm |
| Tx antenna gain (GT) | 0 dBi | 0 dBi |
| fc'=fminfmax: geometric center frequency of waveform (fmin and fmax are the -10 dB edges of the waveform spectrum) | Hz | Hz |
| Path loss at 1 meter (L1=20log10(4πfc'c)) c=3×108 m/s | dB | dB |
| Path loss at d m (L2=20log10(d)) | 29.54 dB at d=30 meters | 12 dB at d=4 meters |
| Rx antenna gain (GR) | 0 dBi | 0 dBi |
| Rx power (PR=PT+GT+GR−L1−L2 (dB)) | dBm | dBm |
| Average noise power per bit (N=−174+10*log10(Rb)) | dBm | dBm |
| Rx Noise Figure (NF) | 7 dB | 7 dB |
| Average noise power per bit (PN=N+NF) | dBm | dBm |
| Minimum Eb/N0 (S) | dB | dB |
| Implementation Loss1 (I) | dB | dB |
| Link Margin (M=PR−PN−S−I) | dB | dB |
| Proposed Min. Rx Sensitivity Level2 | dBm | dBm |

1 Implementation loss is defined here for the AWGN channel only, and could include such impairments as filter distortion, phase noise, frequency errors, etc.

2 The minimum Rx sensitivity level is defined as the minimum required average Rx power for a received symbol in AWGN, and should include effects of code rate and modulation.

## Sensitivity

### Definition

Sensitivity is defined in Error: Reference source not found. It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### Values

The proposal should indicate the power level at which the error criterion is met, consistent with the link budget as presented in section 5.6. PER should be 1% or less as defined in section 3.2.3.

## Power Management Modes

The ability to reduce power consumption for devices compliant with this standard is important.

### Definition

Power management modes and protocols allow device sleep, wakeup, and poll. The IEEE 802.15.4 standard provides such power management capabilities.

### Values

The proposal should explain if it supports each of the power management methods as defined in the IEEE 802.15.4 standard.

## Power Consumption

### Definition

Power consumption is defined as the total average power required by the proposed system to operate in each of the following operations:

- transmit,

- receive,

- clear channel assessment,

- power saving modes.

It includes the power consumed by all components necessary to implement all of the functionality of the proposed alternate PHY from the PHY-SAP interface, defined in the IEEE 802.15.4 standard, down to an antenna, where the gain is disclosed by the proposer. No components supporting operation above the PHY- SAP interface are included in the average power consumption value. Peak current should also be stated.

- Transmit

Power consumption during transmit state is defined as the average power consumed from the PHY-TX-START.request for a given MPDU, to the PHY-TX-END.confirm.

- Receive

Power consumption during receive state is defined as the average power consumed from the PHY-RX-START.request for a given MPDU, to the PHY-RX-END.indication where the PHY-RX-START.request is assumed to be coincident with the remote transmission beginning.

- Clear Channel Assessment

Power consumption during clear channel assessment (CCA) is defined as the average power consumed from the PHY-CCA-START.request to the PHY-CCA-END.confirm.

- Power Save

Power consumption during the power save state is defined as the power consumed from the PHY-PS.request to the PHY-PS.confirm resulting from a subsequent PHY-PS.request with a PSLevel value of 0. Methods for achieving power save modes and the impact to the operation (acquisition, time to come ‘awake”, etc…) of the PHY should be described.

### Value

Power consumption values are to be disclosed with sufficient explanation of how the numbers are derived. These numbers should reflect operation at the RF power necessary to achieve the continuous full bit rate/throughput at the maximum range including the disclosed antenna gain. To help aid comparison among proposals, disclosure should include parameters such as technology process, clock rate, voltage, etc.

- Transmit

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

- Receive

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

- Clear Channel Assessment

The proposal should state the estimated power consumed during both channel "busy" periods and channel "idle" periods.

- Power Save

The proposal should specify the power consumption associated with the lowest supported power consumption level (PwrMgtLevel). The proposal should also provide estimated values for power save group parameters as specified in IEEE 802.15.4. Proposals should provide justification for their stated power save values (for example, circuits disabled, clocks turned off, etc…).

## Antenna Practicality

### Definition

The antenna form factor should be consistent with a form factor such as SD Memory.

### Value

Antenna form factor should be described with reference to expected size. Any additional information the proposer desires to provide on the antenna such as size, frequency response, impulse response and radiation characteristics would be beneficial. Any additional requirements on the antenna(s) should be described in the proposal.

ANNEXE A

# Interferers definition

## Microwave Oven

The microwave oven is modeled as transmitting at an EIRP of 100 mW with an active period of 8 ms, followed by a dormant period of 8 ms. That is, during the active period the transmit power is 100 mW and during the dormant period the transmit power is 0 mW. During the active period, the microwave oven output can be modeled as a continuous wave interferer with a frequency that moves over a few MHz. At the beginning of the active period, the frequency is 2452 MHz, and at the end of the active period, the frequency is 2458 MHz. There is a continuous sweep in frequency as the active period progresses in time. Pseudorandom data should be used for the modulation of the interferers.

## Bluetooth™ and IEEE 802.15.1 Interferer

This model is intended to represent the impact of a Bluetooth™ or 802.15.1 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx power | 0 dBm |
| Tx antenna gain | 0 dBi |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -40 dBm |
| (2) at 0.3 meters | -29.6 dBm |

## IEEE 802.11b and IEEE 802.15.3 Interferer

This model is intended to represent the impact of an 802.11b or 802.15.3 device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | QPSK |
| Tx power | 20 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 40 dB |
| (2) at 0.3 meters | 29.6 dB |
| Rx power (1) at 1 meter | -20 dBm |
| (2) at 0.3 meters | -9.6 dBm |

## IEEE 802.11a Interferer

This model is intended to represent the impact of an 802.11a device. The following table identifies the parameters of this interferer at the receiving antenna of the proposed 802.15.SG3a system. Pseudorandom data should be used for the modulation of the interferers.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 16.6 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM OFDM 52 312.5 KHz |
| Tx power | 15 dBm |
| Tx antenna gain | 0 dBi (handset) |
| Path loss (1) at 1 meter | 46.9 dB |
| (2) at 0.3 meters | 36.5 dB |
| Rx power (1) at 1 meter | -31.9 dBm |
| (2) at 0.3 meters | -21.5 dBm |

## Generic In-band Modulated Interferer

For ultra-wideband based proposals, there may be other wireless systems that may be near the 802.15.SG3a system that could cause in-band interference. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape. The following table specifies the relevant parameters:

| f0 | Within the bandwidth of the proposal |
| --- | --- |
| 1TI | 5 MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

## Generic In-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π. For evaluation, f0 should be chosen to be within the bandwidth of the proposal.

# Victim receivers definition

Each of the considered victim receivers operates in unlicensed spectrum and, according to FCC, 47 C.F.R. Sec. 15.5(b), may not cause and must accept harmful interference. For this reason these systems have been specified to operate in presence of other devices sharing the same spectrum. The 802.15.3a coexistence model is consistent with this principle, limited to devices sharing the same frequency band of operation.

For example, proposers using the 5 GHz ISM band are required to show coexistence with 802.11a, not with 802.11b; proposers using the 2.4 GHz ISM band are required to show coexistence with 802.11b, not with 802.11a; proposers using UWB in the 3.1-10.6 GHz bands are required to show coexistence with 802.11a if their system intentionally emits power in the 5 GHz U-NII (Unlicensed National Information Infrastructure) band, not with 802.11b.

The coexistence model, evaluation method and criteria are based on victim receiver’s performance in presence of 802.15.4a transmitters sharing the same frequency of operation, not on 802.15.3a transmit power. This model is consistent with new FCC interference recommendations, described in Spectrum Policy Task Force report, ET Docket No. 02-135, Nov 2002.

The following sections describe in more detail the reference systems that must be considered by each PHY proposal.

## Bluetooth™ (IEEE 802.15.1) Devices

This model is intended to represent a Bluetooth™ (802.15.1) WPAN™ device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 1 MHz |
| Modulation | GFSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -70 dBm |

## IEEE P802.15.3 Devices

This model is intended to represent a high rate P802.15.3 WPAN™ device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | DQPSK |
| Tx Power | 0 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -75 dBm |

## IEEE 802.11b Devices

This model is intended to represent an 802.11b WLAN device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 2.4 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation | CCK (11 Mb/s) |
| Tx Power | 20 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -76 dBm |

## IEEE 802.11a Device

This model is intended to represent an IEEE 802.11a WLAN device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 5.3 GHz |
| --- | --- |
| Baud rate | 11 MHz |
| Modulation Number of carriers Carrier spacing | 16-QAM Coded-OFDM (24 Mb/s mode) 52 312.5 KHz |
| Tx Power | 15 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -74 dBm |