IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | BIFS and pBackoffSlot |  |
| Date Submitted | [May 12, 2004] |  |
| Source | [Jin-Meng Ho] [Texas Instruments] [12500 TI Boulevard Dallas, TX 75243] | Voice: [214-480-1994] E-mail: [jinmengho@ieee.org ] |
| Re: | [IEEE Std 802.15.3-2003] |  |
| Abstract | [This document describes proposed corrections to the definition of BIFS and pBackoffSlot as specified in IEEE Std 802.15.3-2003.] |  |
| Purpose | [The purpose of this document is to propose changes to IEEE Std 802.15.3 to improve compatibility, performance and clarity in the standard.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Introduction

The definitions for BIFS and pBackoffSlot as given in Table 72 of IEEE Std 802.15.3-2003 appear to be incorrect.  This document illustrates the roles and requirements of these two timing parameters, and correct their definitions accordingly.

# Corrections

The proposed corrections are to be made to the definitions for BIFS and pBackoffSlot as specified in Table 72 of IEEE Std 802.15.3-2003.

Table 72—IFS parameters

| 802.15.3 MAC parameter | Corresponding PHY parameter | Definition |
| --- | --- | --- |
| MIFS | pPHYMIFSTime | 11.2.7.4 |
| SIFS | pPHYSIFSTime | 11.2.7.2 |
| pBackoffSlot | pPHYSIFSTime + pCCADetectTime | 11.6.5 |
| BIFS | pPHYSIFSTime + pCCADetectTime | 11.2.7.2, 11.6.5 |
| RIFS | 2*pPHYSIFSTime + pCCADetectTime | 11.2.7.2, 11.6.5 |

The following figure illustrates the hierarchy of BIFS:

Frame

ACK

A contending DEV cannot correctly determine

the medium busy/idle status prior to this point.

ACK

SIFS

pCCADetectTime

CCA_Status

Busy

BIFS = SIFS + pCCADetectTime + MACDelay + PHYDelay

SIFS

pCCADetectTime

MACDelay

TX_EN

Frame (on air I/F)

PHYDelay

A contending DEV senses the medium to be idle at this point,

decrements its backoff counter to 0 (as it happens to be),

and initiates a frame transmission.

SIFS

pCCADetectTime

If the medium remains idle for SIFS+pCCADetecTime,

the MAC (of a contending DEV)

prepares for a new transmission.

BIFS

It takes the MAC up to MACDelay time

to determine it can initiate a frame transmission

and request the PHY to start a transmission

It takes the PHY up to PHYDelay time

to transmit the frame onto the air interface.

Should the DEV transmit a frame

onto the air interface

later than this point (BIFS boundary),

other contending DEVs would not detect

this transmission in the next backoff slot

and could initiate their own transmission

--hence resulting in a collision.

Slot

boundary

The following figure illustrates the hierarchy of pBackoffSlot:

Frame

ACK

A contending DEV cannot correctly determine

the medium busy/idle status prior to this point.

ACK

pCCADetectTime

CCA_Status

Busy

pCCADetectTime

MACDelay

TX_EN

Frame (on air I/F)

PHYDelay

A contending DEV senses the medium to be idle at this point,

decrements its backoff counter to 0 (as it happens to be),

and initiates a frame transmission.

Slot

boundary

pCCADetectTime

If the medium remains idle for BIFS+pCCADetecTime,

the MAC (of a contending DEV)

prepares for a new transmission.

It takes the MAC up to MACDelay time

to determine it can initiate a frame transmission

and request the PHY to start a transmission.

It takes the PHY up to PHYDelay time

to transmit the frame onto the air interface.

Should the DEV transmit a frame

onto the air interface

later than this point (slot boundary),

other contending DEVs would not detect

this transmission in the next backoff slot

and could initiate their own transmission

--hence resulting in a collision.

pBackoffSlot

Slot

boundary

Slot

boundary

Slot

boundary

Slot

boundary

pBackoffSlot = pCCADetectTime + MACDelay + PHYDelay

BIFS = SIFS + pBackoffSlot

Based on the above analysis, Table 72 is corrected as follows.

Table 72—IFS parameters

| 802.15.3 MAC parameter | Corresponding PHY parameter | Definition |
| --- | --- | --- |
| MIFS | pPHYMIFSTime | 11.2.7.4 |
| SIFS | pPHYSIFSTime | 11.2.7.2 |
| pBackoffSlot | pCCADetectTime + MACDelay + PHYDelay | 11.6.5 |
| BIFS | SIFS + pBackoffSlot | 11.2.7.2, 11.6.5 |
| RIFS | 2*pPHYSIFSTime + pCCADetectTime | 11.2.7.2, 11.6.5 |

The value of pBackoffSlot is 8 s.