IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Ranging, RF Signature and Adaptability |  |
| Date Submitted | 29 June 2004 |  |
| Source | [Rick Roberts] [Harris Corporation] [MS-9842, P.O. Box 37 Melbourne, Fl. 32902] | Voice: [ 321-729-3018 ] Fax: [ ] E-mail: [ rrober14@harris.com ] |
| Re: |  |  |
| Abstract | This contribution discusses several issues associated with the TG4a effort. These are ranging, reduced RF spectrum signature and spectral adaptability. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

A Linear Ranging Deployment Scenario

The Electronic Fence using MESH networking

- In linear topologies we can do relative positioning based upon the geometry of the network with respect to the control node.

- Multi-dimensional localization, such as in a public safety situation, can take multiple readings from various angles and are considered a subset of the linear problem.

Ranging Cuts Across Multiple Protocol Layers

MAC

PHY

HOST

Range Processing

Arrival Epoch

PHY: Generates Time of Arrival Epoch (accurate < 1nS)

MAC: Generates Ranging Commands & Packet Exchange

HOST: Executes Ranging Mathematical Algorithms

- The actual ranging algorithms are considered beyond the scope of the standard

- The standard should address MMLE, PMLE and MAC command packets to initiate and support the ranging function

- The actual ranging algorithms will be ran at a higher layer … probably at the application layer

- Harris suggests support for double packet exchange, differential time of arrival algorithms which are the most flexible and do not require synchronization between the nodes

  - Let Thold represent the time duration the cooperating device holds the ranging token before sending it back

  - Let Tair represent the time of flight (propagation time)

  - 1st packet time of flight … T1=Tair + Thold + Tair

  - 2nd packet time of flight … T2=Tair + 2*Thold + Tair

  - Time of flight … Tair=(2*T1-T2)/2

The key to Time of Arrival ranging is accurate measurement of the cross-correlation peak!

Cross-Correlation Product

Local Copy

Incoming Waveform

Time of Arrival

The most accurate time of arrival estimate can only be done at the PHY level by observing the location of the cross-correlation peak.

- The arrival time is reported to higher layers via the PLME

- The actual interpretation of the arrival time is PHY implementation dependent

- The PHY time of arrival information could be:

  - Relative pulse delay in PPM (pulse position modulation)

  - Oscillator Phase in BPSK DS-SS

  - Correlation Phase in impulse radio

  - etc. … implementation dependent, stored in PLME

The Key to Time of Arrival Ranging Resolution is Adequate RF Bandwidth

- For example, 1 meter resolution requires 125 MHz of bandwidth

- 500 MHz of bandwidth offers approximately 25 cm of resolution in free space, AWGN channel

- Multipath does impact absolute accuracy!

Multipath Induced Uncertainty

Cross-correlation product with multipath

time

- Real multipath introduces “first time of arrival” uncertainty … which of the above peaks represents the direct path and which ones are just noise?

- As the distance gets longer, the multipath can get more severe, and the first time of arrival uncertainty becomes greater … result is less accuracy at longer ranges

- Uncertainty is mitigated by iteratively reducing the range, which can reduce the multipath, hence improving the ranging accuracy

  - Implication is a “walk-in” algorithm based upon a certain level of accuracy at long range and improved accuracy as the operator approaches the unit under test

Reduced RF Signature for Security Applications

The Electronic Fence using MESH networking

FFD Controller

Bad guy smashes middle sensor breaking MESH network and opening a gap in the electronic fence.

- Want to reduce the RF signature (i.e. detectability) of the sensors once they form a net to avoid detection

- Can do this by having all the sensors change some physical aspect of the waveform in a coordinated manner … an example is periodically changing the spreading sequence.

- The FFD (net controller) periodically beacons with a 802.15.4a standard prescribed waveform to invite other devices to the join the network (based upon passing authentication) … detection of the controller is not an issue since it is physically secure

- Also need a waveform that can not be easily detected by simple nonlinear processing … example of a waveform that is not easily detected via a non-linearity is time hopping … an example of an easily detected waveform, via a non-linearity, is BPSK.

BPSK Spectrum

Squared BPSK Spectrum

Spectral Adaptation

- A Cognitive Radio understands the RF environment and knows what to do to avoid ingress or egress interference … the cognitive function is at layer above/outside the MAC (hence outside the scope of the standard)

- A software defined radio is a radio under cognitive software control

- An adaptable PHY is needed to support this concept and is controlled via the PHY management layer

- Typical PHY adaptable parameters are …

  - Operating Frequency

  - TX Power

  - Bits/Hz

  - Spectral Nulls and Notches