IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | <title> |  |
| Date Submitted | [19 January, 2005] |  |
| Source | [Tadahiko Maeda] [Oki Electric Industry Co., Ltd.] [3-4 Hikari-no-oka, Yokosuka, Kanagawa 239-0847, Japan] | Voice:[+81-46-847-5149 ] E-mail:[maeda097@oki.com] |
| Re: | [] |  |
| Abstract | [Draft outline of SG3c System Requirements to be developed] |  |
| Purpose | [This working document has been prepared to be a starting document of SG3c System Requirements] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

Introduction	3

1.0 	Physical Layer Technical Characteristics	3

2.0 	Bit Rate and Range	3

3.0 	Channelization	5

4.0 	Coexistence and Interference Resistance	5

5.0 	Channel Models	6

6.0 	Power Consumption	6

7.0 	Form Factor	6

8.0 	Complexity	7

9.0 Supplements to Standard 802.15.3-2003 functionality	7

10.0 Regulatory	7

Annex A (informative) Applications Tables	7

Video distribution in home environment	7

Ad-hoc information distribution with Point-Point network extension	8

Data Distribution at Apartment	8

Data Distribution in Stadium	9

High Data Rate Distribution to Train	9

# Introduction

This document and the Selection Criteria yy/nnn provide the technical content for the project to develop an alternative physical layer (alternative PHY). This alternative PHY shall be a supplement to the Standard IEEE 802.15.3-2003.

This document serves two purposes. It is first a summary of the three application presentations from the Study Group 3c Call for Applications (CFA) 04/204. Second, it defines the fundamental requirements implied by applications but not necessarily stated explicitly. The selection criteria document, yy/nnn will guide proposers on how to respond to a Call for Proposals. It also provides amplification on key areas not addressed in this document 04/349. The study group may update this document as discoveries are made while working on the yy/nnn Selection Criteria document.

# 1.0 	Physical Layer Technical Characteristics

Anticipated high-level characteristics of the alternative physical layer are: higher bit rates, higher range (for Point to Point); low power consumption; improved coexistence, and higher density piconets. Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 2.0 	Bit Rate and Range

A bit rate of at least 622 Mbps at 3 m is required at the PHY-SAP. The packet error rate used for this requirement is 8 % for 1024 octet frame body. The transmit EIRP is fixed by regulatory emission limits. An additional higher bit rate at least 1 Gbps at more than 20 m for Point to Point is required. Another higher bit rate of at least 2 Gbps at more than 10 m for Point to Point is required.

Figure 1 shows a reference partitioning.

Figure 1 – Reference partitioning

The uncorrected (i.e., without re-transmissions) error rate as defined in clause 2 is 8% packet error rate. The proposer is encouraged to show signal processing techniques that will yield an ‘equivalent’ bit error rate of < 10-n　(TBD) at the PHY SAP. Data should be provided that shows the added complexity, power consumption, and latency due to this signal processing. In addition, estimates should be given to indicate the PHY overhead due to preambles and PHY headers.

| Requirement | Source |
| --- | --- |
| Non-delay (real-time) transmission at 0.4 Gbps (Application data rate) | 04/348r0 |
| Very-little-delay transmission at 1.5 Gbps (Application data rate) | 04/348r0 |

| Requirement | Source |
| --- | --- |
| 1.5 Gbps at 10 m distance (for point to point, application data rate) | 04/348r0 |
| Point to Multipoint | 04/348r0 |
| > 1 Gbps at > 20 m distance for mFDEV-mFDEV link mFDEV: millimeter wave Forwarding Device | 04/352r0 |
| > 622 Mbps at 3 m distance for mFDEV-mMDEV link mMDEV: millimeter wave Mobile Device | 04/352r0 |

# 3.0 	Channelization

The bit rates and ranges specified in section 2.0 shall be achievable on each piconet for operation with at least 3 other piconets operating in close proximity even with some degree of performance degradation.

Combination of analog/digital broadcasting channels and digital communication channels is required.

| Requirement | Source |
| --- | --- |
| 4 non-overlap channels | (04/388r0) |
| Combination of analog/digital broadcasting channels and digital communications channels | 04/352r0 |

# 4.0 	Coexistence and Interference Resistance

The PHY shall be able to coexist with other wireless devices that may be in close proximity to IEEE 802.15.3 devices. This includes interference from 802.15.3c devices to other wireless systems as well as interference from other wireless systems or radiators to 802.15.3c devices.

Coexistence and interference resistance shall, at a minimum, be considered for the following wireless devices: Bluetooth™ (802.15.1), IEEE 802.11, IEEE 802.15 (3&4), IEEE 802.16, ISM cordless phones, and mmWave collision avoidance radars. Interference resistance from radiators, like microwave ovens, shall also be considered.

| Requirement | Source |
| --- | --- |
| Coexistence with Bluetooth, IEEE 802.11, IEEE 802.15.3, IEEE 802.15.4, ISM cordless phones |  |
| Coexistence with IEEE 802.16 |  |
| Coexistence with mmWave collision avoidance radars |  |
| Interference resistance from microwave ovens |  |

# 5.0 	Channel Models

Performance in a multipath environment is a critical feature for the SG3c alternative PHY. Document (yy/nnn) provides a reference alternative PHY channel model. The intention is to use existing narrow band channel models (such as the 802.11 model) for non-UWB alt-PHY proposals. The individual proposers should make use of the SG3c models as they become available as a basis for evaluating the performance of their proposals. Individuals submitting proposals that are not addressed by the RF channels selected by the Study Group are encouraged to provide specific details of the channel models used with references to literature, use in other standards, and/or research.

| Requirement | Source |
| --- | --- |
| [Specification: Channel Model] | yy/nnnrn |

# 6.0 	Power Consumption

The alternative PHY should consume less than nn mW for nn Mbps and less than nn mW for nn Mbps in either the transmit state or the receive state. A power save state is also required.

The proposer should indicate the possible power management actions that can be initiated by Standard 802.15.3-2003 "PHY-PWRMGT.request". Furthermore, the proposers should provide text including a table that indicates the number of power save levels offered and how much power each level saves.

Proposers providing PHY solutions should disclose PHY power budgets. Similarly, proposers providing MAC enhancements should disclose MAC power budgets.

| Requirement | Source |
| --- | --- |
| Less than 1 W for mobile DEV | 04/352r0 |
| [Specification: Power consumption] | yy/nnnrn |

# 7.0 	Form Factor

The PHY components will be capable of fitting into a form factor consistent with a camera, PDA, NIC cards, PCMCIA card or other small form factors of hand-held devices and storage mediums. The antenna size and form factor will not significantly increase the original device size.

| Requirement | Source |
| --- | --- |
| Consistent with a note-PC, PDA, etc. | 04/352r0 |
| [Specification: Form Factor] | yy/nnnrn |

# 8.0 	Complexity

Complexity will be minimal to enable mass commercial adoption of a variety of cost sensitive products.

| Requirement | Source |
| --- | --- |
| [Specification: Complexity] | yy/nnnrn |

# 9.0 Supplements to Standard 802.15.3-2003 functionality

It is envisioned that the alternative PHY project will allow supplements to Standard 802.15.3-2003. In addition to the PHY itself, the system may require MAC modifications necessary to support the mmWave PHY.

It is required that the alternative PHY operate with the 802.15.3 MAC without fundamental change in operation. It is also required that any enhancements or modifications are enabled by reference to alternative PHY specific PIB parameters.

# 10.0 Regulatory

The alternative PHY standard will comply with necessary geopolitical or regional regulations.

# Annex A (informative) Applications Tables

The following applications tables are informative and sourced from: 04/348r0, 04/352r0, and 04/653r1.

## Video distribution in home environment

This application involves a video server distributing video contents and client devices receiving the contents in a same room at home. Video contents in this category include real-time compressed TV broadcast and uncompressed TV transmitting which not necessarily has real-time characteristics. It is assumed that a typical room size where users enjoy a personal/family video life at home is up to 10 m in diagonal.

Relay of digital broadcasts in real-time requires 0.3 Gbps of data rate for Broadcast Satellite (BS) broadcasting, 0.4 Gbps for Communication Satellite (CS) broadcasting. Uncompressed video data from one channel of broadcast, stored contents in the server, or received contents via the server requires 1.5 Gbps in HD-SDI (High Definition Serial Digital Interface) format.

All these group of contents may not be transmitted simultaneously.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| BS broadcast | Digital Broadcast | - | 0.3 Gbps |
| CS broadcast | Digital Broadcast | - | 0.4 Gbps |
| Data | HD-DSI | - | 1.5 Gbps |
| Total |  | 1.5 Gbps | 2.2 Gbps |

## Ad-hoc information distribution with Point-Point network extension

This application involves Point-Point ad-hoc backbone link devices and Point-Multipoint Hot spot access link devices to construct temporal networks such as in exhibition site.

Communications may be for advertisement information distributions and contents downloading services.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Backbone | Not specified | 1 Gbps | - |
| Hot spot | Not specified | 622 Mbps | - |
| Total |  | 1 Gbps | - |

## Data Distribution at Apartment

This application involves a central transceiver on an apartment, which connected with outer networks and/or broadcast antenna, optional repeaters, which overcome the LoS problem, transceivers on apartment rooms, which serve direct links to CEs in apartment rooms, and CEs, which are used for TV watching and/or the Internet link.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Telestrial/BS/CS broadcast | Analogue Broadcast | - | (20 Mbps/ch) |
| Telestrial/BS/CS broadcast | Digital Broadcast | - | (30 MHz/ch x 100 Channels) |
| Internet link |  | - | (100 Mbps/User) |
| Total |  | - |  |

## Data Distribution in Stadium

This application involves HD cameras, access point terminals equipped in a stadium, and mobile terminals of the audience there to watch real-time scene.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Stadium scene | HD-SDI | - | 1.5 Gbps |
| Total |  | - | 1.5 Gbps |

## High Data Rate Distribution to Train

This application involves a transceiver on a station and a transceiver on a train to download huge data in very limited period.

| Application Ingredients | Data Format | Min Data Rate | Max Data Rate |
| --- | --- | --- | --- |
| Download data | Not specified | - | 0.27 Gbps |
| Total |  | - | 0.27 Gbps |