IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | <title> |  |
| Date Submitted | [25 January, 2005] |  |
| Source | [Tadahiko Maeda] [Oki Electric Industry Co., Ltd.] [3-4 Hikari-no-oka, Yokosuka, Kanagawa 239-0847, Japan] | Voice:[+81-46-847-5149 ] E-mail:[maeda097@oki.com] |
| Re: | [] |  |
| Abstract | [Draft outline of SG3c System Requirements to be developed] |  |
| Purpose | [This working document has been prepared to be a starting document of SG3c System Requirements] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 Introduction	3

2.0 	Physical Layer Technical Characteristics	3

3.0 	Bit Rate and Range	3

4.0 	Channelization	5

5.0 	Coexistence and Interference Resistance	5

6.0 	Channel Models	6

7.0 	Power Consumption	6

8.0 	Form Factor	6

9.0 	Complexity	7

10.0 Supplements to Standard 802.15.3-2003 functionality	7

11.0 Regulatory	7

Annex A (informative) Profile of Applications	7

# 1.0 Introduction

This document and the Selection Criteria yy/nnn provide the technical content for the project to develop an alternative physical layer (alternative PHY). This alternative PHY shall be a supplement to the Standard IEEE 802.15.3-2003.

This document serves two purposes. It is first a summary of the application presentations from the Study Group 3c Call for Applications (CFA) 04/204. Second, it defines the fundamental requirements implied by applications but not necessarily stated explicitly. The selection criteria document, yy/nnn will guide proposers on how to respond to a Call for Proposals. It also provides amplification on key areas not addressed in this document 04/349. The group may update this document as discoveries are made while working on the yy/nnn Selection Criteria document.

# 2.0 	Physical Layer Technical Characteristics

Anticipated high-level characteristics of the alternative physical layer are:

- Higher bit rates

- Longer range

- Lower power consumption

- Improved coexistence

- Higher density piconets

Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 3.0 	Bit Rate and Range

A bit rate of at least 622 Mbps at 3 meters is required at the PHY-SAP (Service Access Point). The packet error rate used for this requirement is 8 % for 1024 octet frame body. The transmit EIRP (Effective Isotropic Radiated Power) is fixed by regulatory emission limits. An additional higher bit rate at least 1 Gbps at more than 20 m for Point to Point operation is required. Another higher bit rate of at least 2 Gbps at more than 10 m for Point to Point operation is required.

Figure 1 shows the reference partitioning.

Figure 1 – Reference partitioning

The error rate criterion is equivalent to that specified in 11.6.1 of the Standard IEEE 802.15.3-2003. The proposer is encouraged to show signal processing techniques that will yield an ‘equivalent’ bit error rate of < 10-n　(TBD) at the PHY SAP. Data should be provided that shows the added complexity, power consumption, and latency due to this signal processing. In addition, estimates should be given to indicate the PHY overhead due to preambles and PHY headers.

| Requirement | Source |
| --- | --- |
| Non-delay (real-time) transmission at 0.4 Gbps (Application data rate) | 04/348r0 |
| Very-little-delay transmission at 1.5 Gbps (Application data rate) | 04/348r0 |

| Requirement | Source |
| --- | --- |
| 1.5 Gbps at 10 m distance (for point to point, application data rate) | 04/348r0 |
| Point to Multipoint | 04/348r0 |
| > 1 Gbps at > 20 m distance for mFDEV-mFDEV link mFDEV: millimeter wave Forwarding Device | 04/352r0 |
| > 622 Mbps at 3 m distance for mFDEV-mMDEV link mMDEV: millimeter wave Mobile Device | 04/352r0 |

# 4.0 	Channelization

The bit rates and ranges specified in section 3.0 shall be achievable on each piconet for operation with at least 3 other piconets operating in close proximity even with some degree of performance degradation.

Combination of analog/digital broadcasting channels and digital communication channels should be supported.

| Requirement | Source |
| --- | --- |
| 4 non-overlap channels | (04/388r0) |
| Combination of analog/digital broadcasting channels and digital communications channels | 04/352r0 |

# 5.0 	Coexistence and Interference Resistance

The PHY shall be able to coexist with other wireless devices that may be in close proximity to IEEE 802.15.3 devices. This includes interference from 802.15.3c devices to other wireless systems as well as interference from other wireless systems or radiators to 802.15.3c devices.

Coexistence and interference resistance shall, at a minimum, be considered for the following wireless devices: 802.15.1 (Bluetooth™), IEEE 802.11, IEEE 802.15 (3&4), IEEE 802.16, ISM cordless phones, and mmWave collision avoidance radars. Interference resistance from radiators, like microwave ovens, shall also be considered.

| Requirement | Source |
| --- | --- |
| Coexistence with 802.15.1, IEEE 802.11, IEEE 802.15.3, IEEE 802.15.4, ISM cordless phones |  |
| Coexistence with IEEE 802.16 |  |
| Coexistence with mmWave collision avoidance radars |  |
| Interference resistance from microwave ovens |  |

# 6.0 	Channel Models

Performance in a multipath environment is a critical feature for the SG3c alternative PHY. Document (yy/nnn) provides a reference alternative PHY channel model. The individual proposers should make use of the SG3c models as they become available as a basis for evaluating the performance of their proposals. Individuals submitting proposals that are not addressed by the RF channels selected by the group are encouraged to provide specific details of the channel models used with references to literature, use in other standards, and/or research.

| Requirement | Source |
| --- | --- |
| [Specification: Channel Model] | yy/nnnrn |

# 7.0 	Power Consumption

The proposer shall estimate DC power consumption in both the transmit state and the receive state.

The proposer should indicate the possible power management actions that can be initiated by Standard 802.15.3-2003 "PHY-PWRMGT.request". Furthermore, the proposers should provide text including a table that indicates the number of power save levels offered and how much power each level saves. A power save state is also required.

| Requirement | Source |
| --- | --- |
| Less than 1 W for mobile DEV | 04/352r0 |
| [Specification: Power consumption] | yy/nnnrn |

# 8.0 	Form Factor

The PHY components should be capable of fitting into a form factor consistent with a camera, PDA, NIC cards, PCMCIA card or other small form factors of hand-held devices and storage mediums. A configuration shall be proposed for which the antenna size and form factor will not significantly increase the original device size.

| Requirement | Source |
| --- | --- |
| Consistent with a note-PC, PDA, etc. | 04/352r0 |
| [Specification: Form Factor] | yy/nnnrn |

# 9.0 	Complexity

Complexity will be minimal to enable mass commercial adoption of a variety of cost sensitive products.

| Requirement | Source |
| --- | --- |
| [Specification: Complexity] | yy/nnnrn |

# 10.0 Supplements to Standard 802.15.3-2003 functionality

It is envisioned that the alternative PHY project will allow supplements to Standard 802.15.3-2003. In addition to the PHY itself, the system may include MAC modifications necessary to support the mmWave PHY.

It is required that the alternative PHY should operate with the 802.15.3 MAC without fundamental change in operation. It is also required that any enhancements or modifications are enabled by reference to alternative PHY specific PIB parameters.

# 11.0 Regulatory

The alternative PHY standard should comply with necessary geopolitical or regional regulations.

# Annex A (informative) Profile of Applications

Table below summarizes applications submitted in response to the CFA. The applications have been arranged in the numeric order of the document number (last column).

| # | Description Of Applications | Outdoor | Indoor | CFA (IEEE) Document Number |
| --- | --- | --- | --- | --- |
| 1 | Vertically connected wireless link | - LOS - Data Rate: ? - Range: ≤ 33 m | - | 6, 92 |
| 2 | Gigabit Ethernet link, wireless IEEE1394 applications | - | - LOS - Data Rate: ≤1 Gbps duplex - Range: ≤ 17 m | 19 |
| 3 | Vertical wireless link | - LOS - Data Rate: ? - Range: ≤ 30 m | - | 92 |
| 4 | Adhoc information distribution system | - | - LOS - Data Rate: ? - Range: ? | 97 |
| 5 | Multimedia, information distribution system | - | - LOS - Data Rate: ≥ 1 Gbps - Range: ≤ 10 m | 98 |
| 6 | - Outdoor: Fixed wireless access, distribution in stadiums, inter-vehicle communication, etc. - Indoor: Connecting multimedia devices (wireless home link), ad-hoc meeting, heavy content download, distribution system | - LOS - P2P, P2MP - Data Rate: 156 Mbps to 1.5 Gbps - Range: 400 m to 1 Km | - LOS - Data Rate: 100 Mbps to 1.6 Gbps - Range: ~ 10 m | 118 |
| 7 | Small office/meeting scenario, general office applications | - | - NLOS - OFDM - Data Rate: ≤ 200 Mbps - Range: 2 to 4 m | 141 |
| 8 | Distribution links in apartments, stadium, etc. | - LOS - P2P - Data Rate: ? - Range: ≤ 220 m | - | 153 |
| 9 | Ad hoc network | - | - LOS - Data Rate: 622 Mbps - Range: ? | 155 |
| 10 | Wireless IEEE1394 applications | - | - LOS - Data Rate: 100, 200, 400, 800, 1600, 3200 Mbps - Range: 12 m LOS | 351 |
| 11 | - Outdoor: Distribution links in apartments, stadium, etc. - Indoor: Ad-hoc network | - LOS - P2P and P2MP - Data Rate: ? - Range: ≤ 220 m | - LOS - Data Rate: ≥ 1 Gbps and ≥ 622 Mbps - Range: ≥ 20 m and ≥ 3 m | 352 |
| 12 | Wireless home video server connected to HDTV, PC and other video devices | - | - LOS - Data rate: 300 Mbps, 400 Mbps and 1.5 Gbps uncompressed HDTV data - Range: ≤ 10 m | 348 |
| 13 | PowerPoint and such applications | - | - LOS and NLOS - Data Rate: ? - Range: ≤ 3 m - Space diversity | 514 |
| 14 | Two way vertical wireless link | - LOS - P2P - Data Rate: ? - Range: ? | - | 649 |
| 15 | Wireless Gigabit Ethernet applications | - | - LOS - Data Rate:1.25 Gbps duplex - Range: ~ 6 m | 653 |
| 16 | - Replacement for 1394 FireWire - Replacement for USB - Military – future combat systems, secure communication | - | - LOS and NLOS (people) - 100 to 500 Mbps link, 1 Gbps in 2007 - Short range | 665 |