IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Consolidation of Explanations for No Vote on Confirmation |  |
| Date Submitted | 14 July 2004 |  |
| Source | [Gerald W. Wineinger] [TI] [Dallas, Texas] | Voice: [214 480 1013] Fax: [214 480 6662] E-mail: [gww@ieee.org] |
| Re: | 802.15.3a Task Group No Vote Explanations |  |
| Abstract | No Vote Explanations for Confirmation Task Group 3a in Orlando |  |
| Purpose | No Vote Explanations for Confirmation Task Group 3a in Orlando |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

CONTENTS

Wednesday, 14 July 2004

No Vote Explanation in Alphabetical Order

WEDNESDAY, 14 JULY 2004

The following are NO-vote explanations given:

Roberto Aiello:

I will consider changing my no vote to a yes vote if the following criteria are met:

Worldwide regulatory compliance

It is unclear when and how the proposal can be adopted worldwide. Recent regulatory conferences such as ITU indicate that there is strong interest in adaptive transmitters that can dynamically notch selected frequency to increase coexistence. It is not clear how the proposal can accomplish it and what power consumption and complexity consequences it will have, while it is natural for the MB-OFDM proposal.

US regulatory compliance

It hasn’t been shown how the proposal is compliant with current FCC regulations. Does Freescale have UWB chips that are FCC certified and fabricated according to the current proposal?

Time to Market

It is not clear what the time to market for this proposal is. Freescale has stated in July 2003 “production ICs here today””, then the proposal has changed and it is not clear what the time to market is now.  I would also like to know how the proposers plan to work with interoperability issues, given that the WiMedia and WUSB forum has adopted the MB-OFDM proposal.

Common signaling mode

The common signaling mode is in violation of the PAR. I would like the authors to explain how this is consistent with the PAR.

Intellectual property and RAND

I am concerned that 15 companies have been identified as having potentially IP in the DS-UWB proposal. I would like the authors to explain how they plan to manage this issue.

Performance

The proposers should clarify how their performance meets the downselection criteria without using best case scenarios. For example, what performance (range, bit rate and SOP) can the proposal achieve with 51mW power consumption?

Performance comparison

Clarify the statement made by the authors in document 04-0343r0 that says “Intel/Philps … have already admitted [they picked an inferior scheme] when they abandoned their original Multiband proposal in favour (sic) of TI’s MB-OFDM proposal”.

Naiel Askar:

My main issues with the merger 2 proposal are mentioned below. If these issues are resolved satisfactorily, I will change my vote

- Proof of performance: Merger 2 claims  excellent results for range in multipath with very little complexity. Matt Welborn mentioned in his presentation that existing chips were based on the same solution as the proposal. To give credibility to these numbers,  I would like to know what range  is achieved in the existing solution in NLOS conditions and at what complexity, and how they can be extrapolated to justify the claimed performance in the proposal .

- Power consumption: no power consumption numbers were given. Need to have  the power numbers of existing solution  be given and extrapolated for the solution where multipath range performance is claimed

- Spectrum adaptation: merger 2 proposal has no simple means to adapt its spectrum to worldwide differing regulatory conditions

- Narrow band interference:No results were given based on the selection criteria document.  Performance in the presence of narrow band interference will suffer without adaptive notch filters which are not easy to implement

- CCA performance: Current CCA mechanism only works in CM1 with ranges only up to 4 meters. Need to provide better solution

- FCC certification: Since the solution is already available in chips, I would like to know if it has been certified by the FCC and if not why , since this may seriously impact allowed transmit levels

Mikio Aoki:

There should be multiple silicon vendors to enable a competitive market for me to change my no vote to a yes.

Anuj Batra:

Here are my NO comments:

- Spectral shaping – there is no easy way to shape the spectrum in order to avoid interference, without adding expensive external adaptive notch filters and/or high power consuming digital filters and very high-speed DAC (ex. SSA).

- There are strong IP uncertainties concerning Merged Proposal #2.

- Digital gate count is not accurate and needs to be updated to more accurately reflect the complexity required to implement all data rates and must account for a chip-level equalizer.

- No information has been provided about the power consumption for the proposal.

- Authors of Merged #2 showed heuristic arguments for performance in the presence of narrowband interferers. The selection criteria ask for simulation results. Authors need to do simulations.

- Acquisition curves at Eb/No = 4 dB shows that approx. 15% of the packets are missed even when the false alarm probability is set a value higher than 1%. This is a serious deficiency in the system and will have an impact at lower rates.

- Current evidence shows that DFE works well at very high SNRs (9.6 and 12.6 dB). Must show evidence that DFE will not suffer from error propagation at realistic operating points such at ~1 dB.

- Current CCA mechanism only works in CM1 with ranges only up to 4 meters. This is insufficient.

- CCA mechanism must be defined for lowest data rate that is mandatory.

- Merged proposal #2 may have problems achieving regulatory approval in Europe, because of lack of flexibility in spectrum shaping.

- SOP numbers are inconsistent with any of the sanity checks. Please revise numbers via full level simulation. Do not assume that the interference is white Gaussian noise and shift curves. To verify numbers are based on simulations, please provide each curve used in the averaging process.

- There has been no evidence shown that Merged Proposal #2 can lead to a CMOS implementation and still meet the range, complexity, and powers numbers.

- The simulation results provided make a number of unrealistic assumptions on the system impairments, especially on receiver synchronization (assumes perfect synchronization, with no jitter). Please provide simulation results by including all system impairments.

If these comments are addressed in sufficient detail, I may change my NO vote.

Chuck Brabenac:

I might consider changing my no vote to a yes vote if the following issues with the Merge #2 proposal are resolved:

- During the September 2003 meeting, I contributed to a section of a presentation (ref 03-343-01) that compared CCA capabilities between the two remaining proposals.  The conclusion of that presentation was that the CCA mechanism accompanying the DS-UWB proposal works well only under ideal conditions, suffering in the presence of multipath and local oscillator mismatch.  Unlike the MB-OFDM proposal, my team could not see any practical way to detect conditions where it may not work correctly, such that fallback to using only preamble based or virtual carrier sense techniques is possible.  Based on current information, I believe the merge #2 proposal’s CCA mechanism to be unreliable.  Although a foil was presented by the Merge #2 team in March 2004 (re 04-099-02) making a claim that “techniques are available to enhance the effectiveness of this approach”, no substance (further details, simulation results, etc.) have been offered to back up that claim, and I think that work needs to be done to determine whether or not there is a viable CCA approach.

- I am uncomfortable with the lack of spectral agility offered by the Merge #2 proposal, and would like to see that improved beyond just “high band” and “low band”.  I think it should be able to adapt with finer granularity to conditions where other narrow band users of the same spectrum as UWB are nearby.

Vern Brethour:

I have several concerns about the DS-UWB proposal which motivated my no vote during the comfirmation.

DS-UWB lacks the spectral flex ability to adapt to the world wide regulatory environment.

The DS-UWB proposal does not include adequate CCA algorithms.

The DS-UWB proposal is relying heavily on an equaliser which is aledged to be easy but I do not understand how to achieve the reported levels of performance.

Pat Carson:

I would consider changing my "NO" vote for merged proposal #2 to a "YES" vote if: 

1)  There is a better explanation of the time-to-market considerations. It is unclear as to when silicon compliant to merged proposal #2 would be available and from how many different suppliers. 

2)  An explanation of how the authors of merged proposal #2 plan to address the WW regulatory issue. It is evident that the ITU will require some sort of spectrum shaping before UWB radios are allowed in Europe.

Jonathon Cheah:

I shall change my vote to YES if

- The vulnerability of DS-UWB to strong in-band interferer is addressed satisfactorily.

- The Silicon implementation feasibility is substantiated satisfactorily. ----The Silicon implementation size/power/complexity claims of the ADC/DAC/FEC/Rake/equalizer at high speed do not agree with general knowledge. The past Extreme Spectrum chipset example used as existence proofs, are not pertinent to the current DS-UWB proposal on the table.

- The rationale behind the inclusion of CSM in this proposal to appease a non-existing standard is articulated satisfactorily. ---- CSM increases the complexity in PHY and MAC. No supporting feasibility of its implementation. How it impacts DS-UWB /MBOFDM basic performance. No analyses provided.

Joe Decuir:

The summary of my NO vote is that:

1) a 7 GHz bandwidth _needs_ a 'cognitive radio'.

- All those vulnerable systems to co-exist with.

- All those other radios to co-exist with (from IEEE and others).

OFDM does this easily, it is possible but much tougher with DS.

2) the DS benefit of very high data rates at short range is not compelling

enough reason to drop the MB advantage in adaptability:

2.1) At the very short ranges where these high data rates are offered, existing

wired solutions are competitive: fast, cheaper, lower power.

2.2) Modern PC and PDA hosts can't use that data bandwidth yet.

Cognitive Radios

Previous works by 802.11 and 802.15 have been relatively narrow band.

Their instantaneous spectrum use is in the order of dozens of MHz.

Even in those chunks of spectrum, co-existence is challenging.

(Wait until (ISO 14443?) high power RFID starts stepping on 2.4GHz bands.)

UWB will occupy chunks of a 7GHz space!!  To me, the need for a well mannered and flexible 'cognitive radio' is compelling.  In my view, the news from the ITU-R supports this.  We have convened an entire 802.19 group to address coexistance with other IEEE 802 designs, etc.

DS spreads out over either 1.75 GHz or 3.5 GHz.  Those are huge bands.

It is technically possible to add the complexity necessary to make DS capable of being a cognitive radio.  However, you would end up with the complexity of OFDM.

In that case why bother?  In contrast, MB starts with smaller chunks, and is naturally easier to adjust the spectrum that it does use.

Bit rate needs

The DS design uses very large spectrum to offer high rates at low complexity.

Because the case for needing those bit rates is weak, the case for DS is not strong enough, in my mind, to justify loosing the advantages of MB.

I can imagine wanting to transfer CD-sized or DVD-sized images _wirelessly_.

Not soon, but someday.  We don't need it today; wires work fine.

At the very short ranges at which DS-UWB offers Gbps rates, wires are competitive, and made of obtainium: IEEE 802.3 GigaBit, IEEE 1394 and USB 2.0.  Wires may also be easier for the end-users to deal with at short ranges, given the connection security issues inherent in all modern wireless communications.

This room is full of notebooks that implement USB 2.0 and IEEE 1394 right now.

I think Gigabit ethernet is held back by performance limits in Windows & MAC OS.

USB now has a video class specification, that seems to be quite useful.

The max data rate is 30% of 480Mb/s, or 144Mb/s.  (30% max usage per device is a USB 2.0 rule, not a technical limitation of the physical layer.)

Meantime, there are host limitations:

Microsoft is redesigning the network stack plumbing to keep up with Gigabit Ethernet.

They call this NDIS 6.0.  It will ship in 'Longhorn' in 2006 at the soonest.

Modern PCs with XP have trouble keeping up without PCI-bus hardware accelleration.

(I am expert on Windows network drivers.  I spent 1992-2000 there.)

MCCI does networking software for PDAs, XP and MAC OS hosts.  We can confirm the performance limits on Windows and MAC OS PCs; PDAs are an order of magnitude slower.

In other words, if we gave the market a 1.3 Gbps radio today, the PC and PDA crowd could not handle it.  USB 2.0 works because there is DMA support to the host controller.

'Wireless USB' would need similar host support.

I do appreciate that the DS designers have made attempts to design a system that supports a dual-headed system, with DS _and_ MB _and_ CSM.  This is a lot of work.

I think this is a good committee-political solution, but not a good market solution.

A dual-PHY radio will be even more complex than the individual radios.

A single-PHY radio will have limited interoperability, and confuse the market the way that the 802.11-1997 dual standard did.

Jason Ellis:

The following items are my major concerns that would need to be addressed and fixed for me to consider changing my no vote to a yes vote, in support of merge proposal #2.

Time to market

Merge proposal #2 claims time to market advantage of silicon that is supposedly available in the market as generation 3; however, merge proposal #2 has recently drastically changed and the silicon does not represent the proposal before us; does not meet performance requirements as specified by the selection criteria, including power consumption. For me to change my no vote to yes would require seeing working silicon that implements the proposal as I fear there is significant time to market concerns for merge proposal #2.

Common signaling mode

The inclusion of a common signaling mode violates the PAR, so this would need to be removed from the proposal for me to want to change my vote to yes.

No ecosystem adoption

There would need to be a certification and compliance program in place to support the PHY standard for me to change my no vote to yes; as there is fear of interoperability if there is no organization. Merge proposal #1 has a compliance and certification program in place, as specified by the WiMedia Alliance.

There would need to be adoption of merge proposal #2 by major industry segments for me to believe that the industry believes the performance capabilities of merge proposal #2. Merge proposal #1 has major industry adoption, including the Wireless USB Promoters Group, and I fear that merge proposal #2 has minimal industry support.

Multiple vendors

There would need to be multiple vendors of silicon to enable a competitive ecosystem for me to change my no vote to a yes.

Insufficient and misrepresented information

The information presented by merge proposal #2 is in a fashion that is confusing as it does not stay true to modes of operation, performance capabilities and complexity/power consumption.

There are insufficient details on the transmitter and receiver architecture, coding schemes, modulation for validation of the claims presented by merge proposal #2.

Lack of regulatory clarification

Merge proposal #2 silicon would need to be certified by regulatory agencies as it isn’t clear if their proposal meets out of band emissions. Further, the lack of spectrum flexibility (band selection) concerns me in the worldwide regulatory environment because of a lack of ability to support radio astronomy bands. There is strong concern that merge proposal #2 can serve as a worldwide radio in its present form.

Scalability

I would need to see simulation results, architectures, and have a comfortable feeling regarding merge proposal #2s capabilities to scale in data rate and range, as described in the 802.15.3a selection criteria.

Multipath

Merge proposal #2 would need to show performance criteria with a direct correlation to transceiver architecture and complexity for me to be comfortable with its ability to operate in more sever channels such as CM2, CM3, and CM4.

Intellectual Propterty

It was brought up during the IEEE 802.15.3a meetings that there are 15 companies that may have intellectual property claims to merge proposal #2, and I would need to feel comfortable with their RAND statements before changing my no vote to a yes, especially as it relates to direct sequence spread spectrum technologies and ultrawideband impulse radios, rake receivers, etc…

Kris Fleming:

Here are the reasons for my no vote

- Regulatory issues -- I am very concerned about the lack of ability to support spectral shaping to conform to the various regulatory bodies.

- CCA – I am concerned about lack of data showing reliable CCA method of long (10m) ranges.  This is critical for supporting various MAC functionality for networking

- I also have concern for power and time to market.  Lack of data for these items as still missing

I will consider changing my no to yes if these issues are addressed.

Jeffrey Forester:

Here are my reasons for voting no:

- I have not seen enough definitive evidence that the DS-UWB approach can be efficiently integrated into a standard digital CMOS process.  I would like to see evidence that all of the RF front end components, for both operating bands, can be efficiently integrated into a digital CMOS process.

- I would like to see how the DS-UWB system would support a low complexity and highly reliable method for avoiding potential interference into WiMax systems operating at 3.5 GHz, indoors, and in close proximity to a UWB device.  In addition, I would like to see evidence how the DS-UWB system would perform in the presence of a strong WiMax signal near-by, using implementations suitable for a digital CMOS process.

- I would like to see how the DS-UWB system would support a low complexity method for avoiding potential interference into Radio Astronomy bands operating around the world, based upon the current ITU recommended limits.

- I would like to see how the DS-UWB system can efficiently meet power spectral density limits well below the FCC mask at frequencies below 3.1 GHz, which are being considered by the ITU.

I may change my vote from a no to a yes if the above concerns are addressed to my satisfaction.

Evan Green:

I believe that the proposed DS system is not capable of meeting the world wide regulatory requirements and thus would not be a good choice for an 802.15 standard. For example protection of radio astronomy and other incumbent users of the radio spectrum.

Jeff Harris:

The main areas of concern for my 'no' vote are:

1.  International Regulatory Compliance:  With international regulatory bodies considering UWB emission masks potentially different from those approved by the FCC, no approaches have been shown of how the DSSS approach can address different emission masks without requiring hardware changes or experiencing degradation in performance.  Please explain the approach proposed.

2.  Range/Multipath Performance:   The proposal #2 team has provided simulation results on the implementation of a sophisticated rake structure to solve early product multipath issues.  The claim is that it can be implemented in a remarkably small silicon area.  If the solution is so straighforward, why hasn't it been demonstrated in hardware?

3.  FCC Certification:  Given that DSSS UWB was introduced as a product to the market two years ago, have DSSS products been successfully certified by the FCC and if not, why?

Robert Huang:

No vote reasons

1. The DS-SS system proposed does not meet the easy frequency adaptability (adaptable emissions mask) that will be necessary for a world wide deployment of a single implementation.

2. Draft European spectral mask has a sloping out-of-band emissions mask. It has not been shown that that this mask can be met at reasonable cost.  While some may view that meeting a draft European emissions mask is not necessary at this time. My view is that 802.15.3a is to be world wide standard and will require world wide acceptance. Thus the market place success of the 802.15.3a standard will depend on world wide acceptance. Thus, I assert that easy flexibility in meeting the draft European mask is very important for any candidate technology.

3. More fundamentally, the implementation of the proposed DS-SS system has a level of complexity and a level of performance that places it out of the reach of a major full capability cost-down market segment. Primary in achieving this cost effectiveness is implementation in real CMOS, not theoretical calculations.

4. I believe that a big application area for UWB will be in personal and portable devices. I have not been convinced that the DS-SS proposal will provide good performance in the face of changing propagation conditions, potential due to device movement or movement of people and objection in the propagation path. I have not seen an explanation and a simulation of how the equalizer will quickly adapt.

5. Another critical aspect of the PHY will be robustness. I do not yet have the confidence that the proposed system will deliver the performance necessary to achieve market place acceptance in consumer applications.  I do not question the performance numbers but rather feel that the performance in difference scenarios was not portrayed in a consistent manner.

When these items are addressed and if I find no other compelling reasons for voting no, I will consider changing my vote from no to yes.

Jeyhan Karaoguz:

I like to see the following issues addressed and resolved before I can change my '"no" vote:

(1) DS-UWB requires a much wider RF-bandwidth making the CMOS implementation more costly and difficult

(2) Dealing with narrowband interference is more difficult compared to alternative approaches

(3) In a multipath environment, it is much easier and practical to combine equalization and coding with alternative approaches

Joy Kelly:

I voted no for several reasons.  The most important reasons are 1) lack of flexibility for global regulatory compliance,  2) lack of sufficient high fidelity simulations of equalizer performance, and 3) unclear correlation between various options for improvement and estimates of power consumption/complexity.

I respectflully submit these to Merged Proposal #2 members.

Mike Kelly:

The following items are my major concerns that would need to be addressed and fixed for me to consider changing my no vote to a yes vote, in support of merge proposal #2.

I would vote YES if... The DS-UWB packet error rate was limited by bit errors rather than packet synchronization losses.

I would vote YES if... the DS-UWB scheme had a less complex method for their digital rake receiver, so that a complex analog rake would not be needed.

Also, it would be helpful to see responses to the following queries posed to the MBOA in the past that are just as applicable to the DS-UWB PHY:

- Demonstration of digital/RF CMOS in generally available FABs with sufficient performance to implement 15.3a radios yielding at 6 sigma levels.  Specifically - 130 nM and 90 nM RF & digital CMOS.

- Demonstration of a working prototype that implements effective protection for specific licensed services or frequency ranges of concern (e.g., Japanese Radio Telescopes) and reserved bands without degrading information throughput to a level less than 95% of the expected maximum for the selected operating mode.

- Demonstration of co-location capability with portable electronic devices such as cell phones, portable MP3 players, etc.

- Proven levels of radiated and conducted emissions not only per the FCC rules, but sufficiently low to permit co-integration of the resulting devices in units mentioned above.

- Complete disclosure of interference testing results, including the simulation and Testing of DS-UWB prototypes and/or chips/chipsets for in- and out- of band effects on co-located cellular telephone systems such as GSM, CDMA, and WCDMA.

- The full disclosure of implementation details on DS-UWB prototypes fabricated according to the current DS-UWB proposal, and COMPLETE FCC testing results, along with a matrix of these devices operating in the presence of a cross section of in-band victim receivers.

- Proof that other DS-UWB based systems exist that operate in similar environments, i.e., uncoordinated overlapping signaling that allows multi-user operation.

- Substantiated proof that the analog RF sections are less complex than those seen in 802.11 IC's.

Rioji Kido:

I have voted No and support MB-OFDM by the following reason.

Because I think on a DS-UWB method, spectrum flexibility is less than on a MB-OFDM method, and it seems to be difficult for adapting for several regulatories.

Yongsuk Kim:

I will consider changing my no vote to a yes vote if merge proposal #2 can be addressed and fixed on my major concerns below listed.

1. Lack of simulation results which related on all items of selection criterias for all channel models(CM1,CM2,CM3,CM4).

2. Unclear explanation on implementation complexity,power consumption and die size.

3. Unclear on IPR policy for the #2 proposal especially on DS-CDMA and Impulse radio tech. which used in the proposal.

I don't know which company has the essential IPR on DS-CDMA and  Imulse radio tech ,

and what  the company's policy on the IPR is.

DoHoon Kwon:

The following is my "no-voter response".

1. I am not convinced that the performance simulation results are sufficient to prove its

high-speed connectivity in real-world situations, considering that your proposal underwent

a major change quite recently last March.

2. I understand the effect of antennas can be significant for UWB systems unlike conventional

narrow-band systems. I want to see the effect of antenna performance parameters

pertaining to UWB cases included in the system performance analyses.

David Leeper:

Reason for my "no" vote:

FCC Part 15 requires that UWB systems not cause undue interference to other services.  And our PAR requires that proposals be compared with that capability in mind. I am concerned about UWB interference into future wireless systems that may be operating in close physical proximity to UWB systems on frequencies that lie anywhere in the 3-10 GHz band, but especially those in the 3 to 5 GHz band.  Since we cannot predict exactly what frequencies these future systems will use, we need to be prepared for "on the fly" detection and protection.

Therefore: I would consider changing my "no" vote to a "yes" if I were to see *practical* proposals for DS-UWB spectrum shaping that could be used to protect other nearby services from UWB.   By "practical" I mean reasonable in terms of gate count, power consumption, and cost.  This should include a capability for *on the fly* spectrum shaping to cover unforeseen interference scenarios to at least 2 or more other services -- for example services at 3.6 and 4.2 GHz. I would also need to see the UWB performance penalty that results from providing that protection to those services.  If our choice of modulation scheme does not include practical capabilities for spectrum shaping, we may find our UWB standard is very short-lived!

Susan Lin:

I will change my ‘No’ vote for merged proposal #2 to ‘yes’ when the following are satisfied.

1.  Proposal #2 needs to achieve the spectral flexibility as MB-OFDM in worldwide compliance.

2.  Proposal #2 needs to show FCC compliance approval.

3.  Complexity and range numbers in multipath environments seem optimistic and do not reflect the performance of current IC solution using DS-UWB.  Proposal #2 needs to show and justify all assumptions made to achieve these performance numbers.

Andy Molisch:

I might change my no-vote if

1) flexible spectral shaping would be mandatory in the proposal

2) clear explanations about the required equalizer structure would be given, and the required computational effort for equalization would be small

3) a receiver structure would be suggested that can achieve range comparable with the MBOA proposal, with a digital sampling rate of no more than 528Mbit/s

4) simulation results for narrowband interference suppression in the presence of amplifier nonlinearity and 1-bit A/D conversion resolution were given

5) all simulation results required by the selection criteria document, with all the details specified in there, were provided, and the performance is satisfactory in all of the considered cases

.

Yves-Paul Nakache:

I will consider changing my NO vote to a YES if the following considerations are satisfied and questions are answered:

1/ I would need an example of two standard waveforms (for band 1 and band

2) and several examples which show that it is possible to create a notch for different frequencies, using the interference mitigation option 1 and option 2 (slides 21 and 22 of presentation IEEE 802.15-04/140r7).

2/ In order to create notches using option 1 or 2, we may increase the peak to average ratio of the PSD. We see for example that we have to back off the power up to 3 dB with option 1 (slide 21).

I would need a table with the power back off and the loss of performance for each of the previous examples using the interference mitigation option

1 and option 2.

3/ Substantial proofs of the feasability of such adaptive notches with an evaluation of complexity of the complete transceiver including the interference mitigation option 1 and option 2. (A comparison between the two schemes would be useful)

4/ For each of the above examples, an estimation of the time for the algorithm of option 1 to converge to a solution. Are the solutions of this optimization process stable?

5/ How many pulses (or delay lines) are necessary with option 1 to create a notch at a given frequency to target a power back off of less than 1 dB.

What is the effect of sensitivity (amplitudes and delays) on the depth of the notch, its position in frequency and the power back off. In other words what should be the operational tolerance of these amplitudes and delays in order to preserve the characteristics of the notch.

6/ The demonstration of a working prototype that implements effective protection using option 1 or 2 for specific licensed services without degrading information throughput.

7/ The DS-UWB proposal uses symbols made with an N-chip code sequence. the interference mitigation option 2 propose to use a spreading code to create notches.

How to 'insert' this spreading code in the system?

If these two codes are the same, the design of the spreading code for the creation of a notch affects the number of orthogonal N-chip code sequences. How many SOP can we have if several piconets need to create a notch at the same frequency?

If these two codes are different, what are the length of the new waveform composed of M delayed pulses, the new symbol duration and therefore the new achievable data rate?

8/ Substantiated proof that the analog RF sections are realizable with the Optional Improvement for adaptive Interference Mitigation ('adaptive'

delay lines,etc).

9/ It seems that we have notches of more than 10 dB every 150MHz on slide

22 of presentation IEEE 802.15-04/140r7. Therefore I would need the assurance that a system including the interference mitigation option 2 is still FCC compliant.

10/ The same concern applies to the option 1: as seen on slide 21 of presentation IEEE 802.15-04/140r7, the positions and number of the notches depend on the delay between the pulses. With this example, if we want to have a short waveform, the delay of slide 21 should be small. Smaller the delay is, closer the notches are. I am concerned about the FCC compliance of such system.

11/ As a result I would need the full disclosure of implementation details on the DS-UWB prototypes fabricated according to the current DS-UWB proposal with option 1 or 2, and complete FCC testing results demonstrating that such system is FCC compliant.

Masaki Noda:

The following items are my major concerns that would need to consider changing my no vote to a yes vote, regarding merge proposal #2.

lit isn't clear if merge proposal #2 meets out of band emissions worldwide, so it would need to be certified by regulatory agencies.

2)MB-OFDM is currently supported by the major consumer electronics manufactures and the major alliances such as WiMedia Promoters and Wireless UWB Promoters. I would change my no vote after DS-UWB receives any major support from the industry.

Jonghun Park:

The following item is my major concern.

There are insufficient details on the performance using normal(or worst) case situation.
I would like to know the performance in that case.

Javier del Prado Pavon:

- Lack of spectral flexibility and hence

- worse performance against interferences

- concerns with worldwide regulatory domains

- No clear IP position in merged proposal 2

- Lack of industry support

Jay O'Conor:

The reasons for my 'no' vote are as follows: 

* Lack of information concerning Power Consumption 

* Uncertainty of IP positions within Merged Proposal #2 

* Lack of industry support 

* Lack of support from Wireless USB Key Developers group 

* Apparent lack of flexibility to conform to differing international regulatory requirements

Phil  Orlik:

The following are some reasons for the no vote:

- The current proposal as I understand it includes a  common signaling mode.  This increases the complexity of the system and I believe is also violates the PAR.  CSM would have to be removed from the proposal for me to want to change my vote to yes.

- I currently do not have a clear opinion as to the capability of merge proposal #2s scale in data rate and range, as described in the 802.15.3a selection criteria.  I would need, at a minimum, convincing simulations studies in ALL specified channel models that show performance of all operational modes.

- I'm worried that there are few vendors of silicon that are in a position to bring a solution to market in a timely fashion.  Proof of several vendors and reasonable time to market would be needed in order to change my vote.

Dave Patton:

The following items are my   concerns that would need to be addressed and fixed for me to consider changing my no vote to a yes vote, in support of merge proposal #2.

Time to market

Merge proposal #2 claims time to market advantage of silicon that is supposedly available in the market as generation 3; however, merge proposal #2 has recently drastically changed and the silicon does not represent the proposal before us; does not meet performance requirements as specified by the selection criteria, including power consumption. For me to change my no vote to yes would require seeing working silicon that implements the proposal as I fear there is significant time to market concerns for merge proposal #2.

Common signaling mode

The inclusion of a common signaling mode violates the PAR, so this would need to be removed from the proposal for me to want to change my vote to yes.

No ecosystem adoption

There would need to be a certification and compliance program in place to support the PHY standard for me to change my no vote to yes; as there is fear of interoperability if there is no organization. Merge proposal #1 has a compliance and certification program in place, as specified by the WiMedia Alliance.

There would need to be adoption of merge proposal #2 by major industry segments for me to believe that the industry believes the performance capabilities of merge proposal #2. Merge proposal #1 has major industry adoption, including the Wireless USB Promoters Group, and I fear that merge proposal #2 has minimal industry support.

Multiple vendors

There would need to be multiple vendors of silicon to enable a competitive ecosystem for me to change my no vote to a yes.

Lack of regulatory clarification

Merge proposal #2 silicon would need to be certified by regulatory agencies as it isn’t clear if their proposal meets out of band emissions. Further, the lack of spectrum flexibility (band selection) concerns me in the worldwide regulatory environment because of a lack of ability to support radio astronomy bands. There is strong concern that merge proposal #2 can serve as a worldwide radio in its present form.

Multipath

Merge proposal #2 would need to show performance criteria with a direct correlation to transceiver architecture and complexity for me to be comfortable with its ability to operate in more sever channels such as CM2, CM3, and CM4.

Intellectual Propterty

It was brought up during the IEEE 802.15.3a meetings that there are companies that may have intellectual property claims to merge proposal #2, and I would need to feel comfortable with their RAND statements before changing my no vote to a yes, especially as it relates to direct sequence spread spectrum technologies and ultrawideband impulse radios, rake receivers, etc…

Pekka Ranta:

MBOA provides better range, out of band-emissions levels, and adaptation to worlwide regulatory environment.

Information freescale provided about MBOA power consumption and scalability were misleading. Freescale to my knowledge is not developing MBOA chipsets.

Charles Razzell:

For the record, I would like to state my main reasons for voting against the confirmation of the DS-UWB proposal:

- I am concerned about the true complexity of a receiver that would perform well for DS-UWB at all data rates.  Although complexity data has been presented, it has been quite selective (for example low constraint length codes for high data rates, DFE complexity not tracking with data rates etc.) I consider changing my no vote to a yes if it could be demonstrated that high performance and low complexity could be achieved simultaneously for DS-UWB (as opposed to having the option to make high performance or high complexity).

- I consider the spectral flexibility of OFDM is essential for a world-wide standard that may have to accommodate spectral mask cut-outs in different regulatory regimes than the US.  It would be good if the standard adopted by the IEEE is well suited for world-wide adoption. I would change my no vote to a yes if DS-UWB could demonstrate the same level of spectral flexibility as provided by an OFDM approach in a cost-effective and practical manner.

- Out of band emissions are greater and harder to control for a DS-UWB system, since it relies on RRC pulse shaping to determine OOB. Due to the very high sample rates of the DS-UWB system, a digital implementation of this filter may have limited length and limited stop-band attenuation and due to the high clock rate would be of high power dissipation.  An analog version of this filter may suffer from poor accuracy and repeatability.

- High performance channel equalization for DS-UWB at low power and low complexity may require implementation IP that is only available to the authors of the proposal. In contrast, once you have a suitable FFT block (which has been shown by multiple contributions to require approx. 50 - 70k gates), a high performance equalization of the channel is virtually guaranteed for MB-OFDM and will be consistent between different vendors. I would consider changing my vote from a no to a yes if a reference model of the main receiver structures for DS-UWB were provided (not optimal in gate count and power, but sufficient to provide the claimed link performances for DS-UWB).

- Analog and mixed signal power consumption may be excessive with respect to MB-OFDM due to the need to provide channel filters of 1.3GHz bandwidth and ADCs running at 1.3GHz. The added implementation challenges that come with increased intrinsic bandwidth have not been adequately addressed in IEEE presentations from the DS-UWB proponents. It would be helpful to convince semiconductor vendors that the 1.3GHz receiver approach is practical, especially for mass-market integration technologies and price points. I would consider changing my no vote to a yes if it could be shown that the analog and mixed signal implementation for DS-UWB does not require excessive power consumption and can use low-cost/mass-market semiconductor processes.

- I am sceptical about the practicality of the unprotected 1Gbps data rate. Although a truly wide-band channel can reduce the impact of fading on link outages, I don't believe the DS-UWB proponents have adequately addressed the need to protect against co-channel and impulsive interference. I believe therefore that such claims may be misleading to the IEEE voters and the market in general.  I would consider confirming the DS-UWB proposal if the claims for scalability to high data rates are shown to be practical in real-world interference environments.

Jaeho Roh:

I vote “No” due to the followings.

1. I want to know more simulation results of DS-UWB system for supporting the ultra high speed.

2. I currently know that most of companies currently refused to accept DS-UWB system not MB-OFDM. So I worried about compatibility with other company’s goods

Zafer  Sahinoglu:

Merge proposal #1 has a compliance and certification program, which is specified by the WINA, but the merge proposal #2 not. This increases the possibility that merge proposal #2 will lack wide industrial support.

Also, the data rate and range scalability of the merge proposal #2 is not yet clear.

Erik Schylander:

Here are my concerns that may change my no to a yes:

1. I am concerned about the chip complexity needed to make it work reliable in a multi-path and dynamic fading environment.

2. I'm also concerned about the flexibility to allow worldwide usage and regulatory applicability.

Tomoki Saito:

The following items are my no vote reasons to Merge proposal #2.

1) No spectural flexability to the worldwide regulatory envirnments.

2) No support from major companies in the PC, CE and Mobile area, and

no support from major worldwide industory groups such as Wireless

USB promorters group, WiMedia and so on.

Shusaku Shimada:

No Vote on Confirmation Explanation;

Last year of efforts and discussions in this group should deserve to have a clear consequence of whether MB-OFDM proposal abide by FCC UWB rules, or not. This result would have a significant meaning in the international regulatory bodies.

Also WG chair introduced to this committee that Mr. Jurious Knapp's correspondence from FCC saying that they are carrying out the intensive interference characteristic test and the conclusion will come out soon.

I can't help voting "Against" before these prospected crucial information are provided.

V Somayazulu:

The following are among the reasons I vote "no" to confirm:

1.  CMOS feasibility:  I would like to see evidence of the implementability in standard digital CMOS process technology of the RF portion in particular, with power consumption estimates of the same.

2.  Flexibility in the face of uncertain regulatory and operating environments:  I would like to see evidence of how the DS-UWB system can, in a *practical* manner (i.e., substantiated with performance of the detection and mitigation algorithms, additional gate count, power estimates required to implement these, etc.), mitigate interference to, as well as contend with interference from "inband" narrowband systems such as WiMAX.  Also, I would like you to quantify the impact on the DS-UWB system of having to meet strict out of band emissions requirements for services such as IMT-2000 etc. corresponding to proposed protection levels being discussed in ITU-R, etc.

3.  SOP performance : isolation between interfering piconets occupying the lower band needs to be better than the stated d_int/d_ref = 0.66 for 2-3 uncoordinated interfering piconets in some applications  - I would like to see evidence of how this can be met.

4.  CCA:  The performance of the proposed mechanism for CCA based on different chipping rates and codes for the SOPs needs to be substantiated in the channel models specified by the selection criteria.

Mike Tanahashi:

There should be multiple silicon vendors to enable a competitive market for me to change my no vote to a yes.

Kazuaki Takahashi:

The reason for my no vote is that

spectrum flexiblity to adopt worldwide radio regulatory would need to be addressed.

Steve Turner:

I have voted NO to confirm for the following reasons:

1.  XSI/Motorola/Freescale have not answered direct technical questions posed to them regarding their proposal over the past year, regarding the performance of their proposed solution, even though they have had multiple opportunities to do so.  In addition to not answering direct questions posed to them in the IEEE sessions, they also did not provide answers to these questions in the ad hoc group moderated by Tom Siep.

2.  Freescale has not addressed the IP concerns with their proposal.

3.  Freescale has not addressed the questions raised regarding time to market issues with their proposal.  The proposal has changed significantly several times, and yet they have continued to say they have ICs ready to ship with each version of their proposal.  This is not a realistic statement.

4.  Freescale has not shown the ability to achieve worldwide regulatory compliance, as the MB-OFDM proposal has.  It is critical for suppliers to be able to ship one standard solution worldwide, and Freescale has not shown how they can accomplish this.

Gerald Wineinger:

I  voted no for confirmation of DSUWB (Merged Proposal #2) because:

Simulation results and simulation procedures for DSUWB have been requested several times. The group proposing DSUWB (Merged Proposal #2)  have never provided those results.

I would be willing to change my vote if adequate simulation results and procedures were provided.

Hirohisa Yamaguchi:

Following are my reasons for NO.

Lack of the validity
Part of the missing system parameters (code table etc.) was presented for the first time at this meeting. Information is still insufficient in the other parts of the proposal (equalizer, rake, etc.), and this problem has been keeping me from verifying the validity of the proposal. As of today, there is little data to prove the proposed performance. The shown comparison between MB-OFDM and DS-UWB for the higher data-rates is not consistent with the data presented for the 90% outage range (at 500 Mbps, 3.0 m for CM1 and 1.9 m for CM2 in DS-UWB, while at 480 Mbps, 2.9 m for CM1 and 2.6 m for CM2 in MB-OFDM). I hereby question if their statement such as ‘DS-UWB takes full advantage of UWB propagation, and DS-UWB performance excels as speed goes up. Performance difference is natural consequence of channel physics.’ as stated in their proposal, has any ground. I require a convincing explanation before ever reconsidering my NO vote.

Lack of the interference solution capability
Lack of any effective mechanism to solve the OOB and in-band interference for the incumbent band users. This seems to be one indispensable feature for the UWB device to coexist with the existing and future radio services. Number of the suggested notch techniques by the DS-UWB proposers either accompany excessive performance penalty or lack technical feasibility. MB-OFDM is the frequency-domain signal processing, and possesses the necessary key feature. Without any acceptable solution proposed from the DS-UWB side, I would not ever reconsider my NO vote.

Concern on the time to market
DS-UWB proposal has been much changed from the original 2002 proposal. No convincing explanation has been given whether DS-UWB meets the original time-to-market assertion. Clear explanation must be given before I reconsider my NO vote.
 
Lack of the industry support
MB-OFDM is currently supported by ten major semiconductor manufactures (Infineon, Intel, Mitsubishi, NEC Electronics, Philips, Samsung, ST Micro, TI, Toshiba, Renesas) as well as by the major consumer-electronics manufactures (Mitsubishi, Olympus, Panasonic, Philips, Samsung, Sharp, SONY, Toshiba, Hitachi). Big industry alliances such as WiMedia Alliance and Wireless UWB Promoters have also officially announced their support of MB-OFDM. Industry has clearly chosen MB-OFDM as the preferred standard, and I would not ever reconsider my NO vote unless DS-UWB receives equal or larger support from the industry.