IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Strawman Link Budget |  |
| Date Submitted | 14 September 2004 |  |
| Source | Colin Lanzl 83 Nottingham Drive, Nashua, NH, 03062, USA | Voice: +1-603-889-0526 Fax: E-mail: clanzl@ieee.org |
| Re: | PHY considerations in 802.15.4b |  |
| Abstract | This document provides a strawman proposal for a link budget for 802.15.4b PHY work in the 802.15.4 bands below 1 GHz. |  |
| Purpose | This document is provided in support of 802.15.4b PHY activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Strawman proposal for 802.15.4b PHY link budget for devices         < 1GHz

The purpose of this link budget is to define the baseline elements that 802.15.4b members can use in evaluation of proposals.  The intent is to provide a common set of PHY radio parameters to aid proposers in the preparation of their system simulations.

Pathloss model: Two-breakpoint model from 15-04-0462-02-004a-pathloss-propagation-model-    comparison.ppt (see Appendix 1 here for a copy of that model).

Propagation environment: 			1.0usec RMS delay spread

Antenna gain: 				0dBi (isotropic radiation pattern)

Receiver equivalent noise figure (antenna to baseband): 15dB

Frequency bands addressed:	868MHz			915MHz

Occupied Bandwidth:		0.5MHz			2.0MHz

Transmitter output power:		+6dBm			+6dBm

Desired baseband data rate:		200kbps, minimum

Proposer to specify the minimum Eb/N0 necessary to achieve a 1% PER for 26 octet frame (20 bytes payload, 6 bytes header) in their system.

Appendix 1: Propagation pathloss model

PL(d )= -10log{[c/(4dfm)]2 [1-exp(-(dt1 /d)-2)][1-exp(-(dt2/d)g2- g1]},

where

PL: 	pathloss, dB;

d:	distance between transmitter and receiver;

fm:   	geometric mean of  transmitted frequency band;

1:    	propagation constant after first breakpoint (3.7);

2:    	propagation constant after second breakpoint (10.5);

c:     	velocity of propagation;

dt1:   	first breakpoint distance from transmitter (5 meters);

dt2:   	second breakpoint distance from transmitter (30 meters).