IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Comparison of SAP for QoSC in PAL vs QoSC in MAC |  |
| Date Submitted | 14 November 2004 |  |
| Source | Mike Rudnick (Appairent Tech.) 150 Lucius Gordon Dr., Suite 211 West Henrietta, NY 14586, USA | Voice: +1-585-214-2454 Fax: +1-585- E-mail: cmrudnick at appairent.com |
| Re: |  |  |
| Abstract | This document examines and compares two possible SAP, one to support performing QoS in the PAL and the other to support performing QoS in the MAC. |  |
| Purpose | This document is provided in support of 802.15.3b activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Comparison of SAP for QoSC in PAL vs QoSC in MAC

# Introduction

This document contrasts two possible MLME SAP corresponding to two possible locations for the QoSC (QoS Controller).  One supports performing QoS in the PAL (QoSC in PAL); the other support QoS in the MAC (QoSC in MAC).  The QoSC-in-PAL SAP is based on the QoS SAP presented in 04/591.  The QoSC-in-MAC SAP is based on the QoS portion of the SAP presented in 04/401r2.

First a diagrammatic overview of each SAP is presented along with its salient features.  Then a comparison of the two approaches and their respective SAP is presented.

# QoSC-in-PAL SAP

This section details the QoSC-in-PAL SAP shown in Figures 1-5.  Both new MLME SAP (beyond already existing 802.15.3 MLME SAP) and the already existing 802.15.3 MLME SAP that must be accessed/used when QoSC resides in the PAL are shown.  MLME primitives and parameters that are unchanged from 802.15.3 are shown in violet throughout.

Figure 1: Overview of QoSC-in-PAL SAP’s Structure

Figure 1 shows an overview of the structure of the QoSC-in-PAL SAP.  As described in 04/591, it consists at the top level of two parts.  The first part is a measurement SAP that supports measuring both the QoS currently being delivered (shown in red) and current channel conditions (shown in orange).  The second part is a WPAN and stream control SAP that supports manipulating WPAN and stream free parameters to achieve the desired level of QoS (shown in green).

Figure 2:  Overview of Measured QoS part of QoSC-in-PAL Measurement SAP

Figure 3:  Overview of Measured Channel Condition part of QoSC-in-PAL Measurement SAP

Figure 4: Overview of Control part of QoSC-in-PAL SAP

Figure 5:  Overview of Control part of QoSC-in-PAL SAP (continued from Figure 4)

Figure 2 presents an overview of the measured QoS portion of the measurement SAP.  An unresolved issue for the MLME-STREAM-QoS-STATUS.ind primitive is the following question:  How to know when to give an indication the delivered QoS has changed?

Different answers are possible, some with SAP change implications.  For example, a simple percentage change criteria could be used.  The required percentage change for a QoS status indication to be generated could then be set as a QoS policy parameter. However, it may well be the case that only one or some of the QoS measures are relevant for a particular application (or other higher-layer entity).  In this case, either the application (or other higher-layer entity) could ignore indications signaling change in irrelevant QoS measures.  Alternately, the SAP could be structured so that the application (or other higher-layer entity) could specify which QoS measure changes are to be indicated and how much change is required before an indication is generated.  This could be done on a per-QoS-measure basis or on a power set subset basis.  There are lots of possibilities for how this could be handled.

Figure 3 presents an overview of the measured channel conditions SAP portion of the measurement SAP.  The MLME-CHANNEL-STATUS.ind primitive is similar to the MLME-STREAM-QoS-STATUS.ind primitive in that there are lots of possible ways to determine what counts as a significant change and how it is specified.

The measurement portion of the peer channel-condition part of the measured channel condition SAP is problematic because there are many different ways to characterize and quantify the condition of the peer channel, including characterizing ambient noise levels at the source and destination.  In general, these measurements will be both PHY dependant and PHY-implementation dependant.  Attempts to standardize this aspect of QoS are likely to limit innovation and thereby limit performance of QoSC-in-PAL-delivered QoS.

Figures 4 & 5 present an overview of the control SAP portion of the QoSC in PAL SAP.

# QoSC-in-MAC SAP

This section details the QoSC-in-MAC SAP.   Figure 6 shows the entire MAC-to-higher-layer QoSC-in-MAC SAP.  The SAP consists of two parts, a measured QoS part and a control part.

The MLME-STREAM-QoS-STATUS.ind primitive has the same issues discussed in the QoSC-in-PAL SAP section about what counts as a ‘significant’ change, i.e., indication threshold.

Figure 6:  QoSC-in-MAC SAP.

# Comparison of QoSC/PAL & QoSC/MAC SAP

The most striking difference between the QoSC-in-PAL shown in Figures 1-5 and the QoSC-in-MAC SAP shown in Figure 6 is the difference is size and complexity.  Figures 1-5 take four pages to show a diagrammatic view of the MAC-to-higher-layer SAP needed to support QoSC in the PAL, while the entire MAC-to-higher-layer SAP needed to support QoSC in the MAC takes only a single page.  This difference is a result of the higher level of architectural coherence obtained when putting QoSC in the MAC as compared to putting it in the PAL.

Importantly, a significant issue remains unresolved for the QoSC-in-PAL architecture and SAP:  How are multiple PAL to be accommodated when each is attempting to independently manage QoS?  The main issue is fairness among and between multiple PAL.  Another way to view the problem is as follows:  How is multiplexing and/or coordination of shared control for things like superframe duration, superframe position, and channel selection to be accomplished?