7.1.1.2.1 Semantics of the service primitive

2003

The semantics of the MCPS-DATA.confirm primitive is as follows:

|  | MCPS-DATA.confirm | ( msduHandle, status ) |
| --- | --- | --- |

Table 28 specifies the parameters for the MCPS-DATA.confirm primitive.

# Table 28—MCPS-DATA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| msduHandle | Integer | 0x00 – 0xff | The handle associated with the MSDU being confirmed. |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, INVALID_GTS, NO_ACK, UNAVAILABLE_KEY, FRAME_TOO_LONG, FAILED_SECURITY_CHECK, or INVALID_PARAMETER | The status of the last MSDU transmission. |

4b

The semantics of the MCPS-DATA.confirm primitive is as follows:

|  | MCPS-DATA.confirm | ( msduHandle, Timestamp, status ) |
| --- | --- | --- |

Table 28 specifies the parameters for the MCPS-DATA.confirm primitive.

# Table 28—MCPS-DATA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| msduHandle | Integer | 0x00 – 0xff | The handle associated with the MSDU being confirmed. |
| Timestamp | Integer | 0x00000– 0xfffff | The time, in symbols, at which the data was transmitted. The value of this paramater will only be considered valid if the value of the status parameter is SUCCESS. |
| status | Enumeration | SUCCESS, TRANSACTION_OVERFLOW, TRANSACTION_EXPIRED, CHANNEL_ACCESS_FAILURE, INVALID_GTS, NO_ACK, UNAVAILABLE_KEY, FRAME_TOO_LONG, FAILED_SECURITY_CHECK, or INVALID_PARAMETER | The status of the last MSDU transmission. |

7.1.1.3.1 Semantics of the service primitive

2003

The semantics of the MCPS-DATA.indication primitive is as follows:

|  | MCPS-DATA.indication | ( SrcAddrMode, SrcPANId, SrcAddr, DstAddrMode, DstPANId DstAddr, msduLength, msdu, mpduLinkQuality, SecurityUse, ACLEntry ) |
| --- | --- | --- |

Table 29 specifies the parameters for the MCPS-DATA.indication primitive.

# Table 29—MCPS-DATA.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00 – 0x03 | The source addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16 bit short address. 0x03 = 64 bit extended address |
| SrcPANId | Integer | 0x0000–0xffff | The 16 bit PAN identifier of the entity from which the MSDU was received. |
| SrcAddr | Device address | As specified by the SrcAddrMode parameter | The individual device address of the entity from which the MSDU was received. |
| DstAddrMode | Integer | 0x00 – 0x03 | The destination addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16 bit short device address. 0x03 = 64 bit extended device address. |
| DstPANId | Integer | 0x0000 – 0xffff | The 16 bit PAN identifier of the entity to which the MSDU is being transferred. |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The individual device address of the entity to which the MSDU is being transferred. |
| msduLength | Integer | ≤aMaxMACFrameSize | The number of octets contained in the MSDU being indicated by the MAC sublayer entity. msdu Set of octets — The set of octets forming the MSDU being indicated by the MAC sublayer entity. |
| mpduLinkQuality | Integer | 0x00–0xff | LQ value measured during reception of the MPDU. Lower values represent lower LQ (see 6.7.8). |
| SecurityUse | Boolean | TRUE or FALSE | An indication of whether the received data frame is using security. This value is set to TRUE if the security enable subfield was set to 1 or FALSE if the security enabled subfield was set to 0. |
| ACLEntry | Integer | 0x00 – 0x08 | The macSecurityMode parameter value from the ACL entry associated with the sender of the data frame. This value is set to 0x08 if the sender of the data frame was not found in the ACL |

4b

The semantics of the MCPS-DATA.indication primitive is as follows:

|  | MCPS-DATA.indication | ( SrcAddrMode, SrcPANId, SrcAddr, DstAddrMode, DstPANId DstAddr, msduLength, msdu, mpduLinkQuality, SecurityUse, ACLEntry, Timestamp ) |
| --- | --- | --- |

Table 29 specifies the parameters for the MCPS-DATA.indication primitive.

# Table 29—MCPS-DATA.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SrcAddrMode | Integer | 0x00 – 0x03 | The source addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16 bit short address. 0x03 = 64 bit extended address |
| SrcPANId | Integer | 0x0000–0xffff | The 16 bit PAN identifier of the entity from which the MSDU was received. |
| SrcAddr | Device address | As specified by the SrcAddrMode parameter | The individual device address of the entity from which the MSDU was received. |
| DstAddrMode | Integer | 0x00 – 0x03 | The destination addressing mode for this primitive corresponding to the received MPDU. This value can take one of the following values: 0x00 = no address (addressing fields omitted). 0x01 = reserved. 0x02 = 16 bit short device address. 0x03 = 64 bit extended device address. |
| DstPANId | Integer | 0x0000 – 0xffff | The 16 bit PAN identifier of the entity to which the MSDU is being transferred. |
| DstAddr | Device address | As specified by the DstAddrMode parameter | The individual device address of the entity to which the MSDU is being transferred. |
| msduLength | Integer | ≤aMaxMACFrameSize | The number of octets contained in the MSDU being indicated by the MAC sublayer entity. msdu Set of octets — The set of octets forming the MSDU being indicated by the MAC sublayer entity. |
| mpduLinkQuality | Integer | 0x00–0xff | LQ value measured during reception of the MPDU. Lower values represent lower LQ (see 6.7.8). |
| SecurityUse | Boolean | TRUE or FALSE | An indication of whether the received data frame is using security. This value is set to TRUE if the security enable subfield was set to 1 or FALSE if the security enabled subfield was set to 0. |
| ACLEntry | Integer | 0x00 – 0x08 | The macSecurityMode parameter value from the ACL entry associated with the sender of the data frame. This value is set to 0x08 if the sender of the data frame was not found in the ACL. |
| Timestamp | Integer | 0x00000– 0xfffff | The time, in symbols, at which the data was received. |

7.4.2 MAC PIB attributes

4b

New attribute for table 71.

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| macSyncSymbolOffset | 0x55 | Integer | 0x000 – 0x100 for the 2.4Ghz PHY 0x000 – 0x400 for the 868/915Mhz PHY | The symbol boundary within the frame at which the MLME captures the timestamp of each transmitted or received frame. (A macSyncSymbolOffset of zero corresponds to the onset of the first symbol past the SFD, namely the length field.) | 0x00000 |