IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Physical layer submission to Task Group 3a |  |
| Date Submitted | [4 January 2005] |  |
| Source | [Gadi Shor, Sorin Goldenberg] [Wisair] [24 Raoul Wallenberg st., Tel-Aviv ISRAEL] | Voice: [ +972 3 767 6605 ] Fax: [ +972 3 647 7608 ] E-mail: [gadi.shor@wisair.com ] |
| Re: | 802.15.3a Call for proposal |  |
| Abstract | UWB low rate communication, ranging and location proposal |  |
| Purpose | Response for TG4a CFP |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Physical Layer Submission to Task Group 4a

# Introduction

In this document, we propose a solution for IEEE 802.15.4a. The solution meets worldwide regulation, co-exists with current and future wireless networks, and has the ability to mitigate their interference. The solution supports bit rates ranging from a few Kbps to a few hundreds Kbps, with a trade-off between the power consumption and range vs. bit-rate. The proposal supports ranging and location.

# Proposed System basics

## Frequency Plan

The frequency range 3.1-10.6 GHz is divided into 14 bands, 528 MHz each:

| BAND_ID | Lower frequency | Center frequency | Upper frequency |
| --- | --- | --- | --- |
| 1 | 3168 MHz | 3432 MHz | 3696 MHz |
| 2 | 3696 MHz | 3960 MHz | 4224 MHz |
| 3 | 4224 MHz | 4488 MHz | 4752 MHz |
| 4 | 4752 MHz | 5016 MHz | 5280 MHz |
| 5 | 5280 MHz | 5544 MHz | 5808 MHz |
| 6 | 5808 MHz | 6072 MHz | 6336 MHz |
| 7 | 6336 MHz | 6600 MHz | 6864 MHz |
| 8 | 6864 MHz | 7128 MHz | 7392 MHz |
| 9 | 7392 MHz | 7656 MHz | 7920 MHz |
| 10 | 7920 MHz | 8184 MHz | 8448 MHz |
| 11 | 8448 MHz | 8712 MHz | 8976 MHz |
| 12 | 8976 MHz | 9240 MHz | 9504 MHz |
| 13 | 9504 MHz | 9768 MHz | 10032 MHz |
| 14 | 10032 MHz | 10296 MHz | 10560 MHz |

## Symbol definition, Modulation and Coding Scheme

### Symbol definition

The modulation scheme is based on transmission of 312.5 nSec symbols. The symbol is based on shaped hierarchical sequences:

- 16x8 samples hierarchical sequences

- 37 samples zero padding suffix

- 528 Mcps/165 samples = 3.2 Msps

- For lower data rates symbols can be sent  in a lower duty cycle (can replace transmit  power control and save power consumption)

- Both shaped (flat PSD) and unshaped (constant envelope) allowed

### Modulation scheme

The raw data is modulated in two dimensions to allow both coherent and non-coherent reception:

- Phase modulation (BPSK)

- Orthogonal symbol set modulation (M=2)

- Different interleaving (and coding) for each dimension

- Data can be demodulated from a single dimension

Remark: The modulation method can be replaced by differential encoding (under study)

### Coding scheme

The coding scheme is a concatenation code with repetition code. The raw data is spread over frequency (by definition of the symbol) and over time.

- Coding is systematic (or reversible) to allow reception with no decoder

Remark: Alternatives coding schemes are under study (ideas welcome)

### Interleaving scheme

The interleaving scheme is based on a simple block interleaving

- Two different interleaving (and coding) scheme, one for each modulation dimension, are used

### Scrambler

The scrambler is similar to MB-OFDM scrambler

| PRBS Generator | g(D)=1+D14+D15 |
| --- | --- |
| Initial Seed Value | 4 Initial seed values |

## Transmission Modes

The proposed system has many operation modes to select from. The following table contains some examples:

| Mode | Coding Rate | Symbol Rate [Msym/sec] | Repetition and Duty Cycle factor | Data Rate [Kbits/sec] |
| --- | --- | --- | --- | --- |
| 1 | 1 | 3.2 | 10 | 320 |
| 2 | ¾ | 3.2 | 10 | 240 |
| 3 | ½ | 3.2 | 10 | 160 |
| 4 | ½ | 3.2 | 40 | 40 |
| 5 | ½ | 3.2 | 160 | 10 |
| 6 | ½ | 3.2 | 640 | 2.5 |

Remark: Repetition and Duty Cycle factor is the effective rate NxR after including the symbol duty cycle (i.e. 1 out of N) and the repetition code rate (i.e.  1/R)

## Interference mitigating for co-located simultaneously-operating UWB piconets

The proposal has inherent interference mitigation. In addition, interference mitigation techniques can be used in the receiver:

- Interference mitigation achieved through frequency and time diversity

- Co located simultaneously-operating piconets can use FDMA and TDMA

## Multiple-Access

Multiple-access is achieved through FDMA and symbol definition. TDMA can be added on top by the MAC layer.

## Preamble

The preamble is used for signal acquisition, channel estimation and other pre demodulation activities:

- Preamble is based on shaped hierarchical sequences

- Preamble uses a cover sequence and frame synchronization sequence

- Preamble contains channel estimation symbols for improved reception

- Three different preamble lengths supported

- Preamble includes symbols for antenna diversity

- Ranging and location

Modulation/demodulation scheme allows high resolution estimation of first path (and if needed complete channel):

- Used as the basis for the ranging procedure

- Can use special frame for improved ranging

- Information can be distributed through the communication system to allow location, based on multiple relative ranging operations

This range information can be obtained by estimating the round trip delay between the devices using the following algorithm:

- Dev A to Dev B: Send time A

- Dev B to Dev A: Time Diff A (Receive Time A - Send Time A ) and  Send Time B

- Dev A calculates

  - Time Diff B (Receive Time B - Send Time B )

  - Time between Dev A to Dev B = ½ (Diff A + Diff B)

# Implementation and Feasibility

## Transmitter

- Many possible architectures (constant envelope or constant PSD, with/without DAC)

- No need for I/Q modulator (BPSK)

- Can use digital (+/-1), DAC based or analog sequences

- Can be implemented using CMOS process

- Low power consumption / Small die size / Share MB-OFDM radio

## Receiver

- Many possible architectures

- Support for coherent and non-coherent receivers

- Tradeoff between complexity and performance

- Support analog and digital demodulation

- Correlation/de-spreading can be done in analog or digital domains

- ADC can work in bit rate, sample rate (or sample rate/8)

- Support for architecture with no ADC

# General Solution Criteria

This section defines the system level parameters of the solution.

## Unit Manufacturing Complexity (UMC)

The proposal can be implemented using CMOS process.

## Signal Robustness

The system can trade off complexity and performance

- High complexity receiver can collect all energy and use coherent detection plus error correction decoding

- Low complexity receiver can collect a single path and use non coherent detection without error correction decoding

Remark: Simulation results will be added.

### Interference and susceptibility

The system is designed to minimize interference to and from other radio services. Additional algorithms can be added in the receiver to mitigate interference.

Remark: Simulation results will be added.

## Technical Feasibility

### Manufacturability

The proposed solution can be implemented using CMOS process.

### Time to Market

The proposed solution can be implemented using existing hardware.

### Regulatory Impact

The proposed solution conforms to FCC regulation.

## Scalability

#### The proposed solution can be scaled up and down.

## Location Awareness

The system has the capability to determine the relative location of one device with respect to another.

# Evaluation Matrix

## General Solution Criteria

|  | CRITERIA | Evaluation |
| --- | --- | --- |
|  | Unit Manufacturing Cost (UMC) | + |
| Signal Robustness | Interference And Susceptibility | + |
|  | Coexistence | + |
| Technical Feasibility | Manufacturability | + |
|  | Time To Market | + |
|  | Regulatory Impact | + |
| Scalability |  | + |
| Location Awareness |  | + |

## PHY Protocol Criteria

| CRITERIA | Evaluation |
| --- | --- |
| Size and Form Factor | + |
| Payload Bit Rate | + |
| Packet Overhead | + |
| PHY-SAP Throughput | + |
| Simultaneously Operation Piconets | + |
| Signal Acquisition | + |
| System Performance | + |
| Link Budget | + |
| Sensitivity | + |
| Power Management Modes | + |
| Power Consumption | + |
| Antenna Practicality | + |