IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | <title> DBO-CSK Proposal for IEEE802.15.4a |  |
| Date Submitted | [04 January, 2005] |  |
| Source | [(1) Kyung-Kuk Lee, (2) J.W.Chong, S.H.Yoon, J.D.Jeong, S.D.Kim, H.U.Lee] [(1)Orthotron Co.,Ltd. (2) Hanyang Univ.] [address] (1) 709 Kranz Techono, 5442-1 Sangdaewon-dong, Jungwon-gu, Sungnam-si, Kyungki-do, Korea 462-120 | Voice: [82-31-777-8198] Fax: [82-31-777-8199] E-mail: [kyunglee@orthotron.com] |
| Re: |  |  |
| Abstract | [This document has been submitted for an official proposal in January 2005. DBO-CSK Technology is proposed.] |  |
| Purpose | [Proposal for the IEEE802.15.4a standard] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

< TABLE OF CONTENTS >

1.	Introduction	3

2.	M-ary DBO-CSK Technology	4

2.1.	Introduction to Chirp Signal	4

2.2.	Proposed Chirp Signal	6

2.3.	Chirp-Shift-Keying (CSK) Signal Sets for SOP	8

2.4.	Differentially Bi-Orthogonal CSK (DBO-CSK) Modulation	9

3.	General Solution Criteria	10

3.1.	Unit Manufacturing Cost/Complexity (UMC)	10

3.2.	General Definitions	11

3.3.	Signal Robustness	11

3.4.	Technical Feasibility	12

3.5.	Scalability	13

4.	MAC Protocol Supplement	14

4.1.	MAC Enhancements and Modifications	14

5.	PHY Layer Criteria	15

5.1.	Channel Models and Payload Data	15

5.2.	Size and Form Factor	15

5.3.	PHY-SAP Payload Bit Rate and Data Throughput	16

5.4.	Simultaneously Operating Piconets	17

5.5.	Signal Acquisition	22

5.6.	System Performance	23

5.7.	Ranging	24

5.8.	Link Budget	25

5.9.	Sensitivity	26

5.10.	Power Management Mode	26

5.11.	Power Consumption	27

5.12.	Antenna Practicality	27

# Introduction

- Low Power Consumption: Digital Tx 0.65mW, Rx 61.6mW

- Signal Robustness:

- Orthogonal / Quasi-Orthogonal Signal Set

- Robustness: Heavy Multi-path, SOP,

- Low Correlation with Existing Air-Interfaces

- Feasibility: 2.4GHz ISM Band

- Existing commercial RF Solutions

- Ranging: Based on Chirp Signal (TDOA)

- Precision: less then x cm (Standard Deviation)

- Size & Form Factor: Less than SD-Memory size

- Low Cost / Low Complexity: Tx +Rx Baseband Digital (54K gates)

- Advanced Sleep/Wake-up Capability

# M-ary DBO-CSK Technology

## Introduction to Chirp Signal

- Linear Chirp: Rectangular Window

t

t

- Linear Chirp: Raised-Cosine Window

0

0.2

0.4

0.6

0.8

1

1.2

1.4

-1

-0.8

-0.6

-0.4

-0.2

0

0.2

0.4

0.6

0.8

1

Auto-Correlation Property

## Proposed Chirp Signal

- Waveform of Chirp-Shift-Keying (CSK) Signals

- Spectrum of Sub-Chirp Signals

fC1 = 2.41775GHz

fC2 = 2.43375GHz

fC3 = 2.44975GHz

fC4 = 2.46575GHz

2.4

2.39

2.4835

2.4935

0dBr

-30dBr

-30dBr

-50dBr

-50dBr

f (GHz)

fc1

fc2

fc3

fc4

64MHz

## Chirp-Shift-Keying (CSK) Signal Sets for SOP

I

II

III

IV

Each of CSK Signal consists of 4 sub-chirp signals.

Correlation Property between different Chirp-signals

## Differentially Bi-Orthogonal CSK (DBO-CSK) Modulation

- Bi-Orthogonal Modulation

3 bits/symbol

Bi-Orthogonal Symbol

Mapping Table (M = 8)

Bi-Orthogonal Code

(01,02,03,04)

Binary

(b0,b1,b2)

Decimal

(m)

1  1  1  1

1 -1  1 -1

1  1 -1 -1

1 -1 -1  1

-1 -1 -1 -1

-1  1 -1  1

-1 -1  1  1

-1  1  1 -1

0                000

1                001

2                010

3                011

4                100

5                101

6                110

7                111

- 8-ary DBO-CSK Modulator

Chirp-Shift-Keying

(CSK)

Generator

S/P

Symbol

Mapper

Binary Data

…010011100110001101…

P/S

Binary Symbol

Bi-Orthogonal

Symbol

Differentially

Bi-Orthogonal

Symbol

8-ary DBO-CSK

# General Solution Criteria

## Unit Manufacturing Cost/Complexity (UMC)

- Transmitter Baseband £ 3k gates

  - Scrambler : 15bit register and 1bit XOR => 154 gates

  - Block Encoder : 300 gates

  - 1X3 S/P and 4X1 P/S : 2bit register and 3bit register => 50 gates

  - Symbol Mapper : 3X8 address decoder with hard-wire table => 13 gates

  - Differential Encoder : 4bit XOR and 4bit register => 56 gates

  - Chirp-pulse Modulator : eight 6X56 address decoder with hard-wire table

and four 4bit adder => 1680 gates

  - Other blocks : 700 gates

- Receiver Baseband £ 51k gates

  - Differential Detector : 3240 registers, complex multiplier and four adders

=> 38960 gates

  - Symbol Demapper : (ten 1bit XORs + four 3bit adders) X 2 => 200 gates

  - Max Selector : 3bit two-step comparator X 2 => 54 gates

  - Two 1X4 S/P and one 3X1 P/S: 3bit register X 2 and 2bit register => 80 gates

  - Block Decoder : 3k gates

  - Descrambler : 15bit register (» 10 gates/register) and 1bit XOR => 154 gates

  - Other blocks : 7k gates

- Analogue £ 5.6 mm2 Die (for 0.18 um library)

## General Definitions

- Payload bit rate and throughput

- 1Mbps throughput: 618Kbps

- 250Kbps throughput: 216Kbps

- Error rate: see sub-section 5.6

- Receiver sensitivity: see sub-section 5.11

- Antenna gain: 0dBi

- Band in use:

- 2.4GHz ISM Band

- 64MHz Bandwidth: Consists of 4 sub-chirp signals

## Signal Robustness

- Co-existence / Interference Mitigation Technique

-  Orthogonal / Quasi-Orthogonal Signal Set

-  High Spectral Processing Gain: Chirp

- Interference Susceptibility

-  Low Cross-Correlation property with Existing Signal

- Robustness:

- Heavy Multi-path Environment

- SOP

- Low Sensitivity for Component Tolerance

- Crystal : ± 40ppm

- Mobility

- Wide-band Chirp: Insensitive for Fading & Doppler Shift

## Technical Feasibility

- Manufacturability

- Baseband Digital Chip area: 0.7 mm2 (0.18um Technology)

- Time-to-Market

- 2005. 5. Proto-type DEMO (FPGA)

- 2006. 1. Digital ASIC

- Regulatory Impact

- 2.4GHz ISM Band: Globally Allowed to use

- Tx Power: 0.1mW / 1.0mW / 10mW optional class

- Some Critical Frequency Band within 2.4GHz: Can avoid Tx Power by Skip

- Modulator Block Diagram

< The proposed 8-ary DBO-CSK Modulator >

## Scalability

- Data-Rate:

- 2 rates:  1Mbps   or   250Kbps

- RF Tx Power:

- 3 classes:  0.1mW / 1.0mW / 10mW

- Mobility Value:

- Data:   Link Margin >= 30dB

- Ranging:  Preamble Duration < 800usec

- Chirp is insensitive for Doppler Shift

- Can detect signal from the mode of high-speed moving

# MAC Protocol Supplement

## MAC Enhancements and Modifications

- Supplement for Scalability

-  The proposed PHY has scalability for channelization

-  Scalability which is included in PHY should be added to MAC for 15.4a PHY layer

(Ex. Data-rate Assignment per Packet)

- Wake-up Mode for Power Consumption Consideration

-  Power consumption is of significant concern

-  The proposed PHY can support advanced wake-up mode with superior SOP performance

-  Needing supplement to 15.4 MAC to advanced wake-up mode for low-power consumption

# PHY Layer Criteria

## Channel Models and Payload Data

- Channel models and payload data

- CM1, CM2, & CM3

- Payload Data: 32bytes per Packet

- Data-rate: 1Mbps

## Size and Form Factor

Base

band

RF

Battery

Pattern Antenna (24mm X 20)

SD Memory Size (32mm X 24 mm)

## PHY-SAP Payload Bit Rate and Data Throughput

- Relation between Payload Bit-rate and Throughput

  - Payload bit-rate 1 Mbps : Throughput 618 Kbps

  - Payload bit-rate 250 Kbps : Throughput 216 Kbps

- Proposed PLCP Frame Format

Preamble

Delim

-iter

Length

+

Rate

32Chirp     8Chirp    12Chirp                 86chirp (1Mbps) or 342chirp(250Kbps)

MPDU

(8+1)bit                             32X8 bit

414 usec (1Mbps)  or  1182 usec (250Kbps)

## Simultaneously Operating Piconets

- Multiple Piconets

Duration of 2 Symbols (6microsec)

I

II

III

IV

- Simulation Results for SOP Performance in AWGN

Single Co-Channel Interferer Separation Distance

2 Adjacent Channel Interferers Separation Distance

3 Adjacent Channel Interferers Separation Distance

4 Adjacent Channel Interferers Separation Distance

- Simulation Results for SOP Performance in CM 1

Single Co-Channel Interferer Separation Distance

2 Adjacent Channel Interferers Separation Distance

3 Adjacent Channel Interferers Separation Distance

4 Adjacent Channel Interferers Separation Distance

## Signal Acquisition

- Signal Acquisition Method

Differential

Detector

(T1)

Symbol

De-Mapper

Select

Largest

Data

A/D

Differential

Detector

(T2)

- Miss Detection Probability

## System Performance

Data Rate : 1Mbps

Eb/No

Effect of Crystal

Tolerance (± 40ppm) :

 Negligible

PER

## Ranging

- TDOA Based Ranging with Chirp Signal

- Estimation Precision: < 1m  @ Eb/No greater than 22dB

## Link Budget

| Parameter | Mandatory value |  | optional value |  |
| --- | --- | --- | --- | --- |
| peak payload bit rate(Rb) | 250 | kbps | 1000 | kbps |
| Average Tx Power(Pt) | 10 | dBm | 10 | dBm |
| Tx antenna gain(Gt) | 0 | dBi | 0 | dBi |
| fc' = sqrt(fminfmax) -10dB | 2.44 | GHz | 2.44 | GHz |
| Path loss at 1meter(L1=20log10(4pifc'/c)) | 40.2 | dB | 40.2 | dB |
| distance | 30 | m | 30 | m |
| path loss at d m(L2=20log10(d)) | 29.5 |  | 29.5 |  |
| Rx antenna gain(Gr) | 0 | dBi | 0 | dBi |
| Rx power(Pr = Pt+Gt+Gr-L1-L2(dB)) | -59.7 | dBm | -59.7 | dBm |
| Average noise power per bit | -120.0 | dBm | -114.0 | dBm |
| Rx Noise Figure(Nf) | 7 | dB | 7 | dB |
| Average noise power per bit(Pn=N+Nf) | -113.0 | dBm | -107.0 | dBm |
| Minimum Eb/No(S) | 11 | dB | 11 | dB |
| Implementation Loss(I) | 3 | dB | 3 | dB |
| Link Margin(M=Pr-Pn-S-I) | 39.3 | dB | 33.3 | dB |
| Proposed Min. Rx Sensitivity Level | -99.0 | dBm | -93.0 | dBm |

## Sensitivity

|  | Rx Sensitivity level (250kbps) | Rx Sensitivity level (1000kbps) |
| --- | --- | --- |
| AWGN | -99dBm | -93dBm |
| CM1 | -93dBm | -87dBm |
| CM2 | -93.5dBm | -87.5dBm |
| CM3 | -92.5dBm | -86.5dBm |

## Power Management Mode

- Low-power Mode with Selection Wake-up

  - The proposed PHY has differentially bi-orthogonal detection and correlatively independent chirp-pulse waveform for multiple piconet

=> Superior SOP (Simultaneously Operating Piconets) performance

=> Low-power is achieved by advanced wake-up that the only desired piconets or group of nodes are called and the other piconets or nodes can estimate wake-up time from sleep state

=> Reducing Duty-Cycle and Extending Battery-life

  - This is compliant to “power consumption considerations” of 802.15.4 standard, and the mode operation for advanced wake-up may be added to this standard

## Power Consumption

|  | Gate Equiv. | Die Area (mm2) | Power Consumption |
| --- | --- | --- | --- |
| RF Section | - | 5.6 | 60 mW @ Tx 10mW |
| Baseband @ Tx | 3k | 0.04 | 0.65 mW |
| Baseband @ Rx | 51k | 0.63 | 61.6 mW |
| Deep Sleep |  |  | 12 uW |

## Antenna Practicality

- Antenna Size

- less than SD-Memory size

- Frequency / Impulse Response

- Radiation Characteristics

- Isotropic: 0dBi