IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Physical layer submission to Task Group 3a |  |
| Date Submitted | [27 March 2005] |  |
| Source | [Gadi Shor] [Wisair] [24 Raoul Wallenberg st., Tel-Aviv ISRAEL] | Voice: [ +972 3 767 6605 ] Fax: [ +972 3 647 7608 ] E-mail: [gadi.shor@wisair.com ] |
| Re: | 802.15.4a base line |  |
| Abstract | Frequency plan and PRF option for task group 4a |  |
| Purpose | Suggested text for base line |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Suggested text for Task Group 4a base line

# Frequency Plan

The frequency range 3.1-10.6 GHz is divided into 14 sub-bands, 528 MHz apart and 500 MHz each. Every 3 sub-bands can be used as either three channels or as a single channel. There are 4 possible groups (the last group consists of 5 sub-bands). Sub-band 2 is a mandatory sub-band:

| GROUP_ID | BAND_ID | Lower frequency [MHz] | Center frequency [MHz] | Upper frequency [MHz] |
| --- | --- | --- | --- | --- |
| 1 | 1 | 3182 | 3432 | 3682 |
|  | 2 | 3710 | 3960 | 4210 |
|  | 3 | 4238 | 4488 | 4738 |
| 2 | 4 | 4766 | 5016 | 5266 |
|  | 5 | 5294 | 5544 | 5794 |
|  | 6 | 5822 | 6072 | 6322 |
| 3 | 7 | 6350 | 6600 | 6850 |
|  | 8 | 6878 | 7128 | 7378 |
|  | 9 | 7406 | 7656 | 7906 |
| 4 | 10 | 7934 | 8184 | 8434 |
|  | 11 | 8462 | 8712 | 8962 |
|  | 12 | 8990 | 9240 | 9490 |
|  | 13 | 9518 | 9768 | 10018 |
|  | 14 | 10046 | 10296 | 10546 |

# Pulse Repetition Frequency (PRF)

The PRF represents the rate of transmission of the basic pattern. The PRF is an integer division of all center frequencies. The basic PRF is 66 MHz. A PRF of 33 MHz is also supported for environments with long delay spread.

# Justification for frequency plan and PRF

This section is not part of the suggested text and is aimed to explain the previous two sections. Part of the text can be included as a remark if the group will find it beneficial to the reader.

The frequencies of group 1 are expected to be the first frequencies to be used by the industry. The lower limit is 3.1 GHz due to regulatory issues and the upper limit is 4.9 GHz due to the 802.11j systems. A suitable frequency (between the arithmetic and geometric mean) which can be derived by many off the shelf XOs is 3960 MHz.

The difference between the center frequencies is selected to evenly fill the 3.1 GHz to 4.1 GHz with suitable distance from the edges.

The center frequencies can be generated using a single PLL by dividing the center frequencies down to 1 MHz or 1.2 MHz and locking on a reference frequency derived from off the shelf XOs (e.g. 13 MHz, 19.2 MHz, 24 MHz and many others)

The PRF can be generated from the PLL by dividing the center frequency down to the PRF. The first divisions can be based on the divisions implemented as part of the PLL. The center frequencies and PRF are selected such that the first two divisions for off the shelf XOs are divisions by 2. This simplifies the implementation of the high frequency division elements.