| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Prioritized List of Band Plan Criteria for TG4a |  |
| Date Submitted | [6 June 2005] |  |
| Source | [Saeid Safavi] [Wideband Access Inc.] [10225 Barnes Canyon Road, Suite A209, San Diego, CA] | Voice: [+01 858-642-9114] Fax: [+01 858-642-2037] E-mail:[ssafavi@widebandaccess.com] |
| Re: | 802.15.4a Band Plan |  |
| Abstract | List of Sorted Criteria for Band Plan of TG4a |  |
| Purpose | To address the important criteria used in selection of a band plan for TG4a |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

| Order of Importance | Criterion |
| --- | --- |
| 1 | Margin from 3.1 GHZ (FCC Compliance), the larger, the better → to address out-of-band interference |
| 2 | Margin from 4.9 GHZ (Co-existence with WLAN), the larger, the better → to address out-of-band interference |
| 3 | Integer relationship between each of the 3 Center Frequencies and Channel Separation (Chip Rate), Pulse Burst Rates and PRF’s → to address ease of implementation/lower cost |
| 4 | The PRF’s, Burst Rates ,Channel Separation (Chip Rate), and Center Frequencies should be generated by a VCO using a Single PLL → to address ease of implementation/lower cost |
| 5 | SOP Support; - Via Channel Separation - Via Code Division → to address larger number of SOP’s |
| 6 | Support of Xtal frequencies (12, 13, 9.6 and their multiples) → to address ease of implementation/lower cost |
| 7 | Primary division(s) in PLL implementation is 2, the more initial divide by 2 divisions, the better → to address ease of implementation/lower cost |
| 8 | Each sub-band’s bandwidth does not have to be related to the separation between the center frequencies (channel separation) and is defined by pulse shape → to address ease of implementation/Definition of UWB signal based |
| 9 | Above 6 GHz band plan → to address future implementations |
| 10 | Coexistence with WiMedia and UWBforum through Center Frequencies and other means → to address coexistence |
| 11 | Coexistence with other Standards (IEEE or Non-IEEE) → to address coexistence |
| 12 | Same pass-band pulse shape (needs to be further clarified!!) → to address pulse generation simplicity |