| Project | IEEE 15.4a |  |
| --- | --- | --- |
| Title | Draft for CSS PHY |  |
| Date Submitted | [Juli 18, 2005] |  |
| Source | [Rainer Hach] [Nanotron Technologies GmbH Alt-Moabit 61, 10555 Berlin, Germany] [Kyung-Kuk Lee ] [Orthotron Co., Ltd. 709 Kranz Techno, 5442-1 Sangdaewon-dong, Jungwon-gu, Sungnam-si, Kyungki-do, Korea 462-120] | Voice : [+49 30 399 954 0] Fax : [+49 30 399 954 288] E-mail : [r.hach@nanotron.com] Voice : [+82 31 777 8198] Fax : [+82 31 777 8199] E-mail : [kyunglee@orthotron.com] |
| Re: | [Preliminary CSS PHY draft for further editing ] |  |
| Abstract | [Definition of the CSS PHY] |  |
| Purpose | [Document to be discussed and edited] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Draft for CSS PHY

## General Description

This PHY uses Chirp Spread Spectrum (CSS) techniques in combination with differential phase shift keying (DPSK). Furthermore this CSS PHY utilizes Code Devision as well as frequency devision.

## Frequencies

A total of 14 frequency channels, numbered 1 to 14, are available across the 2.4 GHz band. Different subsets of these frequency channels are available in different regions of the world. In North America and Europe 3 frequency channels can be selected such that three nonoverlapping frequency bands are used.

| Frequency channel number | Frequency [GHz] |
| --- | --- |
| 1 | 2.412 |
| 2 | 2.417 |
| 3 | 2.422 |
| 4 | 2.427 |
| 5 | 2.432 |
| 6 | 2.437 |
| 7 | 2.442 |
| 8 | 2.447 |
| 9 | 2.452 |
| 10 | 2.457 |
| 11 | 2.462 |
| 12 | 2.467 |
| 13 | 2.472 |
| 14 | 2-482 |

## Waveforms and Code sequences

Four individual chirp signals, called sub-chirps in the following, are concatenated to form a full chirp symbol occupying two adjacent frequency sub-bands. Four different sub-chirp sequences are defined. Each sub-chirp is weighted with a raised cosine window in the time domain

### Graphical presentation of sub-chirp sequences

### 4 possible sequences of sub-chirps

### Mathematical presentation of sub-chirp sequences

sm(t)=n=0∞sm(t,n)n=0∞k=03cn,kexpjωk,m+μ2ξk,mt−Tn,k,mt−Tn,k,m⋅PRCt−Tn,k,m

ωk,m=2π fk,n

Tn,k,m=k+12 Tsub+nTchirp−1−−1nτm

### The Raised Cosine Window

PRC(t)=1,|t|< 1−α1+αTsub2121+cos1+ααTsubπ|t|−1−α1+αTsub2,1−α1+αTsub2 < |t|<Tsub20,|t|>Tsub2

### Numerical Parameters

| Sub-band center frequencies, fk,m [Mhz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| m\k | 1 | 2 | 3 | 4 |
| 1 | fc-3.15 | fc+3.15 | fc+3.15 | fc-3.15 |
| 2 | fc+3.15 | fc-3.15 | fc-3.15 | fc+3.15 |
| 3 | fc-3.15 | fc+3.15 | fc+3.15 | fc-3.15 |
| 4 | fc+3.15 | fc-3.15 | fc-3.15 | fc+3.15 |

| Sub-band sub-chirp directions, ξk,m |  |  |  |  |
| --- | --- | --- | --- | --- |
| m\k | 1 | 2 | 3 | 4 |
| 1 | +1 | +1 | -1 | -1 |
| 2 | +1 | -1 | +1 | -1 |
| 3 | -1 | -1 | +1 | +1 |
| 4 | -1 | +1 | -1 | +1 |

| Timing values (subject to fine adjustment) |  |
| --- | --- |
| Tchirp | 4.8 |
| Tsub | 1.2 |
| τ1 | 450 ns |
| τ2 | 300 ns |
| τ3 | 150 ns |
| τ4 | 0 ns |

| Other Parameters |  |
| --- | --- |
| α | 0.25 |

### Transmit power spectral density (PSD) mask

## DBPSK and DQPSK

| QPSK Symbol Mapping |  |
| --- | --- |
| Input bits (b0,b1) | Phase Value |
| 0,0 | π |
| 0,1 | 1/2 π |
| 1,0 | 3/2 π |
| 1,1 | 0 |

### Reference Modulator for 1 MBit/s

| BPSK Symbol Mapping |  |
| --- | --- |
| Input bit | Phase Value |
| 0 | Π |
| 1 | 0 |

### Reference Modulator for 250 kBit/s

## Coding and data rates

The mandatory data rate of 1Mbit/s is achived by using QPSK and  biorthogonal ¾ block coding

| Biorthogonal ¾ Code |  |  |
| --- | --- | --- |
| Code word number | Information Bits | Code Bits |
| 0 | 0,0,0 | 1, 1, 1, 1 |
| 1 | 0,0,1 | 1,-1, 1,-1 |
| 2 | 0,1,0 | 1, 1,-1,-1 |
| 3 | 0,1,1 | 1,-1,-1, 1 |
| 4 | 1,0,0 | -1,-1,-1,-1 |
| 5 | 1,0,1 | 1,-1, 1,-1 |
| 6 | 1,1,0 | -1, 1,-1, 1 |
| 7 | 1,1,1 | -1, 1, 1,-1 |

The optional data rate of 250kBit/s is achieved by using BPSK and a performant ½ code.

## PHY constants and PIB atributes

PHY constants

| Constant | Description | Value |
| --- | --- | --- |
| aMaxPHYPacketSize | The maximum PSDU size (in octets) the PHY shall be able to receive. | 255 |

phyPage

| Bit 0..13 | Bit 14 ..17 | Bit 18 | Bit 27..31 |
| --- | --- | --- | --- |
| Frequency band | Sub chirp sequence | Data rate | PhyPage |

## PPDU

| Chirp Symbols: 8 | 4 | 8 | variable |
| --- | --- | --- | --- |
| Preamble | SFD |  | PSDU |
| SHR |  | PHR | PHY payload |

The preamble consists of 8 unmodulated (phase zero) chirp symbol