IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3c Selection Criteria (draft) |  |
| Date Submitted | [4 October, 2005] |  |
| Source | [Alireza Seyedi] [Philips] [345 Scarborough Rd, Briarcliff Manor, NY, 10510] | Voice: [+1-914-945-6318] Fax: [+1-914-945-6330] E-mail: [alireza.seyedi@ieee.org] |
| Re: | [This document is the draft of TG3c Selection Criteria Document. Text has been used from the TG3a selection criteria (03-031r11), with modifications to fit the TG3c purposes.] |  |
| Abstract | [This document is the draft of TG3c Selection Criteria Document.] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG P802.15.3c.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1. Introduction	5

2. References	5

3. General Definitions	5

3.1. RF power measurements/caluculations	5

3.2. Eb/N0 reference point [Text drafted by A. Seyedi; to be discussed]	6

4. General Solution Criteria	6

4.1. Unit Manufacturing Cost/Complexity (UMC)	6

4.1.1. Definition	6

4.1.2. Values	7

4.2. Signal Robustness	7

4.2.1. General Definitions	7

4.2.2. Interference and Susceptibility	7

4.2.3. Coexistence	10

4.3. Technical Feasibility	13

4.3.1. Manufacturability	13

4.3.2. Time to Market	13

4.3.3. Regulatory Impact	13

4.4. Scalability	14

4.4.1. Definition	14

4.4.2. Values	14

5. MAC Protocol Supplements	15

5.1. Alternate PHY Required MAC Enhancements and Modifications	15

5.1.1. Definition	15

5.1.2. Values	15

6. PHY Layer Criteria	16

6.1. Size and Form Factor	16

6.1.1. Definition	16

6.1.2. Values	16

6.2. PHY-SAP Payload Bit Rate and Data Throughput	16

6.2.1. Payload Bit Rates	16

6.2.2. Packet Overhead	16

6.2.3. PHY-SAP Throughput	17

6.3. Simultaneously Operating Piconets	18

6.3.1. Definition	18

6.3.2. Values	18

6.4. Signal Acquisition	21

6.4.1. Definition	21

6.4.2. Values	21

6.5. System Performance	21

6.5.1. Definition	21

6.5.2. Values	21

6.6. Link Budget	22

6.6.1. Definition	22

6.6.2. Values	22

6.7. Sensitivity	24

6.7.1. Definition	24

6.7.2. Values	24

6.8. Power Management Modes	24

6.8.1. Definition	24

6.8.2. Values	24

6.9. Power Consumption	24

6.9.1. Definition	24

6.9.2. Value	25

6.10. Antenna Practicality	25

6.10.1. Definition	25

6.10.2. Value	26

Annex A:  Criteria Self-Evaluation Method and Matrix	27

A.1. General Solution Criteria	28

A.2. PHY Protocol Criteria	29

A.3. MAC Protocol Enhancement Criteria	29

Notice

The performance results shall include implementation losses as indicated in the link budget table

# 1. Introduction

This is the criteria for the selection of the alternate PHY Draft Proposals. In order to accurately and consistently judge the submitted proposals, technical requirements are needed that reflect the application scenarios that were contributed in response to the call for applications and projected applications.

This working document will become the repository for the requirements to be used in the selection process for a PHY Draft Standard for P802.15.3c. The criteria presented in this document are based on TG3c System Requirements document [05-0353-05], which takes precedence, and may also contain more general marketing requirements on which the proposers are asked to comment.

The document is divided into four sections: General Solution Criteria, MAC Protocol Supplements Criteria, PHY Layer Criteria, and Annex A. Annex A includes a self evaluation, expected with each proposal, and the evaluation process. Document [yy-nnnn-rr] provides the down selection process.

This document and the TG3c System Requirements document [05-0353-05] provide the technical content for the project to develop an alternate physical layer (alt-PHY). This alt-PHY shall be a supplement to the IEEE 802.15.3-2003 Standard. This Selection Criteria document references the IEEE 802.15.3-2003 Standard.

In this document, as per [05-0353-05], the reader will see reference to 1Gb/s mandatory data rate.  The associated distance for these data rates are, respectively, 10 meters, 4 meters and a distance given by the presenter.  The mentioned data rate is a minimum and data rates in the actual proposals may be higher than the minimum.

It is recognized by the committee that the effort required to respond to all of the selection criteria is substantial. To help proposers prioritize their efforts, simulation results for the mandatory minimum rate (>=1Gbps) are expected from the proposers during the first round of presentations. Results for the remainder of the proposal can be provided in subsequent presentations by proposers if desired.

# 2. References

[15.3]		IEEE 802.15.3-2003 Standard

[05-0353-05]	IEEE P802.15-05-0353-05, TG3c System Requirements

[yy-nnnn-rr]	IEEE P802.15-yy-nnnn-rr, TG3c Down Selection Process

[yy-nnnn-rr]	IEEE P802.15-yy-nnnn-rr, Channel Modeling Sub-committee Report

# 3. General Definitions

## 3.1. RF power measurements/caluculations

Unless otherwise stated, all RF power measurements/calculations for the purposes of this document, either transmit or receive, shall be made at the appropriate transceiver to antenna connector. For the systems without an antenna connector, the measurements/calculations shall be interpreted as EIRP (i.e. a 0dBi gain antenna) and any radiated measurements/calculations shall be corrected to compensate for the antenna gain in the system.

## 3.2. Eb/N0 reference point [Text drafted by A. Seyedi; to be discussed]

Eb is defined as the average energy per information bit calculated at the transmitter to antenna connector. For the systems without an antenna connector, the calculations shall be interpreted as equivalent isotropic radiated energy (i.e. a 0dBi gain antenna) and any radiated measurements/calculations shall be corrected to compensate for the antenna gain in the system.

N0 should be injected at the receiver to antenna connector. For the systems without an antenna connector, the calculations shall be interpreted as equivalent isotropic radiated energy (i.e. a 0dBi gain antenna) and any radiated measurements/calculations shall be corrected to compensate for the antenna gain in the system.

For purposes of converting Eb/N0 to range (meters) an exponent of 2 shall be used (i.e. r2 propagation loss).  The related performance results shall include implementation losses as indicated in the link budget table. Oxygen absorption losses must also be taken into account based on ….

# 4. General Solution Criteria

This section defines the technical and marketing system level concerns of the proposals.

## 4.1. Unit Manufacturing Cost/Complexity (UMC)

### 4.1.1. Definition

The cost/complexity of the device must be as minimal as possible for use in the personal area space, see [05-0353-05]. Fig. 1 illustrates the logical blocks in the transceiver PHY layer. Not all blocks are required to implement a communications system. However, if the functionality is used (even optionally) in the specification, then the complexity for implementing the functionality must be included in the estimate. The order and contents of the blocks may vary.

Figure 1: Logical blocks in the transceiver PHY layer

- Encode/Decode: packet formation including headers, data interleaving, error correction and detection (FEC, CRC, etc.), bias suppression, data scrambling.

- Modulate/Demodulate: convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Transmit/Receive: transition the signal to/from the channel.

### 4.1.2. Values

Complexity estimates should be provided in terms of both analog and digital die size estimates, semiconductor processes, specified year for process technologies, gate count estimates, and major external components. Similar considerations should be made with regard to MAC enhancements. Reasonable and conservative values should be given. Relative comparisons to existing technologies are acceptable.

## 4.2. Signal Robustness

### 4.2.1. General Definitions

#### 4.2.1.1. Error Rate

The error rate is the maximum packet error rate (PER) for a specified payload size. Payload size for the PER test is 2048 bytes. The error ratio should be determined at the PHY-SAP interface, after any error correction methods required in the proposed device have been applied.

#### 4.2.1.2. Receiver Sensitivity

The receiver sensitivity is the power level of a signal in dBm present at the input of the receiver for which the error rate criteria are achieved in the AWGN environment at the minimum data rate of 1Gb/s. The proposer should include all the calculations used to determine the receiver sensitivity. The minimum required receiver sensitivity is the total power available to the inputs of the receiver, which produces PER less than 8% for 2048 byte packets.  The receiver sensitivity is calculated in clause 5.6.2.

#### 4.2.1.3. Peer-to-peer data throughput

The PHY-SAP peer-to-peer data throughput of the device is the net amount of data that is transferred from one PHY SAP to another. Throughput should be measured over at least 200 packets. The connection should already have been established and in progress. The units of the data throughput are in Gb/s. The payload length should be as described above.  The throughput should include the normal overhead associated with a packet transmission.

### 4.2.2. Interference and Susceptibility

#### 4.2.2.1. Definition

Interference susceptibility refers to the impact that other co-located intentional and unintentional radiators may have on a proposed alt-PHY. This section is mainly concerned with the interference coming from other non-802.15.3c devices.

#### 4.2.2.2. Interference Model

Since the effect of RF radiators that operate in frequency bands below 10GHz, on an IEEE 802.15.3c system is minimal, the proposers are not required to consider these interferers.

The following interferers will be considered:

- In-band generic interferers

- Out-of-band generic interferers

To reduce the simulation load on the proposers, the interference from IEEE802.16, ARIB STD-T69 and ARIB STD-T74 are addressed through the generic interferer models.

Although other wireless systems may be present, the above systems represent a broad representative set of interferers whose impact has been determined to be sufficient for the evaluation of the proposed alt-PHY solutions based upon the IEEE P802.15.3c target applications. Since this document is concerned only with evaluating the capabilities, complexities, and performance implications of proposed physical layers, it is sufficient to use generic models of the above systems in order to ease the burden on the proposers.

The following representative models are suggested.

##### 4.2.2.2.1. Generic In-band Modulated Interferer

There may be other wireless systems that may be near the 802.15.3c system that could cause in-band interference. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape. The following table specifies the relevant parameters:

| f0 | Within the system bandwidth (within and outside desired signal bandwidth) |
| --- | --- |
| 1TI | 200 MHz, 1500 MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

##### 4.2.2.2.2. Generic In-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices. An IEEE 802.16 interferer can also be modeled as a tone interferer for the purposes of this document. In order to understand how much protection the system will provide in this case of atone interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π. For evaluation, f0 should be chosen to be within the system bandwidth (within and outside desired signal bandwidth).

4.2.2.2.3. Generic Out-of-band Modulated Interferer

There may be other wireless systems that may be near the 802.15.3c system that could cause in-band interference. In order to understand how much protection the system will provide in this case of an unknown modulated interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)k=−∞∞gkv(t−kTI−τI)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, θ is a random phase of the carrier uniformly distributed in 0,2π, {gk} are the randomly modulated BPSK symbols where gk∈±1, TI is the symbol period, τI is a random delay uniformly distributed in [0,TI], and v(t) is the baseband waveform shape. The following table specifies the relevant parameters:

| f0 | Outside the system bandwidth |
| --- | --- |
| 1TI | 200 MHz, 1500MHz |
| Modulation | BPSK |
| Baseband waveform | Root Raised Cosine with a roll-off of 0.25 |

##### 4.2.2.2.4. Generic Out-of-band Tone Interferer

All systems may experience tone interference resulting from close proximity to unintentional radiators like PCs or consumer electronic devices. In order to understand how much protection the system will provide in this case of a tone interferer, the following model is proposed for evaluation.

I(t)=2PIcos(ω0t+θ)

where PI is the average received power of the interfering waveform, ω0=2πf0 is the carrier frequency of the “narrowband” waveform, and θ is a random phase of the carrier uniformly distributed in 0,2π. For evaluation, f0 should be chosen to be outside the system bandwidth.

#### 4.2.2.3. Evaluation Method and Minimum Criteria

The following subsections describe how the above models can be used for evaluating the performance impact on the proposal. Since the performance of these systems may depend on particular receiver designs, and it is not the intent to standardize certain receiver designs, the proposer should describe any special circuits that were needed to obtain these results (e.g., interference suppression algorithms, notch filters, steep roll-off filters, etc.). The proposers should also provide the front end characteristics of the receiver (third order input intercept, and input 1dB compression point, number of bits in ADC, etc) that is used in obtaining these results.

##### 4.2.2.3.1. Generic In-band Modulated Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average tolerable received interference power, PI, after the receiver has executed any interference mitigation algorithms, while still maintaining a PER less than 8% for 2048 byte packets. The proposer is to show results for a number of different center frequencies or describe how the performance changes as the center frequency changes. The proposer has to show the maximum PI−Pd for their proposed system, where Pd is the received power which is defined here as 6 dB above the receiver sensitivity level.

##### 4.2.2.3.2. Generic In-band Tone Interferer

When this interferer is present, using simulation results, analysis, or technical explanations, determine the average tolerable received interference power,PI, after it has executed any interference mitigation algorithms, while still maintaining a PER less than 8% for 2048 byte packets. The proposer is to show results for a number of different tone frequencies or describe how the performance change as the  frequency changes. The proposer has to show the maximum PI−Pd for their proposed system, where Pd is the received power which is defined here as 6 dB above the receiver sensitivity level.

##### 4.2.2.3.3. Out-of-Band Interference from Intentional or Unintentional Radiators

Proposers should report the minimum out-of-band rejection in dB provided by the proposed system.

### 4.2.3. Coexistence

#### 4.2.3.1. Definition

Coexistence, in this context, refers to the co-location of IEEE 802.15.3c devices with other, non-802.15.3c devices. The criteria described in this section focuses only on the impact the 802.15.3c devices have on other non-802.15.3c devices that may be sharing the same frequency bands. The impact of the non-802.15.3c devices on a 802.15.3c receiver is addressed in Section 3.2.2.

#### 4.2.3.2. Coexistence Model

The following victim receivers which may be co-located with 802.15.3c devices, will be considered here:

- IEEE 802.16

- ARIB STD-T69

- ARIB STD-T74

Although other wireless systems may be present, the above systems represent a broad representative set of systems whose impact has been determined to be sufficient for the evaluation of the proposed PHY solutions based upon the IEEE 802.15.SG3c target applications.

The following sections describe in more detail the reference systems that must be considered by each PHY proposal.

##### 4.2.3.2.1. IEEE 802.16

[text to be filled]

##### 4.2.3.2.2. ARIB STD-T69

This model is intended to represent an ARIB STD-T69 device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 60 GHz |
| --- | --- |
| Bandwidth | 1700 MHz |
| Modulation | Analog TV |
| Tx Power | 10 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -37.7 dBm |

[references for the given values]

##### 4.2.3.2.3. ARIB STD-T74

This model is intended to represent an ARIB STD-T74 device. The following table identifies the relevant parameters of the reference system.

| Center frequency | 60 GHz |
| --- | --- |
| Bandwidth | 200 MHz |
| Modulation | GMSK (BT=0.5) |
| Tx Power | 10 dBm |
| Rx Antenna Gain | 0 dBi |
| Rx Sensitivity | -71.4 dBm |

[references for the given values]

#### 4.2.3.3. Evaluation Method and Minimum Criteria

In order to simplify the criteria, the Interfering Average Power generated by the P802.15.3c transmitter and measured in the relevant bandwidth of the victim receiver at any frequency at which that receiver operates should be used as a parameter to evaluate the coexistence capability of the proposed PHY. This power received by a 0 dBi antenna at the victim receiver frequency should be calculated at 1 m distance separation between 802.15.3c transmitter and victim receiver.  The proposers should provide sufficient data and/or technical explanations regarding any coexistence methods or measures that are employed.

For example, the minimum receiver sensitivity for an IEEE 802.16 device is –82 dBm in the 6 Mb/s mode and –65 dBm in the 54 Mb/s mode, according to Clause 17.3.10 in IEEE P802.11a/D5.0. The impact on the victim receiver may be estimated by stating the Interfering Average Power in relation to the victim receiver’s minimum sensitivity. Furthermore, the impact on PER can be estimated from curves published in the specifications and standards for the victim receiver.

##### 4.2.3.3.1. IEEE 802.16 Victim

Minimum Criteria: The interfering average power generated by the P802.15.3c transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the 802.16 device operating in the 6 Mb/s mode, when the separation between the P802.15.3c transmitter and victim receiver is 1 m.

##### 4.2.3.3.2. ARIB STD-T69 Victim

Minimum Criteria: The interfering average power generated by the P802.15.3c transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the STD-T69 device, when the separation between the P802.15.3c transmitter and victim receiver is 1 m.

##### 4.2.3.3.3. ARIB STD-T74 Victim

Minimum Criteria: The interfering average power generated by the P802.15.3c transmitter and measured in the relevant bandwidth of the victim receiver should be at least 6 dB below the minimum sensitivity level of the STD-T74 device, when the separation between the P802.15.3c transmitter and victim receiver is 1 m.

## 4.3. Technical Feasibility

This is intended to determine if the proposal is real or academic. Proposers will be asked to comment on criteria listed in the following sections.

### 4.3.1. Manufacturability

#### 4.3.1.1. Definition

Manufacturability is defined in terms of the use of available, cost effective manufacturing processes with evidence of effective mass production capability, with respect to the time line of the standard.

#### 4.3.1.2. Values

The proposers are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations.

### 4.3.2. Time to Market

#### 4.3.2.1. Definition

Time to Market addresses the question of when the proposed technology will be ready for market.

#### 4.3.2.1. Values

The proposal shall include an estimate of a schedule for when the PHY would be available for market.

### 4.3.3. Regulatory Impact

#### 4.3.3.1. Definition

The proposal should specify to which geopolitical regions it applies and identify any applicable requirements with which it conflicts. Merit will be awarded for proposals with regulatory compliance of wider geopolitical scope.

#### 4.3.3.2. Values

The proposer shall state which regions the proposal is in regulatory compliance. Merit is awarded for each region of compliance.

Regulatory domains to be considered:

- Australian Regulations

- Canadian Regulations

- Japanese regulations

- US FCC regulations

- Other National Regulations

For details on specific regulations in different regions refer to document [yy-nnnn-rr]

Specific conflicts and potential derogations should be detailed.

## 4.4. Scalability

### 4.4.1. Definition

Scalability refers to the ability to adjust important parameters, such as those mentioned below, (if they are required by the applications) without rewriting the standard. The MAC should be able to support the scaling of the PHY.

### 4.4.2. Values

Scalability parameters include; power consumption, payload bit rate and data throughput both measured at the PHY-SAP, channelization (physical or coding), complexity, range, frequencies of operation, occupied bandwidth of operation, and other functions deemed appropriate.

# 5. MAC Protocol Supplements

## 5.1. Alternate PHY Required MAC Enhancements and Modifications

### 5.1.1. Definition

Supplements and modifications to the MAC may be required to accommodate the alternate PHY. It is preferred that the supplements be additions which expand the solution capability as opposed to changes in the MAC that represent an alternative way to do a particular function.

### 5.1.2. Values

Proposals should justify and explain the supplements that may be necessary in support of additional features for the alternate PHY.

Proposals should justify and explain the modifications that may be necessary to support or enhance operation of the alternate PHY.

# 6. PHY Layer Criteria

## 6.1. Size and Form Factor

### 6.1.1. Definition

Size is important for consumer electronic systems.  The smaller the package, the easier it is to embed. Antennas are not considered in the size requirements. Antenna size is considered separately from the size of the PHY and the MAC.

### 6.1.2. Values

Proposers shall report whether their proposed solution can be fully integrated. The proposers should also report whether their proposed PHY and the 802.15.3 MAC complies with form factors such as Mini PCI, USB dongle, etc.. The proposers shall also provide the size of the antenna used for performance evaluation.

## 6.2. PHY-SAP Payload Bit Rate and Data Throughput

### 6.2.1. Payload Bit Rates

#### 6.2.1.1. Definition

The payload bit rate is defined as the bit rate at which the frame body, FCS and any stuffing bits and tail symbols are transmitted. For IEEE Std 802.15.3-2003, examples of optional payload bit rates at the PHY-SAP are 11, 33, 44, 55 Mb/s and the mandatory payload bit rate is 22 Mb/s.

#### 6.2.1.2. Values

The proposer should provide the payload bit rates provided by their proposal, including those required to meet the mandatory payload bit rates for the PHY-SAP as defined in clause 3 of [05-0353-05].

### 6.2.2. Packet Overhead

#### 6.2.2.1. Definition

For each of the proposed rates the proposer should provide all of the following packet overhead times (illustrated in Figure 2. Note: If the packet structure in a proposal is different from that displayed in Figure 2, the proposer should provide sufficient information for throuput calculations.):

T_PA_INITIAL: Length of the initial (long) preamble

T_PHYHDR: Length of the PHY header

T_MIFS: Length of the Minimum Inter Frame Space (MIFS)

T_PA_CONT: Length of the short preamble

where T_PA_CONT is the time for a potentially shortened preamble to be used for subsequent data packets in a single CTA (channel time allocation). The proposer should also provide times proposed for the other interframe spacings, SIFS, RIFS, HCS, and BIFS, as defined in the IEEE Std 802.15.3-2003 standard.

In addition, the proposer should indicate the MPDU times, T_DATA, for a 2048-octet data packet (MPDU data bits and FCS combined) and T_MACHDR. In support of these numbers, the proposer should provide the equations and values used to derive these times. The number of octets for the FCS is specified in the MAC clauses of the IEEE Std 802.15.3-2003 standard while the HCS is part of the Alt-PHY. The proposer should provide the descriptions of the proposed PHY Header and MAC Header (if different from the proposed 802.15.3 standard) and description and specification of the functional parts of the PHY preamble. The overhead for interleavers and FEC should be included.  Note: the HCS is a CRC that protects both the MAC and the PHY header.

Figure 2. Packet overhead parameters for data throughput comparison

#### 6.2.2.2. Values

Time values should be stated in nanoseconds.

### 6.2.3. PHY-SAP Throughput

#### 6.2.3.1. Definition

The PHY-SAP data throughput (in the NO-ACK mode) is defined as the bit rate at which a series of 5 MPDUs are transferred from the MAC to the PHY across the PHY-SAP, in the NO-ACK mode. The data throughput rate will be lower than the payload bit rate due to packet overhead as defined in 5.2.2.1. The payload throughput, Payload_Throughput_PHY_SAP, is given by:

Payload_Throughput_PHY_SAP = n Payload_bits / [T_PA_INITIAL+T_SIFS+(n-1)  (T_PA_CONT+T_MIFS) + n  (T_DATA+T_MACHDR + T_PHYHDR+T_HCS+T_FCS)]

The PHY-SAP data throughput (in the Imm-ACK mode) is defined as the bit rate at which a series of 5 MPDUs are transferred from the MAC to the PHY across the PHY-SAP, in the Imm-ACK mode. The data throughput rate will be lower than the payload bit rate due to packet overhead as defined in 5.2.2.1. The payload throughput, Payload_Throughput_PHY_SAP, is   given by:

Payload_Throughput_PHY_SAP = Payload_bits / [2*T_SIFS+2*T_PA_INITIAL+  T_DATA+2*T_MACHDR + 2*T_PHYHDR+2*T_HCS+2*T_FCS]

In this calculation it has been assumed that only long preambles are used.

Depending on the specific packet structure in each proposals, the proposers may report equivalent values for the throughput in the NO-ACK and Imm-ACK modes.

#### 6.2.3.2. Values

The proposed data throughput rates should be specified in Mb/s for both single frame and the multiframe transmission, both for the NO-ACK and the Imm-ACK modes.

## 6.3. Simultaneously Operating Piconets

### 6.3.1. Definition

The proposed PHY should operate in the close proximity of multiple uncoordinated piconets, at specific bit and error rates.

### 6.3.2. Values

The proposal should evaluate the effect of simultaneously operating piconets for the following specified parameters:

- Packet length of 2048 octet frame body

- PHY-SAP bit rate (>1Gbps)

- Random initial symbol alignment between reference link and interferers

- Meet the baseline performance as indicated in clause 6.5.

- A minimum of 200 packets should be used in estimating the packet error rate.

- The proposer should indicate the values of dint that cause the PER to degrade to a specific level. At a minimum, a free space channel is to be used for all links. It is desired that the environments specified in document [yy-nnnn-rr] also be used for the interfering link. The acquisition time should also be stated for all tests.

Evaluation geometry and procedure: [Text drafted by A. Seyedi; to be discussed]

The proposers should evaluate the performance of their proposal in presence of other uncoordinated piconets for two different geometries:

- Test geometry A: The victim receiver, the desired transmitter and the interfering transmitter are located along a straight line. All transmitters and receivers have equal antenna gain equal to xdBi. The directional antennas of the victim receiver and both the interfering and desired transmitters are perfectly aimed at each other (Figure 3). It is assumed that the desired and the interfering transmitters have equal transmit powers. It is assumed that the two transmitters do not shadow each other.

- Test geometry B: The victim receiver, the desired transmitter and the interfering transmitter are located along a straight line. All transmitters and receivers have equal antenna gain equal to xdBi. The directional antennas of the victim receiver and the desired transmitter are perfectly aimed at each other. However, the antenna of the interfering transmitter is aimed away from the victim receiver. It is assumed that a side lobe of the antenna of the interfering transmitter is perfectly aimed at the victim receiver and has a gain of ydBi. (Figure 4). It is assumed that the two transmitters do not shadow each other.

Figure 3. Test geometry A for simultaneously operating piconets

Figure 4. Test geometry B for simultaneously operating piconets

For each test geometry, there are two cases to be considered:

- Co-channel interferer, occupying the same channel and

- Cross-channel interferer, occupying other channels.

If the interfering PHY would have a different impact on the receiver at different supported data rates, the PHY proposer should quantify this.

An “interfering transmitter” is an uncoordinated transmitter operating at the same power as the reference transmitter. “Multiple piconet interference separation distance” is defined as the threshold distance separation (dint) of multiple interfering transmitters on different adjacent channels equidistant from the test receiver such that the test receiver PER degrades to a specified error rate.

Multiple Piconet interference separation distance test procedure

1. 	Establish a test link with a test receiver at a fixed distance from the reference transmitter, such that the distance from the transmitter to the receiver is 0.707 of the 90% link success probability distance.  Continue by sending packets to the test receiver for a specified modulation format and data rate.  For the N=1 case, the proposer is to use the first 5 channels from each required channel model for the reference and the next 5 channels (6 through 10) from each required channel model for the interferer.  The energy of each realization is normalized to unity.  For the N=2 and N=3 the first interfere shall use channels 6 through 10, the second interfere shall use 99, and the third interferer shall use 100.  All distance computations are made with a path loss exponent of 2.

2. 	Verify PER at the test receiver.

3.   Begin transmitting continuous packets on the reference link with N different adjacent channel interfering alt-PHY transmitters at a distance from the test receiver.  The synchronization between piconets must be random for each transmitted packet on the reference link.  At a minimum, the proposer should consider the cases N equal 1, 2, and 3.  As indicated in step 1, for the N=1 case the proposer is to use the first 5 channels from each required channel model for the reference and the next 5 channels (6 through 10) from each required channel model for the interferer.  The energy of each realization is normalized to unity.  For the N=2 and N=3 the first interfere shall use channels 6 through 10, the second interfere shall use 99, and the third interferer shall use 100.  The interferers shall all be selected from the same channel environment as the reference link and shall be normalized to unit power.  The reference link is to use the previously mentioned first 5 normalized channels.

4. 	Continue PER verification at the test receiver.

5. 	Incrementally move the N different adjacent channel interfering alt-PHY transmitters closer to the test receiver until the test link PER exceeds 8%.  At each incremental distance, the link must transfer at least 100 packets to include a cold acquisition on each packed.

6. 	Record the distance associated with the last acceptable PER as the multi-channel separation distance (dint) for the selected test receiver.

7. 	Since the proposal includes multiple data rates (110, 200 and optional 480 Mb/s) and may include multiple modulation types or other factors that may affect close proximity operation of uncoordinated piconets, the proposer should repeat the test procedures and include sufficient test combinations to characterize system operation under these conditions.

## 6.4. Signal Acquisition

### 6.4.1. Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### 6.4.2. Values

The proposer should provide the false alarm probability and the miss detect probability for the proposed preamble design in both AWGN and the environment specified by the channel model in document [yy-nnnn-rr].  The proposer should consider both the single piconet environment. The proposer should indicate a time-line showing the overall acquisition process, according to the preamble resources devoted to acquisition as specified in this document, at the payload bit rates specified in document [05-0353-05] Clause 3 subject to the channel model provisions in [05-0353-05] Clause 7. Additional information concerning how well the acquisition process scales with payload bit rate would be beneficial.

## 6.5. System Performance

### 6.5.1. Definition

System performance refers to the ability of the system to successfully acquire and demodulate data packets at the required data rates and bit and packet error rates, both in the free space AWGN channel and in the multipath channels specified by the channel model document [yy-nnnn-rr].

Performance of the proposed system should be simulated and reported in various multipath environments defined in the latest revision of document [yy-nnnn-rr]. The multipath performance can be used to evaluate the losses incurred due to multipath.

The proposer will be asked for 90% PER link success probability where a 90% PER link success is defined as the PER averaged over the channels which result in the 90% best performance at a given Eb/N0 for a particular channel environment, i.e., the PER performance due to the worst 10% channels at a given Eb/N0 should not be included in the average PER calculation.

### 6.5.2. Values

The proposer should provide a 90% link success probability distance and the mean 90% link success probability distance, for each payload bit rate over each of the 4 channel models from 020/490 and in an AWGN environment. The 90% link success probability distance is defined as the distance at which the PER, for all of the best 90% of channels for a given channel environment, is less than or equal to 8%. The mean 90% link success probability distance is defined as the distance at which the mean PER of the best 90% channels for a given channel environment, is less than or equal to 8%.  The performance results shall include implementation losses as indicated in the link budget table

## 6.6. Link Budget

### 6.6.1. Definition

Link budget is used to determine proposal capabilities under certain operating conditions for the standards specified bit rates, ranges, and bit error rate.

### 6.6.2. Values

The table below, see [02/490], identifies the necessary parameters and equations that should be used to compute the final link margin. Proposers should complete this link budget table and identify and explain all assumptions. Although the proposers may need to make minor alterations to this table to more adequately reflect their proposal, the table identifies the minimum expected level of thoroughness, detail, and justification.

Source: [02/490]

| Parameter | Value | Value | Value |
| --- | --- | --- | --- |
| Throughput (Rb) | > 110 Mb/s | > 200 Mb/s | > 480 Mb/s (optional) |
| Average Tx power (PT) | dBm | DBm | dBm |
| Tx antenna gain (GT) | 0 dBi | 0 dBi | 0 dBi |
| fc'=fminfmax: geometric center frequency of waveform (fmin and fmax are the -10 dB edges of the waveform spectrum) | Hz | Hz | Hz |
| Path loss at 1 meter (L1=20log10(4πfc'c)) c=3×108 m/s | dB | DB | dB |
| Path loss at d m (L2=20log10(d)) | 20 dB at d=10 meters | 12 dB at d=4 meters | presenter specified |
| Rx antenna gain (GR) | 0 dBi | 0 dBi | 0 dBi |
| Rx power (PR=PT+GT+GR−L1−L2 (dB)) | dBm | DBm | dBm |
| Average noise power per bit (N=−174+10*log10(Rb)) | dBm | DBm | dBm |
| Rx Noise Figure Referred to the Antenna Terminal (NF)1 | dB | DB | dB |
| Average noise power per bit (PN=N+NF) | dBm | DBm | dBm |
| Minimum Eb/N0 (S) | dB | DB | dB |
| Implementation Loss2 (I) | dB | DB | dB |
| Link Margin (M=PR−PN−S−I) | dB | DB | dB |
| Proposed Min. Rx Sensitivity Level3 | dBm | DBm | dBm |

1 Per text book definition, the NF is the ratio of the SNR at the antenna output with respect to the SNR at the demodulator input (reference figure 1).  The NF should include not only the LNA but also cascaded stages as per Friis’ equation.  Each proposer should justify the proposed noise figure number, or else use a default value of 11 dB.

2 Implementation loss is defined here for the AWGN channel only, and could include such impairments as filter distortion, phase noise, frequency errors, etc.

3 The minimum Rx sensitivity level is defined as the minimum required average Rx power for a received symbol in AWGN, and should include effects of code rate and modulation.

## 6.7. Sensitivity

### 6.7.1. Definition

Sensitivity is defined in . It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### 6.7.2. Values

The proposal should indicate the power level at which the error criterion is met, consistent with the link budget as presented in document [02/490], Table 1. The proposal should also indicate the PER used in the determination of this value.

## 6.8. Power Management Modes

The ability to reduce power consumption for consumer electronic devices is important.

### 6.8.1. Definition

Power management modes and protocols allow device sleep, wakeup, and poll. The proposed 802.15.3 standard provides such power management capabilities.

### 6.8.2. Values

The proposal should explain if it supports each of the power management methods as defined in the proposed 802.15.3 standard.

## 6.9. Power Consumption

### 6.9.1. Definition

Power consumption is defined as the total average power required by the proposed system to operate in transmit, receive, clear channel assessment, or power saving modes. It includes the power consumed by all components necessary to implement all of the functionality of the proposed alternate PHY from the PHY-SAP interface, defined in the proposed 802.15.3 standard, down to an antenna, where the gain is disclosed by the proposer. No components supporting operation above the PHY- SAP interface are included in the average power consumption value.

#### 6.9.1.1. Transmit

Power consumption during transmit state is defined as the average power consumed from the PHY-TX-START.request for a given MPDU, to the PHY-TX-END.confirm.

#### 6.9.1.2. Receive

Power consumption during receive state is defined as the average power consumed from the PHY-RX-START.request for a given MPDU, to the PHY-RX-END.indication where the PHY-RX-START.request is assumed to be coincident with the remote transmission beginning.

#### 6.9.1.3. Clear Channel Assessment

Power consumption during clear channel assessment (CCA) is defined as the average power consumed from the PHY-CCA-START.request to the PHY-CCA-END.confirm.

#### 6.9.1.4. Power Save

Power consumption during the power save state is defined as the power consumed from the PHY-PS.request to the PHY-PS.confirm resulting from a subsequent PHY-PS.request with a PSLevel value of 0. Methods for achieving power save modes and the impact to the operation (acquisition, time to come ‘awake”, etc…) of the PHY should be described.

### 6.9.2. Value

Power consumption values are to be disclosed with sufficient explanation of how the numbers are derived. These numbers should reflect operation at the RF power necessary to achieve the continuous full bit rate/throughput at the maximum range including the disclosed antenna gain. To help aid comparison among proposals, disclosure should include parameters such as technology process, clock rate, voltage, etc.

#### 6.9.2.1. Transmit

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### 6.9.2.2. Receive

The proposal should estimate the power consumption for the PHY throughputs specified in section 5.2 with proposed minimum and maximum PHY frame lengths.

#### 6.9.2.3. Clear Channel Assessment

The proposal should state the estimated power consumed during both channel "busy" periods and channel "idle" periods.

#### Power Save

The proposal should specify the power consumption associated with the lowest supported power consumption level (PwrMgtLevel). The proposal should also provide values for power save group parameters as specified in 802.15.3. Proposals should provide justification for its stated power save values (for example, circuits disabled, clocks turned off, etc…).

## 6.10. Antenna Practicality

### 6.10.1. Definition

The antenna form factor should be consistent with the following form factors:

- PC Card

- Compact Flash

- Memory Stick

- SD Memory

### 6.10.2. Value

Antenna form factor should be described with reference to expected size. Any additional information the proposer desires to provide on the antenna such as size, frequency response, impulse response and radiation characteristics would be beneficial.

# Annex A:  Criteria Self-Evaluation Method and Matrix

Each proposer will evaluate their own proposal, based on the selection criteria referenced in the tables below, and will include this information in their proposal presentation. In order to indicate the importance level of each of the criteria, an ABC leveling scheme is used, as was determined by the mutual consent of SG3a during the November 2002 meeting.

- A – Most Important Requirement

- B – Important Desired Requirement

- C – A “Nice to Have” Requirement

Once the proposals have been presented, each token-bearing TG3a committee member will have an opportunity to score each proposal. This information will not be collected and is intended solely to help individual committee members with their own evaluations of each of the proposals. Scoring will be achieved by rating each proposal against each of the criteria indicated below. Three scoring levels are possible:  -, 0, +.

## A.1. General Solution Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| Unit Manufacturing Complexity (UMC) | 3.1 | B |  |
| Signal Robustness |  |  |  |
| Interference And Susceptibility | 3.2.2 | A |  |
| Coexistence | 3.2.3 | A |  |
| Technical Feasibility |  |  |  |
| Manufacturability | 3.3.1 | A |  |
| Time To Market | 3.3.2 | A |  |
| Regulatory Impact | 3.3.3 | A |  |
| Scalability (i.e. Payload Bit Rate/Data Throughput, Channelization – physical or coded, Complexity, Range, Frequencies of Operation, Bandwidth of Operation, Power Consumption) | 3.4 | A |  |
| Location Awareness | 3.5 | C |  |

## A.2. PHY Protocol Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| Size And Form Factor | 5.1 | B |  |
| PHY-SAP Payload Bit Rate & Data Throughput |  |  |  |
| Payload Bit Rate | 5.2.1 | A |  |
| Packet Overhead | 5.2.2 | A |  |
| PHY-SAP Throughput | 5.2.3 | A |  |
| Simultaneously Operating Piconets | 5.3 | A |  |
| Signal Acquisition | 5.4 | A |  |
| System Performance | 5.5 | A |  |
| Link Budget | 5.6 | A |  |
| Sensitivity | 5.7 | A |  |
| Power Management Modes | 5.8 | B |  |
| Power Consumption | 5.9 | A |  |
| Antenna Practicality | 5.10 | B |  |

## A.3. MAC Protocol Enhancement Criteria

| CRITERIA | REF. | IMPORTANCE LEVEL | PROPOSER RESPONSE |
| --- | --- | --- | --- |
| MAC Enhancements And Modifications | 4.1. | C |  |

TX Bandlimited Output

+

Eb

Channel

No

RX

Input

See Section 3.2 for definition of Eb/N0.

See Section 3.2 for definition of Eb/N0.

The PER should represent the cumulative rate of packet errors of all types.  Types of packet errors include failure to detect and acquire the desired signal within the preamble time frame described in the proposal, failure to correctly demodulate the header, and failure to correctly demodulate the payload.