IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft on Ranging Service Primitives and Attributes |  |
| Date Submitted | October 10, 2005 |  |
| Source | Lars Menzer Nanotron Technologies Alt-Moabit 61 D-10555 Berlin, GERMANY | Voice: +49 30 399954 0 Fax: E-mail: l.menzer@nanotron.com |
| Re: | Draft specification for 802.15.4a |  |
| Abstract | Draft text proposing ranging amendments on service primitives and attributes in PHY and MAC. |  |
| Purpose | In support of discussions leading to the draft for 802.15.4a. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Text in black = existing text

Text in blue = ranging amendments

...

        - Semantics of the service primitive

The semantics of the PD-DATA.request primitive is as follows:

PD-DATA.request (

psduLength,

psdu,

PreambleType

)

Table 6 specifies the parameters for the PD-DATA.request primitive.

Table 6—PD-DATA.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| PreambleType | Enumeration | TYPE_1, TYPE_2, TYPE_3, .. | Optional. The preamble type of the PHY frame to be transmitted by the PHY entity. |

        - Semantics of the service primitive

The semantics of the PD-DATA.confirm primitive is as follows:

PD-DATA.confirm (

status,

Timestamp

)

Table 7 specifies the parameters for the PD-DATA.confirm primitive.

Table 7—PD-DATA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Timestamp | Integer | 0x000000-0xFFFFFF | Optional. The timestamp with high resolution (see 6.8.3.3) of the PHY frame transmitted by the PHY entity. Implementation specific. |

....

        - Semantics of the service primitive

The semantics of the PD-DATA.indication primitive is as follows:

PD-DATA.indication (

psduLength,

psdu,

ppduLinkQuality,

PreambleType,

Timestamp

)

Table 8 specifies the parameters for the PD-DATA.indication primitive.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| ppduLinkQuality | Bitmap | 0x0000000-0xFFFFFFFF | The 8 LSBs represent the link quality (LQI) value measured during reception of the PPDU (see 6.9.8). Optional. The 24 MSBs represent the figure of merit information of a ranging operation (see 6.8.3.4 ). |
| PreambleType | Enumeration | TYPE_1, TYPE_2, TYPE_3, .. | Optional. The preamble type of the PHY frame received by the PHY entity. |
| Timestamp | Integer | 0x000000-0xFFFFFF | Optional. The timestamp with high resolution (see 6.8.3.3) of the PHY frame received by the PHY entity. Implementation specific. |

....

      - PTI field

The PTI field is ? bits in length and specifies the preamble length. For improved robustness the field contains redundant information to tolerate bit errors. Table 21a summarizes the PTI field value versus the preamble length.

Table 21—PTI values

| PTI field | Preamble length |
| --- | --- |
| 0x? | 50 µs |
| 0x? | 500 µs |
| 0x? | 4000 µs |

...

Table 23—PHY PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |  |
| phyRangingSupported† | 0x05 | Boolean | TRUE or FALSE | This indicates whether the PHY sublayer supports the optional ranging features*. | FALSE |
| phyTxSyncSymbolOffset† | 0x06 | Integer | 0x000000-0xFFFFFF | Optional. The offset, measured in high resolution, between the symbol boundary at which the PLME captures the timestamp of each transmitted frame, and the onset of the first symbol past the SFD leaving the antenna. | Implementation specific |
| phyRxSyncSymbolOffset† | 0x07 | Integer | 0x000000-0xFFFFFF | Optional. The offset, measured in high resolution, between the symbol boundary at which the PLME captures the timestamp of each received frame, and the onset of the first symbol past the SFD arriving on the antenna. | Implementation specific |

*optional PHY ranging features: to be listed ...

...

        - Semantics of the service primitive

The semantics of the MCPS-DATA.request primitive is as follows:

MCPS-DATA.request (

SrcAddrMode,

SrcPANId,

SrcAddr,

DstAddrMode,

DstPANId,

DstAddr,

msduLength,

msdu,

msduHandle,

TxOptions,

SecurityLevel,

KeyIdMode,

KeyId,

ServiceType

)

Table 41 specifies the parameters for the MCPS-DATA.request primitive.

Table 41—MCPS-DATA.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| ServiceType | Enumeration | SERVICE_DATA, SERVICE_RANGING | Optional. The service type of the MSDU to be transmitted by the MAC sublayer entity. |

....

        - Semantics of the service primitive

The semantics of the MCPS-DATA.confirm primitive is as follows:

MCPS-DATA.confirm (

msduHandle,

status,

Timestamp,

TimestampAck,

mpduLinkQualityAck

)

Table 42 specifies the parameters for the MCPS-DATA.confirm primitive.

Table 42—MCPS-DATA.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| Timestamp | List of Integer | 0x000000-0xFFFFFF | Optional. List of two values. The first value represents the time in symbols, at which the data were transmitted (see 7.5.4.1). The value(s) of this parameter will only be considered valid if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. The symbol boundary is determined by macSyncSymbolOffset (see Table 86). This is a 24-bit value, and the accuracy of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. The second value determines the symbol boundary with high resolution (see 6.2.1.1.4) instead of by the macSyncSymbolOffset attribute. Implementation specific. |
| TimestampAck | List of Integer | 0x000000-0xFFFFFF | Optional. List of two values. The first value represents the time in symbols, at which the acknowledgment frame was received. The values of this parameter will only be considered valid if the value of the status parameter is SUCCESS; if the status parameter is not equal to SUCCESS, the value of the Timestamp parameter shall not be used for any other purpose. This is a 24-bit value, and the accuracy of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. The second value determines the symbol boundary with high resolution. Implementation specific. Parameter is only valid, when an acknowledged transmission was requested and the acknowledgment was received successfully. |
| mpduLinkQualityAck | Bitmap | 0x00000000-0xFFFFFFFF | Optional. The 8 LSBs represent the link quality indication (LQI) value measured during reception of the acknowledgment frame. Lower values represent lower LQI (see 6.9.8). The 24 MSBs represent the figure of merit information of a ranging operation (see 6.8.3.4 ). Parameter is only valid, when an acknowledged transmission was requested. |

....

        - Semantics of the service primitive

The semantics of the MCPS-DATA.indication primitive is as follows:

MCPS-DATA.indication (

SrcAddrMode,

SrcPANId,

SrcAddr,

DstAddrMode,

DstPANId

DstAddr,

msduLength,

msdu,

mpduLinkQuality,

SecurityLevel,

DSN,

Timestamp,

TimestampAck,

GrpAddress,

SrcAddrMatch,

status,

ServiceType

)

Table 43 specifies the parameters for the MCPS-DATA.indication primitive.

Table 43—MCPS-DATA.indication parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ... | ... | ... | ... |
| mpduLinkQuality | Bitmap | 0x00000000-0xFFFFFFFF | The 8 LSBs represent the link quality indication (LQI) value measured during reception of the acknowledgment frame. Lower values represent lower LQI (see 6.9.8). Optional. The 24 MSBs represent the figure of merit information of a ranging operation (see 6.8.3.4 ). |
| ... | ... | ... | ... |
| Timestamp | List of Integer | 0x000000-0xFFFFFF | Optional. List of two values. List of three values. The first value represents the time in symbols, at which the data were received (see 7.5.4.1). The symbol boundary is determined by macSyncSymbolOffset (see Table 86). This is a 24-bit value, and the accuracy of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. The second value determines the symbol boundary with high resolution (see 6.2.1.2.1) instead of by the macSyncSymbolOffset attribute. Implementation specific. |
| TimestampAck | List of Integer | 0x000000-0xFFFFFF | Optional. List of two values. The first value represents the time in symbols, at which the acknowledgment frame was transmitted. The value of this parameter will only be considered valid if the value of the status parameter is ACK; if the status parameter is not equal to ACK, the value of the Timestamp parameter shall not be used for any other purpose. This is a 24-bit value, and the accuracy of this value shall be a minimum of 20 bits, with the lowest 4 bits being the least significant. The second value determines the symbol boundary with high resolution. Implementation specific. |
| status | Enumeration | ACK, NO_ACK | Optional. The status of the acknowledgment. |
| ServiceType | Enumeration | SERVICE_DATA, SERVICE_RANGING | Optional. The service type of the MSDU received is indicated to the higher layer. |

        - When generated

....

If the MPDU was received successfully, and an acknowledgment, if requested, was transmitted, the MAC

sublayer will issue the MCPS-DATA.indication primitive with a status of ACK, otherwise with a status of NO_ACK.

....

Table 86—MAC PIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |  |
| macAckWaitDuration | 0x40 | List of Integer | 38, 50, 54 or 120, ? | The maximum number of symbols to wait for an acknowledgment frame to arrive following a transmitted data frame. List of two values, second is optional. These values are dependent on the supported PHY, which determines both the selected logical channel and channel page and the used preamble length for ranging. The calculated values are time to commence transmitting the ACK plus the length of the ACK frame. The commencement time is described in 7.5.6.4.2. The first list value is used for the data service, the second for the ranging service. | Dependent on supported PHY and default ranging preamble length. |
| ... | ... | ... | ... | ... | ... |
| macRangingSupported† | 0x60 | Boolean | TRUE or FALSE | This indicates whether the MAC sublayer supports the optional ranging features*. | FALSE |
| macRangingPreambleType | 0x61 | Enumeration | TYPE_2, TYPE_3,... | Optional. The preamble type used for ranging. | TYPE_2 |

*optional MAC ranging features: to be listed ...

....

Table 90— PIB security attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |  |
| macMPDUOffset | 0x7c | Integer | 0x00-0xFF | Optional. Added delay of the MPDU for privacy on ranging. Implementation specific, minimum range is 300? ns, maximum step size is 3? ns. | 0x00 |