| Project | IEEE 15.4a |  |
| --- | --- | --- |
| Title | Draft for Ultra Wide Band (UWB) PHY |  |
| Date Submitted | [Nov 7, 2005] |  |
| Source | [Philip Orlik] [Mitsubishi Electric Research Laboratories, Inc. 201 Broadway, 8th floor, Cambridge, MA, USA] [Ismail Lakkis] [novowave] 10225 Barnes Canyon Rd, Suite A209 San Diego, CA 92121] | Voice : [+1 617 621 7570] Fax : [+1 617 621 7550] E-mail : [porlik@merl.com] Voice : [+1 858 231 9753] Fax : [+1 858 641 9114] E-mail : [ilakkis@novowave.com] |
| Re: | [Preliminary UWB PHY draft for further editing ] |  |
| Abstract | [Definition of the UWB PHY] |  |
| Purpose | [Document to be discussed and edited] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

## 6.1.1a Operating Frequency Range

The UWB PHY operate in the frequency range from 3211 – 4693 MHz and optionally from 5931.9-10304.25 MHz.

## 6.1.2a Channel Assignments

The UWB PHY admits 15 frequency bands as listed in Table 1.  A compliant device shall be capable of transmitting in channel 2 with a signal whose 3dB bandwidth is 494MHz.  Transmission in all other frequency band is optional.  However, if transmission in the frequency range 5931.9-10304.25 MHz is desired then a transmitter shall be capable of transmitting in channel 8.

Table 1 UWB PHY Channel Frequencies

| Channel Number | Center frequency (MHz) | Band Width (3dB) | Mandatory/Optional |
| --- | --- | --- | --- |
| 1 | 3458 | 494 | Optional |
| 2 | 3952 | 494 | Mandatory |
| 3 | 4446 | 494 | Optional |
| 4 | 3952 | 1482 | Optional |
| 5 | 6337.5 | 507 | Optional |
| 6 | 7098 | 507 | Optional |
| 7 | 7605 | 507 | Optional |
| 8 | 8112 | 507 | Optional (Mandatory in High Band) |
| 9 | 8619 | 507 | Optional |
| 10 | 9126 | 507 | Optional |
| 11 | 9633 | 507 | Optional |
| 12 | 10140 | 507 | Optional |
| 13 | 6591 | 1318.2 | Optional |
| 14 | 8112 | 1352 | Optional |
| 15 | 8961.75 | 1342.5 | Optional |

#### 6.1.2.1b Channel numbering for UWB

#### In the frequency range 3211 – 4693 MHz the channel numbering is as follows

#### fc=3458+(k−1)∗494

#### 6.1.2.2 Channel pages

#### TBD

## 6.8a. 3100 to 10000MHz PHY Ultra Wide Band (UWB)

Add text giving general overview of UWB Phy Features

## 6.8a.1 Data Rates

The data rate of the UWB PHY shall be 0.811 Mbps.  (EDITOR’S NOTE:  Do we need to define this to be the data rate at the PHY/MAC SAP??).

Table 2 Mandatory and optional data rates

| Data Rate (MHz) | Mandatory/Optional (M/O) |
| --- | --- |
| 0.1 | O |
| 0.811 | M |
| 3.24 | O |
| 6.49 | O |
| 12.97 | O |
| 26.03 | O |

## 6.8a.2 Ranging/Acquisition Symbol (Preamble)

The preamble field is used by the transceiver to obtain chip and symbol synchronization with an incoming message and to track signal leading edge for ranging. The available options for the preamble length are shown in Table 1.

Table 1 Preamble field length

| Code Length | Preamble Index | Mandatory /Optional | Mean PRF (MHz) | Preamble Length | Duration |
| --- | --- | --- | --- | --- | --- |
| 31 | 1 | M | 15.875 | 64 symbols | 124.976 uS |
| 31 | 2 | M | 15.875 | 256 symbols | 500 uS |
| 31 | 3 | M | 15.875 | 1024 symbols | 2 mS |
| 31 | 4 | M | 3.96875 | 64 symbols | 500 uS |
| 31 | 5 | M | 3.96875 | 256 symbols | 2 mS |
| 31 | 6 | M | 3.96875 | 1024 symbols | 7.998 mS |
| 127 | 7 | O | 127.48 | 64 symbols | 32.907 uS |
| 127 | 8 | O | 127.48 | 256 symbols | 131.627 uS |
| 127 | 9 | O | 127.48 | 1024 symbols | 526.51 uS |

A code from Table 1 is used as a ranging/acquisition code Si. The code can be selected from length 31 or length 127 Ternary codes. Length 31 Ternary codes are given in Table Z1. These are the six codes with the best cross-correlation of the 12 possible codes with perfect periodic auto-correlation. Out of seventy two length-127-Ternary-codes, listed in Table Z2 are the 26 codes with perfect periodic autocorrelation and best cross-correlation properties.

Table Z1 Length 31 Ternary codes

| Index | ID | Sequence |
| --- | --- | --- |
| 1 | S1 | +0++000−+−++00++0+00−0000−0+0−− |
| 2 | S2 | +−0+0+00+000+0++−−−0−+00−++0000 |
| 3 | S3 | 000+−00−00−++++0+−+000+0−0++0−0 |
| 4 | S4 | 0+0000−00−0+−00+++−+000−+0+++0− |
| 5 | S5 | +0+−0+0+000−++0−+−−−00+00++0000 |
| 6 | S6 | 000+00−0−0++0000−−+00−+0++−++0+ |

Table Z2 Length 127 Ternary codes

| Index | ID | Sequence |
| --- | --- | --- |
| 1 | S7 | 00000-0-0+0+-0+00-00-0+00--++00-000-+0-0-0000+++++++0+-+0++0-+--0+000+0+-00-0++-+-000+000000-+00-+0-+000+--00+++0-0+--0000-00++ |
| 2 | S8 | -0++0-+00-00-000+-+0000-0++-++00+0+0+--00+-0-000-00-+-+000-0+0000--00000-000000+++-++-0+0+0-00-+00+++0--+0+00-0++000++0+++-0--0 |
| 3 | S9 | +0-000+-++-+-00-000000-0-+00000-++0-0000+00-+-000--0-00+00-0+-+0++0-++00++0+-00-0+0++0-0++++-0++--0000--000+000+0+00--00-+++0+0 |
| 4 | S10 | 0+-0++0+000+--+-0000++-000+0+00++000000++0-0--+0-00+0-0+0++0+--00+0000+000+00-00+-++0-0+00000-0-+-+00---0----+++0+-00+0-+000-+0 |
| 5 | S11 | 0+--+000-+-0---0+0-+-+0+00+0+-00+0-00+++00-000++000+0+++0000-0000-+0+00000+--+-0++000000-0+0-+0----+--00++0+-0+0-0+000+00-00++0 |
| 6 | S12 | -+000000++0-0+--0+00-0-0+0++0+++00+0000-000+00-00--++0+0+00000-0-+++00--+0-+--+--0-+00-0+-000--00++0-+0+000+-+-+0000+++000-0+00 |

The preamble consists of repetitions of the selected code Si. The ranging preamble should be transmitted in one of the forms given in Figure Z1.

Figure Z1 Illustration of synchronization part of ranging packet preamble. The Si indicates the selected code with corresponding index i. Sync / CE is the combined synchronization frame and channel estimation sequence. SFD is the synchronization frame delimiter (a) Normal preamble (b) Preamble for 128kps.

## 6.8a.3 Waveform, Pulse Shape, and Chipping Rates

The UWB PHY uses an impulse radio based signaling scheme in which each information bearing symbol is represented by a sequence/burst of short time duration (hence large bandwidth) pluses.  The duration of an individual pulse is nominally considered to be the length of a chip.  A UWB PHY compliant device shall be capable of transmitting pulses at a rate of 494 MHz.  This is equivalent to a chip duration of 2.02429 ns or a chipping rate of 494MHz.

Figure 1 Reference Modulator (After FEC encoding)

As the UWB PHY is required to support both coherent and non coherent receivers the modulation format chosen is a combination of both Pulse Position Modulation (PPM) and Binary Phase Shift Keying (BPSK).  Nominally, a UWB PHY symbol is capable of carrying two bits of information one bit is used to determine the position of a burst of pulses while an additional bit is used to modulate the phase (polarity) of this same burst.  Error: Reference source not found is provided as a reference for specifying the processing of coded symbols and their subsequent conversion to an analog waveform.  Each block is described in more detail in the following

subsections of this clause.

## 6.8a.3.1 Structure of a UWB PHY symbol

Figure 2 a depicts the structure and timing of a UWB PHY symbol assuming the mandatory data rate of 1 Mbps.  Each symbol shall consist of an integer number of chips which have duration of 2.02429 ns.  Several consecutive chips are grouped together to form a burst.  And the location of the burst in either the first half or second half of the symbol indicates one bit of information.  Additionally, the phase of the burst is used to indicate a second bit of information.  For a given symbol duration, Tsym, the number of chips each in each symbol is

Nc=Ts2.0429								1

Where  indicates a floor operation.  A burst duration,Tburst, is related to the chip duration, Tc, and Nc and by

Tburst=Nb∗Tc								2

Figure 2 UWB PHY Symbol Timing

In addition to the modulation of data the UWB PHY symbol provides for some multi-user access interference rejection in the form of time hopping.  Since each symbol contains a single burst of pulses and the burst length is typically much shorter than the duration of the symbol the location of the pulse within each burst can be varied from on a symbol to symbol basis according to a time hopping code.  This is part of the functionality provided by the “Scrambler and Burst Positon Hopping” block as depicted in Error: Reference source not found.

## 6.8a.3.2 UWB PHY Symbol Timing Details

The UWB PHY shall support two average Pulse Repitition Frequencies (PRF).  Namely 15.4375MHz and 3.859375MHz.  These PRFs in addition to the data rate, modulation and coding rate determines the overall timing of a UWB PHY symbol.  Table 3 defines the parameters of the PHY UWB symbol.

Table 3 UWB PHY Symbol Timing Parameters (Mandatory Data Rate)

| Avg. PRF (MHz) | Chip Rate (MHz) | Modulation Order (bits/Symbol) | Data Rate (Mbps) | FEC rate (outer code) | FEC rate (inner code) | Code rate | Symbol Rate (MHz) | Pulses per Burst (Nburst) | Burst Duration (ns) | # of slots (Ns) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15.4375 | 494 | 2 | 1 | 1/2 | .88 | .44 | 0.996 | 16 | 32.4 | 31 |
| 3.859375 | 494 | 2 | 1 | 1/2 | .88 | .44 | 0.996 | 4 | 8.1 | 124 |

The additional data rates shown in Table 2 require that the number of pulses per burst are modified this in turn alters the burst duration and other symbol timing parameters as specified in the following tables.

Table 4 UWB PHY Symbol Timing Parameters for Optional Data Rates and Average PRF of 15.94MHz

| Data Rate (Mbps) | Symbol Duration (us) | Pulses per Burst (Nburst) | Burst Duration (ns) | # of slots (Ns) |
| --- | --- | --- | --- | --- |
| 0.1 | 8.0324 | 128 | 259 | 31 |
| 3.24 | 0.251 | 4 | 8.1 | 31 |
| 12.97 | 0.0628 | 1 | 2.02 | 31 |
| 26.03 |  |  |  |  |

Table 5 UWB PHY Symbol Timing Parameters for Optional Data Rates and Average PRF of 3.98MHz

| Data Rate (Mbps) | Symbol Duration (us) | Pulses per Burst (Nburst) | Burst Duration (ns) | # of slots (Ns) |
| --- | --- | --- | --- | --- |
| 0.1 | 8.0324 | 32 | 64.8 | 124 |
| 3.24 | 0.251 | 1 | 2.02 | 124 |
| 12.97 | 0.0628 | 1/4 | 0.51 | 124 |
| 26.03 |  |  |  |  |

## 6.8a.3.3 UWB PHY Modulation Symbol Details

The UWB PHY symbol may be expressed using the following equation

x(k)(t)=j=1Nburstg1(k)sjpt−g0(k)TPPM−jTc−h(k)Tburst				3

In the above equation the x(k)(t)is the waveform of the kth information bearing symbol, g0, and g1 are the modulation symbols obtained from a mapping of the coded bits, sj {j = 0,1,…,Nburst -1}, is the scrambling sequence and takes the possible values {-1 or 1}, p(t) is the transmitted pulse shape at the input to the antenna, TPPM is the duration of the binary pulse position modulation time slot.  The hopping sequence h(k), provides suppression of multiuser interference and the scrambling sequence, sj, provides additional interference supporesion among coherent receivers as well as spectral smoothing of the transmitted waveform.  The tables below call out the numerical values for the various data rates and average pulse repetition frequencies.

Table 6 Numerical Parameters for Equation 3 (Mandatory data rate)

| Average PRF (MHz) | Chip Duration (Tc) ns | Postion Duration (TPPM) ns | Burst Length (Tburst) ns |
| --- | --- | --- | --- |
| 15.4375 | 2.02429 | 502 | 32.4 |
| 3.859375 | 2.02429 | 502 | 8.1 |

Table 7 Numerical Parameters for Equation 3 (Optional data rates and Average PRF =15.94 MHz )

| Data Rate (Mbps) | Chip Duration (Tc) ns | Postion Duration (TPPM) ns | Burst Length (Tburst) ns |
| --- | --- | --- | --- |
| 0.1 | 2.02429 | 4.016 | 259 |
| 3.24 | 2.02429 | 0.126 | 8.1 |
| 12.97 | 2.02429 | 0.031 | 2.02 |
| 26.03 |  |  |  |

Table 8 Parameters for Equation 3 (Optional data rates and Average PRF =3.98 MHz)

| Data Rate (Mbps) | Chip Duration (Tc) ns | Postion Duration (TPPM) ns | Burst Length (Tburst) ns |
| --- | --- | --- | --- |
| 0.1 | 2.02429 | 4.016 | 64.8 |
| 3.24 | 2.02429 | 0.126 | 2.02 |
| 6.49 | 2.02429 | 0.063 | 1.01 |
| 12.97 | 2.02429 | 0.031 | 0.51 |
| 26.03 |  |  |  |

### 6.8a.3.3.1 UWB PHY Symbol Mapping

The UWB PHY shall map groups of two consecutive bits into modulation symbols according to Table 9

Table 9 UWB PHY Bit to Modulation Symbol Mapping

| Information bits (b1b0) | Modulation Symbols (g1g0) |
| --- | --- |
| 00 | -10 |
| 01 | -11 |
| 10 | 10 |
| 11 | 11 |

### 6.8a.3.3.2 UWB PHY Pulse Shape

The pulse shape, p(t), of the UWB PHY shall be constrained by its cross correlation properties with a standard reference pulse, r(t).  The cross correlation between two waveforms is defined as

φ(τ)=1ErEpr(t)p(t+τ)dt							4

The reference, r(t), pulse used by the UWB PHY is a root raised cosine pulse with roll-off factor of  = 0.6.  Mathematically this is

r(t)=sincπtTccosπβt/Tc1−2βt/Tc2								5

In order for a UWB PHY transmitter to be compliant with the standard the transmitted pulse shall have a cross correlation coefficient that is greater or equal to 0.7,  that is

φ(0)=1ErEpr(t)p(t)dt≥0.7							6

### 6.8a.3.3.3 UWB PHY Optional Pulse Shapes

An optional pulse shape that consists of a weighted linear combination of the pulses.  This new optional pulseshape is denoted p’(t)  and is the sum of N  weighted and delayed “fundamental” pulses p(t)

p'(t)=i=1Naip(t−τi)									 7

where p(t)  has to follow the specifications of fundamental pulses according to Sec. 6.8a.3.3.2. The number of pulses N is set to a fixed value of 4 (though smaller values can be realized by setting the amplitudes of some of the pulses to zero. The values of the pulse delays shall be limited to . The numerical values of the delays and amplitudes of the pulses shall be transmitted following the general framework of optional pulseshapes, as defined in Sec. ???

### 6.8a.3.3.4 UWB PHY Optional Chirp pulses

This clause specifies a scheme of Chirp on UWB (CoU) which is an optional mode in addition to the mandatory direct sequence (DS) mode of UWB PHY. The purpose of CoU is to provide an additional dimension to support SOP as well as to achieve better ranging accuracy.

Since CoU mode is an optional pulse shape in addition to the mandatory pulse shape, all modulation specifications will remain the same as they are in the mandatory DS mode except those defined description of the pulse shape when a device implements the CoU option..

A mathematical representation of chirped DS pulse of CoU at baseband is given by Equation (1) and a graphical example of chirped DS pulse is shown in Figure 3.

PCoU(t)=P(t)expj−πμt22;−T2≤t≤T20;otherwise                     (1)

where P(t) denots the mandatory pulse and μ=BT the chirping rate (chirping slope). Moreover, B and T are the bandwidth and time duration of the chirped pulse respectively. When Raised cosine pulse is used as the mandatory pulse, we have

P(t)=1;t≤1−α1+αT2121+cos1+απαTt−1−α1+αT21;t>T2;1−α1+αT2<t≤T2          (2)

Figure 3 Graphical view of chirped pulse.

(HUAN BANG please include a table that shows what the parameters of equation 1 need to be for each channel shown in table 1.  From the motion passed in Vancouver we have 2 slopes in each of the 500MHz bands and 4 in each of the optional wider bandwidth channels)

| Channel Number |  (slopes) |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |
| 11 |  |
| 12 |  |
| 13 |  |
| 14 |  |
| 15 |  |

### 6.8a.3.3.5 UWB PHY Optional Continious Spectrum (CS) pulses

This clause specifies a scheme of Continuous Spectrum (CS) pulse which is an optional mode in addition to the mandatory direct sequence (DS) mode of UWB PHY. The purpose of CS filtering is to reduce the interference level between different piconets and then enhance SOP performance

Since CS mode is an optional pulse shape in addition to the mandatory pulse shape, all modulation specifications will remain the same as they are in the mandatory DS mode except those defined description of the pulse shape when a device implements the CS option..

A mathematical representation of CS filtered pulse is given by Equation (3) and a graphical example of chirping operation is shown in Figure 6.

gcs(t)=G(f)exp−j2πft−τ(f)df               (3)

where, g(t) denotes the original mandatory pulse, τ(f) the group delay and

G(f)=g(t)exp(−j2πft)dt               (4)

Figure 7 Examples of CS filtered pulses.

Some examples of the CS filtered pulses with different group delays are given in Figure 7.

(HUAN BANG please include a table that shows what the parameters of equation 1 need to be for each channel shown in table 1.  From the motion passed in Vancouver we have 2 CS waveforms in each of the 500MHz bands and 4 in each of the optional wider bandwidth channels)

| Channel Number | τ(f) (group delays) |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |
| 11 |  |
| 12 |  |
| 13 |  |
| 14 |  |
| 15 |  |

### 6.8a.3.3.5 UWB PHY Optional Chaotic pulses

### 6.8a.5. UWB PHY Spreading and Hopping Sequences

The constituent pulses in each burst are scrambled by applying a time varying scrambling sequence {(sj) in equation 3}.  This scrambler is simply a the pseudo-random binary sequence (PRBS) defined by a polynomial generator.  The polynomial generator, g(D), for the pseudo-random binary sequence (PRBS) generator shall be g(D) = 1 + D14 + D15, where D is a single bit delay element. The polynomial not only forms a maximal length sequence, but is also a primitive polynomial. Using this generator polynomial, the corresponding PRBS, sj, is generated as

sj=sj−14⊕sj−15, j=0,1,2,...

where “” denotes modulo-2 addition.

Figure 3 Realization of the scrambler linear feedback shift registers

Furtermore the hopping sequence is derived from the same linear feedback shift registers by using the output of the first three registers.  Specifically, when each symbol, x(k) (t), is generated by the UWB PHY the spreader of Error: Reference source not found is run at the chip rate for Nburst cycles the Nburst  consecutive outputs of the spreader are the spreading sequence for the symbol (sj, j = 1,2, …, Nburst).  Additionally, the current hopping position, h(k) is determined according to the following equation

h(k)=sj×20+sj−1×21+sj−2×22

Here the values of the state variables are sampled at the start of the transmission of the current modulation symbol.

### 6.8a.6. UWB PHY Channel coding within a band

TBD (Phil and Ismail aren’t sure what goes here is it description of when to start scrambling and FEC within a PHY PDU, e.g., after the PHY header?)

### 6.8a.7. UWB PHY Forward Error Correction

The FEC used by the UWB PHY is a concatenated code consiting of an outer Reed-Solomon (RS) systematic block code and an inner systematic convolutional code.

The outer RS code shall be a RS6(K+8,K) over Galois field GF(26). The Galois field GF(26) is built as an extension of GF(2). The systematic Reed Solomon code shall use the generator polynomial

where a= 010000 is a root of the binary primitive polynomial 1+x+x6 in GF(26). Both RS encoding with default codeword operation (K = 55) and shortened codeword operation, as defined below, are shall be required.

6.8a.7.1 Default Codeword Operation

In default codeword operation, RS6(63,55), a block of 330 bits is encoded into a codeword of 378 bits. The RS encoding procedure is performed in the following 3 steps:

- Bit to Symbol Conversion

The information bits {d(0), d(1), …, d(329)} are converted into 55 RS symbols {D(0), D(1), …, D(54)} as following:

D(k) = 32d(6k+5) + 16d(6k+4) + 8d(6k+3) + 4d(6k+2) + 2d(6k+1) + d(6k) for k = 0,1, …,54

Resulting 6-bit symbols are presented as:

D(k) = {d(6k+5), d(6k+5), d(6k+4), d(6k+3), d(6k+2), d(6k+1), d(6k)} for k = 0,1, …,54

where d(6k+5), …, d(6k) are ordered from the most significant bit (MSB) to the less significant bit (LSB).  The polynomial representation of a single information symbol over GF(26) in terms of a is given by:

Da(k) = a5d(6k+5) + a4d(6k+4) + a3d(6k+3) + a2d(6k+2) + a1d(6k+1) + d(6k)

- Encoding

The information symbols D(0), D(1), …, D(54) are encoded by systematic RS6(63,55) code with output symbols U(0), U(1), …, U(54) ordered as:

U(k) = D(k) 		for k = 0,1, …,54;

U(k) = P(k) 		for k = 55, 56, …,62;

where P(k) are parity check symbols added by RS6(63,55) encoder.

Information symbols are ordered in the descending polynomial order such that Da(54) corresponds to the lowest degree term of: D(x) = Da(54) + Da(53) x+ … + Da(0) x54, where D(x) is the polynomial representation of information symbols {D(0), D(1), …, D(54)} over Galois field.

Parity check symbols in polynomial representation over Galois field are ordered in the descending polynomial order such that Pa(62) is the lowest degree of                           P(x) = Pa(62) + Pa(61) x+ … + Pa(0) x7. The parity check symbols are calculated as:

P(x) = remainder [x8D(x)/g(x)], and U(x) = D(x) + x8P(x), i.e.

Ua(k) = Da(k) for k = 0, 1, …,54;

Ua(k) = Pa(k) for k = 55,56, …,62.

- Symbol to Bit Conversion

The output symbols {Ua(0), Ua(1), …, Ua(62)} are converted back into symbols {U(0), U(1), …, U(62)} and then into binary with LSB coming out first, resulting in a block of 378 bits {u(0), u(1), …, u(377)}.

6.8a.7.2 Shortened Codeword Operation

Shortened codeword operation, RS6(K,K+8),of a block of N bits is described by the follwing steps:

- Expand the block to 330 bits by adding 330-N dump (zero) bits

- Encode the expanded block using default codeword operation

- Remove the 330-N dump bits,

The outer encoding of the frame shall be peeformed in the following manner. The PLCP header (consisting of the PHY header, the MAC header and the HCS) shall be encoded using shortened codeword operation as an outer code. Next, the MPDU (MAC frame body followed by the FCS) shall be partitioned into blocks of 330 bits. Each block shall be encoded as RS6(63,55) using default codeword operation. The last block shall be shortened to the appropriate length using the shortened codeword operation.

The inner convolutional encoder shall use the rate R = 1/2 code with generator polynomials, g0 = [010]2 and g1 = [101]2, as shown in Figure 3.  In order to return the encoder to the all zero state one ‘zero’ bit shall be appended to the PDPU by the UWB PHY.

Figure 3 systematic convolutional encoder