| Project | IEEE 15.4a |  |
| --- | --- | --- |
| Title | Draft for optional CSMA multiple access control |  |
| Date Submitted | [Nov 23, 2005] |  |
| Source | [Bin Zhen, Yihong Qi, Huan-bang Li, Ruyji Kohno] [NiCT, 3-4 Hikarino-oka, Yokosuka, 239-0847, Japan] | Voice: [+81-46-847-5445] Fax: [+81-46-847-5431] E-mail: [zhen.bin@nict.go.jp |
| Re: | [] |  |
| Abstract | [Definition of optional CCA modes] |  |
| Purpose | [Preliminary optional CCA modes draft for further editing] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Draft for optional CSMA multiple access control of 15.4a

#### 6.9.9a Frame structure which supports optional CCA modes

The optional CCA modes are used to support heavy loaded network which consists of large number of devices in a piconet.

Because of the low radiation power, carrier-less, sparse and transient nature of impulse UWB signal, CCA of impulse UWB signal is considered to be difficult. Distinguishing from the carrier sense in the narrow band systems where CCA is provided by detecting energy from carrier, the CCA of impulse UWB symbol is implemented by detecting presense of UWB traffic. There are regular structures in the preamble portion of a frame (7.5.7a.2.1). The periodicity of preamble remains ever after multipath channel propogation. This enables the time average processing without frame synchronization. Spreading gain of the preamble symble also benefits detection of preamble symbol. As a contrast, after scrambling by a long scrambler sequence, the data portion is random and lack of reliable structure. To enable CCA of impulse UWB signal at any time, we introduce redular structure into the data portion of frame by interleaving preamble segments in the PSDU segments in time domain. The insterted preamble segments server as regular CCA structure of the frame.

Figure CS-1 shows the frame structure which support optional CCA modes. The PSDU part of frame originates from the MAC sublayer (5.4.3). After adding SHR and PHR at the beginning of PSDU, the PHY layer of transmitter regularly inserts preamble segments into the PSDU. Therefore the PSDU segments are interleaved by preamble segments alternatively. Each preamble segment consists of numPreSegSym preamble symbols. Each PSDU segments consists of numPSDUBit PSDU bits. The PSDU shall be ended with a preamble segment.

Fig.CS-1  Schematic view of frame structure which support optional CCA modes

The modulation of insterted preamble segements shall use mandatory data rate. The spread code of insterted preamble symbol shall be the same as that of the regular frame preamble. The time interval between neighbour insterted preamble segments is computed as per mandatory data rate (1Mbps). The PHY layers shall guarantee the constant time interval no matter which optional data rate is used.

The PHY PIB attribute phyCCAMode (see 6.4) shall indicate the appropriate optional CCA mode. Figure CS-2 shows the constant CCA detection window shall be equal to numCCAWin. Wherever the CCA detection window starts, either from the regular preamble or from the data portion in a frame, the CCA detectors shall find at least numPreSeg*numPreSegSym preamble symbols in the CCA detection window.

Fig.CS-2  CCA detection window

The frame structures shall only be applied to data frame and MAC frame in the CAP when the PHY PIB attribute phyCCAMode (see 6.4) is set to the optional CCA mode. The frame structures shall not applied to

- all frames when PHY PIB attribute phyCCAMode (see 6.4) is set to mandatory ALOHA mode;

- data frame in the CFP;

- beacon frame and acknowledgement frame.

If the PLME-CCA.request primitive is received by the PHY during reception of a PPDU, CCA shall report a busy medium. Otherwise, an idle medium shall be reported.

When receving the frame structure, the PHY layer of the destinated device simply skipps or discards the inserted preamble segements. Only the de-spread PSDU is passed to the MAC.

We are waiting for input of typical preamble length and data portion to decide the optimal value of numPreSeg, numPreSegSym, and numCCAWin.

In the beacon frame, only the mandotary features can be used. The ACK frame does not need to be CCA enabled because there are tow times of CCA in the CSMA-CA algorithm.