IEEE P802.15.4a

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Preamble codes for length 31 and 127 in Bands 1 to 4 |  |
| Date Submitted | November 15, 2005 |  |
| Source | Michael Mc Laughlin, Decawave | Voice: +353-1-295 4937 Fax: NONE E-mail: michael@decawave.com |
| Re: | Preamble codes for draft specification for 802.15.4a |  |
| Abstract | Provides 8 codes of length 127 for the preamble with low intra-band cross-correlation, 8 alternative codes of length 127 that reduce the cross-correlation to band 4 and 8 codes of length 31 that minimize intra-band cross-correlation. |  |
| Purpose | To provide a working framework document for the TG4a draft. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Set of 8 codes for Length 127 preamble – Version of set of 8 provided by Ismail. Phase shifted to maximize merit factor.

### Band 1 codes

S1 = −−+−0+0000+−−000+00−00++0+−0−0−+0++++0−+000+−0+00+0−−−0++−00−+00−0+0−0−+−−−−+000000+00000−+0000+0+000+−−+00−000−0++00+++0+0+00+

S2 = ++0000+000+00+−−0+−++0−000−−00+−0+00++000+++00+0+0−0−+−0−0+00+00+0++−−−−+00++−−+0+−0−−+000000−0−0000−+0−−00+00000+−++000−0−+0+0

Band 2 codes

S3 = −+−0000+00−−00000−0+0+0+−0+00+00+0−00−+++00+000−+0+0−0000+++++−+0+−−0+−0++−−0−000+0−+00+0+−−−−000−000000−+00+−0++000++−00++−0−0

S4 = +++0000++−−+000+0−−0++00+−000−+0+00++0+++++−+0−−+00+−−−0+00000+0−0−+−−00+00−000−0000+00+−+0+−0+0−0−00+0++−0−0+−000000+−00−0−000

Band 3 codes

S5 = −−0+000−00+−+−000−0−0000++00000+000000+++−++−0+0+0−00++00−++0+−−0−00+0++000+−0+++−0−+0−0−−0−−00+00+000+−+0000+0−++−−00+0+0+−−00

S6 = +000++0−0+0−00+−0−+0−00+0+0000+0+−0000++00+0+++++−+0−0+−0−−+0++−−000−−−0+000+0+0−+−000000+−+−0−−00++000−00+00++−00−−++−00−00000

Band 4 exclusive codes (Note: Band 4 may also use Band 1−3 codes)

S7 = +0000−0000−+0+00000−−++−0−+000000−0+0+−0−+++++−00+−0++0+0−0+000−00−00++00−+−−000−++0+++0+0++−+0−00+0−−00+0+00++−00−000+−000−0−−

S8 = −000−0−00000+−−0++0−00++−−0000+00−+0+−+0+000−000−+0−0−−00+−+00+−00+0++++0−0−00−0−0+0+++000−−+++0−++−+++00+00+0000−+000000−0+−0+

Coherent Cross Correlation Matrix

| Band 1; Band 2; Band 3; Band 4. S1; S2; S3; S4; S5; S6; S7; S8. Band 1; S1; 64; 8; 12; 10; 13; 14; 8; 12. S2; 8; 64; 12; 8; 8; 8; 8; 12. Band 2; S3; 12; 12; 64; 8; 12; 12; 8; 8. S4; 10; 8; 8; 64; 8; 8; 12; 12. Band 3; S5; 13; 8; 12; 8; 64; 8; 12; 8. S6; 14; 8; 12; 8; 8; 64; 12; 8. Band 4; S7; 8; 8; 8; 12; 12; 12; 64; 8. S8; 12; 12; 8; 12; 8; 8; 8; 64. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 12 | 8 | 10 | 12 | 8 | 8 |
|  | S2 | 8 | 64 | 8 | 8 | 12 | 12 | 12 | 8 |
| Band 2 | S3 | 12 | 8 | 64 | 8 | 8 | 8 | 12 | 12 |
|  | S4 | 8 | 8 | 8 | 64 | 12 | 8 | 12 | 8 |
| Band 3 | S5 | 10 | 12 | 8 | 12 | 64 | 8 | 8 | 12 |
|  | S6 | 12 | 12 | 8 | 8 | 8 | 64 | 8 | 12 |
| Band 4 | S7 | 8 | 12 | 12 | 12 | 8 | 8 | 64 | 8 |
|  | S8 | 8 | 8 | 12 | 8 | 12 | 12 | 8 | 64 |

Alternative Set of 8 codes for Length 127 preamble

S1 = −0−+++00−−+0−+−−+−−0−+00−0+−000−−00++0−+0+000+−+−+0000+++000−0+00−+000000++0−0+−−0+00−0−0+0++0+++00+0000−000+00−00−−++0+0+00000

S2 = +−−000−0−0000+−00000+000000+−−+−++0−0+0+00+−00+++0−++0−00+0−+000++0+++−0−−0+0+−0−−00−00+000−++0000+0++−+−00+0+0+−−00−−0−000+00+

S3 = −0−++00−++000++0−+00+−000000−000−−−−+0+00+−0+000−0−−++0−+0−−+0+−+++++0000−0+0+−000+00+++−00−0+00+00+0−+0+0+0−00000−−00+0000−+−0

S4 = 0−+0+−−−000000−+−0−0+000+0++−000−−−+0++−0−−0+0−+−−+++0+00++0000++0+0000−0+00+0+−0++00+0−0−0−−000+00000+00−++−−00+−+00+00+000−+0

S5 = −00−+−+0000−00+−0−−+0+000−000++0+0+−00+++00−−00−0−−++0−0+00−0+0−0−−+000−+−−+0−−+−+++00+00+0000−+000000+0+−0++000+0−00000+++0++0

S6 = 0++00+00000−−++000+0++0+0++0000−000−00−+−0+++−0−000−+00++0+00−+000+++00−0−0+0+−−0−0+00−00+0−−++−−+00+−+−+0−−0−−+000000−0+0000−+

S7 = −−+00000+0−−0000−0000+−−0−000−+000+00−++00+0+00++0−00−0++++0−0++−0−+−000++−+00+−00−00−000+0+0+0++0+−00++−+−−−0+−0+0−000000++0+−

S8 = −−0+++0000+++−−−−000+++0+−000+0+00+0+−++−0−0−0−0000+0−+0+−++00+−−00+0−0++00−+00000+−0−+0−0+−+0−000−−00−000−000000+00+00+−0+00++

Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 12 | 12 | 8 | 8 | 8 | 8 |
|  | S2 | 8 | 64 | 8 | 20 | 22 | 22 | 8 | 8 |
| Band 2 | S3 | 12 | 8 | 64 | 8 | 22 | 12 | 8 | 8 |
|  | S4 | 12 | 20 | 8 | 64 | 20 | 20 | 8 | 8 |
| Band 3 | S5 | 8 | 22 | 22 | 20 | 64 | 8 | 8 | 8 |
|  | S6 | 8 | 22 | 12 | 20 | 8 | 64 | 8 | 8 |
| Band 4 | S7 | 8 | 8 | 8 | 8 | 8 | 8 | 64 | 8 |
|  | S8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 64 |

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 8 | 8 | 8 | 12 | 12 | 12 |
|  | S2 | 8 | 64 | 12 | 12 | 12 | 12 | 8 | 8 |
| Band 2 | S3 | 8 | 12 | 64 | 8 | 8 | 8 | 12 | 12 |
|  | S4 | 8 | 12 | 8 | 64 | 8 | 8 | 8 | 12 |
| Band 3 | S5 | 8 | 12 | 8 | 8 | 64 | 8 | 12 | 12 |
|  | S6 | 12 | 12 | 8 | 8 | 8 | 64 | 8 | 8 |
| Band 4 | S7 | 12 | 8 | 12 | 8 | 12 | 8 | 64 | 8 |
|  | S8 | 12 | 8 | 12 | 12 | 12 | 8 | 8 | 64 |

Set of 8 codes for Length 31 preamble

S1 = +−00+−0−−−++0+000+00+0+0−+0000+

S2 = 0000−0++0−0+000+−+0++++−00−00−+

S3 = 0000+−00−00−++++0+−+000+0−0++0−   Was S3

S4 = −0+−00+++−+000−+0+++0−0+0000−00   Was S4

S5 = 0+00−0−0++0000−−+00−+0++−++0+00   Was S6

S6 = ++00+00−−−+−0++−000+0+0−+0+0000

S7 = +0000+−0+0+00+000+0++−−−0−+00−+   Was S2

S8 = −+0++000−+−++00++0+00−0000−0+0−   Was S1

Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 6 | 6 | 4 | 4 | 9 | 6 |
|  | S2 | 4 | 0 | 9 | 6 | 6 | 6 | 6 | 6 |
| Band 2 | *S3* | 6 | 9 | 0 | 4 | 6 | 4 | 4 | 4 |
|  | S4 | 6 | 6 | 4 | 0 | 6 | 4 | 6 | 6 |
| Band 3 | S5 | 4 | 6 | 6 | 6 | 0 | 4 | 6 | 4 |
|  | S6 | 4 | 6 | 4 | 4 | 4 | 0 | 6 | 6 |
| Band 4 | S7 | 9 | 6 | 4 | 6 | 6 | 6 | 0 | 4 |
|  | S8 | 6 | 6 | 4 | 6 | 4 | 6 | 4 | 0 |

** Note : S3 is best 3rd choice code for Band 4.

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 4 | 4 | 16 | 4 | 6 | 4 |
|  | S2 | 4 | 0 | 6 | 4 | 4 | 6 | 4 | 4 |
| Band 2 | S3 | 4 | 6 | 0 | 4 | 4 | 16 | 4 | 4 |
|  | S4 | 4 | 4 | 4 | 0 | 4 | 4 | 4 | 6 |
| Band 3 | S5 | 16 | 4 | 4 | 4 | 0 | 4 | 6 | 4 |
|  | S6 | 4 | 6 | 16 | 4 | 4 | 0 | 4 | 4 |
| Band 4 | S7 | 6 | 4 | 4 | 4 | 6 | 4 | 0 | 4 |
|  | S8 | 4 | 4 | 4 | 6 | 4 | 4 | 4 | 0 |