IEEE P802.15.4a

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Preamble codes for length 31 and 127 in Bands 1 to 4 |  |
| Date Submitted | November 15, 2005 |  |
| Source | Michael Mc Laughlin, Decawave | Voice: +353-1-295 4937 Fax: NONE E-mail: michael@decawave.com |
| Re: | Preamble codes for draft specification for 802.15.4a |  |
| Abstract | Provides 8 codes of length 127 for the preamble with low intra-band cross-correlation, 8 alternative codes of length 127 that reduce the cross-correlation to band 4 and 8 codes of length 31 that minimize intra-band cross-correlation. |  |
| Purpose | To provide a working framework document for the TG4a draft. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Four sets of codes are presented. Two possibilities for Length 127 and two possibilities for length 31.

Length 127 set of 8, Option 1.

The first set of Length 127 codes assigns 2 codes per band and has worst case cross correlation of 8/64 between each pair in the band.

Length 127 set of 8, Option 2.

The second set of Length 127 codes also assigns 2 codes per band and has worst case cross correlation of 8/64 between each pair in the band. It has the added advantage that the worst case cross correlation between the codes in band 4 and the other codes is 8. This is a big advantage because there is no frequency isolation between band 4 and bands 1 to 3.

The second of these options doesn’t seem to have any disadvantages and is therefore recommended.

Length 31 set of 8, Option 1.

The first set of Length 31 codes assigns 2 codes per band and has worst case cross correlation of 4/16 between each pair in the band. The down side is that the cross-correlation between S1 and S7 is 9/16.

Length 31 set of 8, Option 2.

The first set of Length 31 codes assigns 2 codes per band and has worst case cross correlation of 4/16 between each pair in bands 1 to 3 and 6/16 in band 4. The worst case cross-correlation between band 4 and bands 1 to 3 is 6/16.

The second of these options is also recommended, because of the better worst case cross correlation. It also means that we have added two codes to out previous set of 6, without making the previous worst case cross correlation of 6 any worse.

Set of 8 codes for Length 127 preamble – Version of set of 8 provided by Ismail. Phase shifted to maximize merit factor.

### Band 1 codes

S1 = −−+−0+0000+−−000+00−00++0+−0−0−+0++++0−+000+−0+00+0−−−0++−00−+00−0+0−0−+−−−−+000000+00000−+0000+0+000+−−+00−000−0++00+++0+0+00+

S2 = ++0000+000+00+−−0+−++0−000−−00+−0+00++000+++00+0+0−0−+−0−0+00+00+0++−−−−+00++−−+0+−0−−+000000−0−0000−+0−−00+00000+−++000−0−+0+0

Band 2 codes

S3 = −+−0000+00−−00000−0+0+0+−0+00+00+0−00−+++00+000−+0+0−0000+++++−+0+−−0+−0++−−0−000+0−+00+0+−−−−000−000000−+00+−0++000++−00++−0−0

S4 = +++0000++−−+000+0−−0++00+−000−+0+00++0+++++−+0−−+00+−−−0+00000+0−0−+−−00+00−000−0000+00+−+0+−0+0−0−00+0++−0−0+−000000+−00−0−000

Band 3 codes

S5 = −−0+000−00+−+−000−0−0000++00000+000000+++−++−0+0+0−00++00−++0+−−0−00+0++000+−0+++−0−+0−0−−0−−00+00+000+−+0000+0−++−−00+0+0+−−00

S6 = +000++0−0+0−00+−0−+0−00+0+0000+0+−0000++00+0+++++−+0−0+−0−−+0++−−000−−−0+000+0+0−+−000000+−+−0−−00++000−00+00++−00−−++−00−00000

Band 4 exclusive codes (Note: Band 4 may also use Band 1−3 codes)

S7 = +0000−0000−+0+00000−−++−0−+000000−0+0+−0−+++++−00+−0++0+0−0+000−00−00++00−+−−000−++0+++0+0++−+0−00+0−−00+0+00++−00−000+−000−0−−

S8 = −000−0−00000+−−0++0−00++−−0000+00−+0+−+0+000−000−+0−0−−00+−+00+−00+0++++0−0−00−0−0+0+++000−−+++0−++−+++00+00+0000−+000000−0+−0+

Coherent Cross Correlation Matrix

| Band 1; Band 2; Band 3; Band 4. S1; S2; S3; S4; S5; S6; S7; S8. Band 1; S1; 64; 8; 12; 10; 13; 14; 8; 12. S2; 8; 64; 12; 8; 8; 8; 8; 12. Band 2; S3; 12; 12; 64; 8; 12; 12; 8; 8. S4; 10; 8; 8; 64; 8; 8; 12; 12. Band 3; S5; 13; 8; 12; 8; 64; 8; 12; 8. S6; 14; 8; 12; 8; 8; 64; 12; 8. Band 4; S7; 8; 8; 8; 12; 12; 12; 64; 8. S8; 12; 12; 8; 12; 8; 8; 8; 64. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 12 | 8 | 10 | 12 | 8 | 8 |
|  | S2 | 8 | 64 | 8 | 8 | 12 | 12 | 12 | 8 |
| Band 2 | S3 | 12 | 8 | 64 | 8 | 8 | 8 | 12 | 12 |
|  | S4 | 8 | 8 | 8 | 64 | 12 | 8 | 12 | 8 |
| Band 3 | S5 | 10 | 12 | 8 | 12 | 64 | 8 | 8 | 12 |
|  | S6 | 12 | 12 | 8 | 8 | 8 | 64 | 8 | 12 |
| Band 4 | S7 | 8 | 12 | 12 | 12 | 8 | 8 | 64 | 8 |
|  | S8 | 8 | 8 | 12 | 8 | 12 | 12 | 8 | 64 |

Alternative Set of 8 codes for Length 127 preamble. Maximum coherent cross correlation from band 4 to any other band is now 8 .

### Band 1 codes

S1 = −0−+++00−−+0−+−−+−−0−+00−0+−000−−00++0−+0+000+−+−+0000+++000−0+00−+000000++0−0+−−0+00−0−0+0++0+++00+0000−000+00−00−−++0+0+00000

S2 = +−−000−0−0000+−00000+000000+−−+−++0−0+0+00+−00+++0−++0−00+0−+000++0+++−0−−0+0+−0−−00−00+000−++0000+0++−+−00+0+0+−−00−−0−000+00+

### Band 2 codes

S3 = −0−++00−++000++0−+00+−000000−000−−−−+0+00+−0+000−0−−++0−+0−−+0+−+++++0000−0+0+−000+00+++−00−0+00+00+0−+0+0+0−00000−−00+0000−+−0

S4 = 0−+0+−−−000000−+−0−0+000+0++−000−−−+0++−0−−0+0−+−−+++0+00++0000++0+0000−0+00+0+−0++00+0−0−0−−000+00000+00−++−−00+−+00+00+000−+0

### Band 3 codes

S5 = −00−+−+0000−00+−0−−+0+000−000++0+0+−00+++00−−00−0−−++0−0+00−0+0−0−−+000−+−−+0−−+−+++00+00+0000−+000000+0+−0++000+0−00000+++0++0

S6 = 0++00+00000−−++000+0++0+0++0000−000−00−+−0+++−0−000−+00++0+00−+000+++00−0−0+0+−−0−0+00−00+0−−++−−+00+−+−+0−−0−−+000000−0+0000−+

### Band 4 exclusive codes (Note: Band 4 may also use Band 1−3 codes)

S7 = −−+00000+0−−0000−0000+−−0−000−+000+00−++00+0+00++0−00−0++++0−0++−0−+−000++−+00+−00−00−000+0+0+0++0+−00++−+−−−0+−0+0−000000++0+−

S8 = −−0+++0000+++−−−−000+++0+−000+0+00+0+−++−0−0−0−0000+0−+0+−++00+−−00+0−0++00−+00000+−0−+0−0+−+0−000−−00−000−000000+00+00+−0+00++

Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 12 | 12 | 8 | 8 | 8 | 8 |
|  | S2 | 8 | 64 | 8 | 20 | 22 | 22 | 8 | 8 |
| Band 2 | S3 | 12 | 8 | 64 | 8 | 22 | 12 | 8 | 8 |
|  | S4 | 12 | 20 | 8 | 64 | 20 | 20 | 8 | 8 |
| Band 3 | S5 | 8 | 22 | 22 | 20 | 64 | 8 | 8 | 8 |
|  | S6 | 8 | 22 | 12 | 20 | 8 | 64 | 8 | 8 |
| Band 4 | S7 | 8 | 8 | 8 | 8 | 8 | 8 | 64 | 8 |
|  | S8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 64 |

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 64 | 8 | 8 | 8 | 8 | 12 | 12 | 12 |
|  | S2 | 8 | 64 | 12 | 12 | 12 | 12 | 8 | 8 |
| Band 2 | S3 | 8 | 12 | 64 | 8 | 8 | 8 | 12 | 12 |
|  | S4 | 8 | 12 | 8 | 64 | 8 | 8 | 8 | 12 |
| Band 3 | S5 | 8 | 12 | 8 | 8 | 64 | 8 | 12 | 12 |
|  | S6 | 12 | 12 | 8 | 8 | 8 | 64 | 8 | 8 |
| Band 4 | S7 | 12 | 8 | 12 | 8 | 12 | 8 | 64 | 8 |
|  | S8 | 12 | 8 | 12 | 12 | 12 | 8 | 8 | 64 |

Set of 8 codes for Length 31 preamble

### Band 1 codes

S1 = +−00+−0−−−++0+000+00+0+0−+0000+

S2 = 0000−0++0−0+000+−+0++++−00−00−+

### Band 2 codes

S3 = 0000+−00−00−++++0+−+000+0−0++0−   Was S3

S4 = −0+−00+++−+000−+0+++0−0+0000−00   Was S4

### Band 3 codes

S5 = 0+00−0−0++0000−−+00−+0++−++0+00   Was S6

S6 = ++00+00−−−+−0++−000+0+0−+0+0000

### Band 4 exclusive codes (Note: Band 4 may also use Band 1−3 codes)

S7 = +0000+−0+0+00+000+0++−−−0−+00−+   Was S2

S8 = −+0++000−+−++00++0+00−0000−0+0−   Was S1

Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 6 | 6 | 4 | 4 | 9 | 6 |
|  | S2 | 4 | 0 | 9 | 6 | 6 | 6 | 6 | 6 |
| Band 2 | *S3* | 6 | 9 | 0 | 4 | 6 | 4 | 4 | 4 |
|  | S4 | 6 | 6 | 4 | 0 | 6 | 4 | 6 | 6 |
| Band 3 | S5 | 4 | 6 | 6 | 6 | 0 | 4 | 6 | 4 |
|  | S6 | 4 | 6 | 4 | 4 | 4 | 0 | 6 | 6 |
| Band 4 | S7 | 9 | 6 | 4 | 6 | 6 | 6 | 0 | 4 |
|  | S8 | 6 | 6 | 4 | 6 | 4 | 6 | 4 | 0 |

** Note : S3 is best 3rd choice code for Band 4.

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 4 | 4 | 16 | 4 | 6 | 4 |
|  | S2 | 4 | 0 | 6 | 4 | 4 | 6 | 4 | 4 |
| Band 2 | S3 | 4 | 6 | 0 | 4 | 4 | 16 | 4 | 4 |
|  | S4 | 4 | 4 | 4 | 0 | 4 | 4 | 4 | 6 |
| Band 3 | S5 | 16 | 4 | 4 | 4 | 0 | 4 | 6 | 4 |
|  | S6 | 4 | 6 | 16 | 4 | 4 | 0 | 4 | 4 |
| Band 4 | S7 | 6 | 4 | 4 | 4 | 6 | 4 | 0 | 4 |
|  | S8 | 4 | 4 | 4 | 6 | 4 | 4 | 4 | 0 |

Set of 8 codes for Length 31 preamble

### Band 1 codes

−0000+0−0+++0+−000+−+++00−+0−00

0+0+−0+0+000−++0−+−−−00+00++000   (was S5 previously)

### Band 2 codes

−+0++000−+−++00++0+00−0000−0+0−   (was S1 previously)

0000+−00−00−++++0+−+000+0−0++0−   (was S3 previously)

### Band 3 codes

−0+−00+++−+000−+0+++0−0+0000−00   (was S4 previously)

++00+00−−−+−0++−000+0+0−+0+0000

### Band 4 exclusive codes (Note: Band 4 may also use Band 1−3 codes)

+0000+−0+0+00+000+0++−−−0−+00−+   (was S2 previously)

0+00−0−0++0000−−+00−+0++−++0+00   (was S6 previously)

Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 4 | 6 | 11 | 6 | 6 | 4 |
|  | S2 | 4 | 0 | 6 | 6 | 6 | 11 | 4 | 6 |
| Band 2 | *S3* | 4 | 6 | 0 | 4 | 6 | 6 | 4 | 4 |
|  | S4 | 6 | 6 | 4 | 0 | 4 | 4 | 4 | 6 |
| Band 3 | S5 | 11 | 6 | 6 | 4 | 0 | 4 | 6 | 6 |
|  | S6 | 6 | 11 | 6 | 4 | 4 | 0 | 6 | 4 |
| Band 4 | S7 | 6 | 4 | 4 | 4 | 6 | 6 | 0 | 6 |
|  | S8 | 4 | 6 | 4 | 6 | 6 | 4 | 6 | 0 |

** Note : S3 is best 2nd choice code for Band 4.

Non−Coherent Cross Correlation Matrix

|  |  | Band 1 |  | Band 2 |  | Band 3 |  | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| Band 1 | S1 | 0 | 4 | 16 | 4 | 6 | 4 | 4 | 4 |
|  | S2 | 4 | 0 | 4 | 6 | 4 | 6 | 4 | 4 |
| Band 2 | S3 | 16 | 4 | 0 | 4 | 6 | 4 | 4 | 4 |
|  | S4 | 4 | 6 | 4 | 0 | 4 | 16 | 4 | 4 |
| Band 3 | S5 | 6 | 4 | 6 | 4 | 0 | 4 | 4 | 4 |
|  | S6 | 4 | 6 | 4 | 16 | 4 | 0 | 4 | 4 |
| Band 4 | S7 | 4 | 4 | 4 | 4 | 4 | 4 | 0 | 6 |
|  | S8 | 4 | 4 | 4 | 4 | 4 | 4 | 6 | 0 |