IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | doc.: IEEE-15-06-0055-02-003c (UMD), Draft |  |
| Date Submitted | [18Jan06] |  |
| Source | [Ali Sadri] [Intel Corporation, 13290 Evening Creek Drive ,San Diego , CA 92128-3419 ,USA ] | Voice: [+1 858-774-6202] FAX: [] E-Mail: [ali.s.sadri@intel.com] |
| Re: | Move reference to reference section |  |
| Abstract | [802.15.3c Usage Model Document] |  |
| Purpose | This document defines usage models for 802.15.3c, The UMD, or Usage Model Document, defines the standard’s features, and all other elements which must be defined to enable standard success in the marketplace. The UMD is the guide for the Technical Requirements, and to generate simulation results for specified well-defined simulation scenarios provided by the Selection Criteria and Channel Modeling documents.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Ali Sadri | Intel Corporation | Intel Corporation, 13290 Evening Creek Drive ,San Diego , CA 92128-3419 ,USA | 858-774-6202 |  | ali.s.sadri@intel.com |
| Alireza Seyedi | Philips | Philips, 345 Scarborough Rd., Briarcliff Manor, NY, 10510 | 914-945-6318 |  | Alireza.seyedi@philips.com |
| Tony Pollock |  |  |  |  |  |
| Kazuaki Takahashi | Panasonic | 4-12-4, Higashi-Shinagawa, Shinagawa-ku, Tokyo 140-8507, JAPAN | +81-6710-2029 |  | takahashi.kazu@jp.panasonic.com |
| Raymond Yu Zhan | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 |  |  | Raymond.Yuz@sg.panasonic.com |

Abstract

This document defines usage models for 802.15.3c, The UMD, or Usage Model Document, defines the standard’s features, and all other elements which must be defined to enable standard success in the marketplace. The UMD is the guide for the Technical Requirements, and to generate simulation results for specified well-defined simulation scenarios provided by the Selection Criteria and Channel Modeling documents.]

NEED TO UPDATE AS WE COMPLETE THE DOCUMENT

# Revision History of Document 15-06-0055

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Initial version of the Usage Model Document. Changed name from MRD to UMD IEEE 802.15-06-0055-00-003c | January 18, 2006 | Ali Sadri, Ian C. Gifford |
| R1 | Add Draft to Usage Model Document (UMD) to the name IEEE 802.15-06-0055-01-003c | January 18, 2006 | Ali Sadri |
| R2 | Add Contributors list | February 1, 2006 | Ali Sadri |
| R3 | Consolidate comments from Alireza, Tony and Kazuaki | February 8, 2006 | Ali Sadri |

# Introduction

To support the definition of the mmWave higher throughput WPAN standard (which will incorporate changes to PHY and MAC if needed) within the IEEE, this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of simulations that will allow 802.15 members to evaluate the performance of various proposals in terms of, for example, network throughput, delay, packet loss and other metrics. It is intended that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by 802.15.3c.

Note - These usage models that the usage model committee develops here are subject to the following constraints:

C1:	They are relevant to the expected uses of the technology

C2:	They require higher throughput than can be achieved with existing 802.15 and similar technologies

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

The 802.15.3c Usage Model Document committee has been given responsibility for maintaining this document.

# Definitions

This section defines some of the terms used in this document.

Application – a source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video, Wireless Display, High Capacity disc drive synchronization.

Environment – The type of place where a WPAN, or short range wireless communications system, is deployed.  Examples are home and office.

Use case – A use case is a description of how an end user uses a system that exercises that system’s deployment of WPAN.  A use case includes an Application in a deployment environment with details regarding the user activity and both sides of the link.

Examples: Watching television remote from the cable or set-top box within the home.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage models are created to "cover" use cases.

Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario: environment linked to a channel model, position of the transmitter and receiver traffic (# packets, size of packets, interference (number and types of users on the same WPAN channel.  A Simulation Scenario is created from a Usage Model by characterizing the traffic profile of the Applications and possibly merging multiple Applications together to reduce simulation time.

# Mappings between Application, Environment, Channel Model, Use case, Usage Model and Simulation Scenario

Understanding and defining the application, environment, channel model, use case, usage model and simulation scenario are all necessary to create comparative results from 802.15.3c proposals.

Channel models have been defined in [xxx], with YY channel models.  .

Each use case involves the use of one or more applications and is defined for one or more environments.  It represents a single type of use of a system using the technology.

Each application reflects a source or sink of data.   They will eventually be characterized in terms of a traffic profile that allows a simulation of the application to be created.

Each usage model contains a representative mixture of applications and channel models designed to adequately cover the important use cases.  There is a many to many mapping between use cases and usage models (i.e., the same use case may contribute to multiple usage models and the same usage model may include applications from multiple use cases).

There will be a one-to-one mapping between usage models and simulation scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# Environments

The channel models identified in [xxx] are described in Table 1.

Table 1 - Environment to Channel Model Mapping

| Environment |  |  | Model |
| --- | --- | --- | --- |
| Indoor | Enterprise | Convention center | A |
|  |  | Open office |  |
|  |  | Warehouse |  |
|  |  | Intra closed office | B |
|  | Residential | Intra closed room |  |
|  | Enterprise | Inter closed office | C |
|  | Residential | Inter closed room |  |
|  | Enterprise | Train - platform link | D |

Need to update with the following data

The list of environments being considered is shown in Table 2.  This list is here to allow this document to relate an environment to a channel model.

Table 2 - Environment Definitions

| Applicable Model | Multipath | Obstruction in LOS | Doppler |
| --- | --- | --- | --- |
| A | Light to moderate | Humans, walls, etc. | Some: ≤ 3 m/s |
| B | Heavy | Humans, walls, etc. | Some: ≤ 3 m/s |
| C | Very heavy | Humans, walls, etc. | Some: ≤ 3 m/s |
| D | Very light to moderate | Atmospheric particulates, glass, etc. | None |

Need to update with the following data

# Applications

Table 4 lists the applications that are referred to from the usage models, together with relevant traffic parameters.

The parameters used to define the application are defined in Table 3.

Table 3 - Application Parameter Definitions

| Parameter | Definition |
| --- | --- |
| MSDU size | Packet size at the top of the MAC |
| Maximum PLR | Maximum packet loss rate at the top of the MAC. This is defined by the loss rate that can be tolerated by the application. |
| Maximum Delay | Maximum transport delay at the top of the MAC |
| Protocol | Indicates the network-layer protocol running between the data source and the MAC. It takes one of two values: TCP or UDP. These two protocols are intended to represent a generic acknowledged and a generic unacknowledged network-layer protocol. |
| Offered Load | This parameter is interpreted differently according to the Protocol parameter: Applications identified as being carried by UDP are assumed to generate MSDUs at a fixed rate, as identified in the "Offered load" column. Inability to carry the traffic generated by a UDP application, due to insufficient throughput capability, results in lost MSDUs, which is reported in simulation results as a packet loss rate, or an outage, associated with the application. The comparison criteria include a measure of whether this packet loss rate exceeds the maximum specified for the application in this table. Traffic carried by TCP is assumed to be served on a best-effort basis, and applications using TCP are assumed to generate MSDUs at rates up to the value given in the "Offered load" column. Being an acknowledged protocol with a constrained window size, TCP responds to congestion in the BSS by reducing application throughput without losing MSDUs. This effect is reflected in simulation results by reporting achieved throughput for applications using TCP. Note: Acknowledgement traffic is generated by TCP sinks, this is not explicitly specified in the simulation scenarios, but it is included in the count of non-QoS flows and measurement of goodput. |

The delay and PLR requirements for each application are specified in Table 4. Simulations may map applications and flows to specific QoS classes as necessary to satisfy these requirements. Proposers shall clearly state how applications and flows are mapped to specific QoS classes in their simulations.

Table 4 - Application Definitions

| Number | Application | Offered Load (Gbps) | MSDU Size (B) | Maximum PLR | Maximum Delay (ms) | Range (meter) | Number |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HDTV Video/Audio streaming [DVD players and other power-line operated devices] | 0.72 0.81 0.96 2.2275 | ? | ? | 200 | 3-10 | 1 |
| 2 | HDTV Video/Audio streaming [video camera/mobile devices and other battery operated devices] | 0.27 0.72 0.81 0.96 | ? | ? | 200 | 1-6 | 2 |
| 3 | Internet bulky music and video downloading [computing devices] | 2-3 | ? | ? | 200 | 1-10 | 3 |
| 4 | Internet bulky music and video downloading [mobile devices] | 1-2 | ? | ? | 200 | 0-3 | 4 |
| 5 | Internet small size file transfer (email, web, chat) | 1 | ? | ? | 500 | 0-6 | 5 |
| 6 | Local file transfer for printing, document and small size file | 1 | ? | ? | 500 | 0-6 | 6 |
| 7 | Local file transfer for bulky music and video, point-to-point connection (photo/video camera and photo/video handphone, mp3 player) | 1-2 | ? | ? | 200 | 0-3 | 7 |
| 8 | Wireless desktop | 1-2? | ? | ? | 200 | 2-6 | 8 |
| 9 | Wireless display at projector and TV | 1-2? | ? | ? | 200 | 2-10 | 9 |
| 10 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type | 1? | ? | ? | 50ms? | 0-6 | 10 |
| 11 | Interactive Gaming [between mobile devices] | 1? | ? | ? | 50ms? | 0-3 | 11 |

# Use Cases

Table 5 contains a definition of the use cases used in this document.

The score relates to the results reported in [4] from the vote on 21 July 2003.  This scores 3 for high, 2 for medium and 1 for low priority.  The "Devn" column shows the weighted absolute deviation in the votes (0 shows complete agreement and 1 shows complete disagreement).

Table 5 - Use Case Definitions

| # | Covered by model # | Use case | Application | Environment | Score | Devn. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1, 2 | Video streaming. | Video streaming everywhere – home, office, devices with fixed power line or battery | Residential, Enterprise – large and small, conference room |  |  |
| 2 | 5, 6 | Small size file transferring and sharing | Email, news, chat, printing, small size video and audio | Residential/small enterprise/ public transportation |  |  |
| 3 | 3, 4, 7 | Bulky music and video downloading | Large music and video contents and files downloading | Residential/small enterprise/public kiosk and transportation |  |  |
| 4 | 8, 9 | Wireless display | Wireless desktop, wireless display at TV and projector | Residential/small enterprise/ conference room |  |  |
| 5 | 10, 11 | Gaming | Interactive gaming anywhere within the home / public (console to internet, between mobile devices) | Residential/small enterprise (internet cafes)/public transportation |  |  |
| # | Covered by model # | Use case | Application | Environment | Score | Devn. |

# Usage Models

Table 6 defines the usage models defined by this document.

The purpose of these models is to merge representative use cases to create a small number of credible worst-case mixtures of applications.  The usage models have to be realistic (in terms that they are covered by the use cases listed above), different from each other and cover some subset of the use cases that are identified to be priorities and capable of implementation in proposed 802.15.3c technology.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the 802.11n comparison criteria.

Table 6 - Usage Model Definitions

| Model # | Usage Model | Covers Use Cases | Score (ave/devn) | Application mix | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Residential | 3, 8, 10, 33, 1, 32, 11, 29, 7, 21 | 2.8/0.3 | 1 x AP STA 1: 19.2 Mbps HDTV (A4) , VoD control channel (A2) STA 3: 24 Mbps HDTV (A4) , VoD control channel (A2) STA 4: SDTV (A3), internet file transfer (A14), local file transfer (A15) STA 5 & 6: Video Phone (A23) STA 7,8 & 9: VoIP (A9) STA 10: Internet streaming video (A7), MP3 Audio (A12), Video gaming, console to internet (A18), local file transfer (A15) STA 11: Video gaming, controller to console (A16) | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. 20m range. |
| 2 | Residential IBSS | 4, 33, 11, 34, 15, 21 | 2.3/0.6 | 0 AP STA 1: peer-peer DV Audio/Video (A1) sink STA 2,3: local file transfer (A15) sink, local file transfer (A15) source STA 4,5,6,7: Video Phone (A23) sink, Video Phone (A23) source STA 8: 4 x Video gaming, controller to console (A16), sink STA 9: content download (A13) sink STA 10: peer-peer DV Audio/Video (A1) (rate, range combination > current technology) source STA 11,12,13,14: Video gaming, controller to console (A16) source | Note, all these devices are operating on the same channel. |

Usage models 3,8,7,10,13,14 and 15 are reserved (they were defined in an earlier revision of this document, but have now been deleted);

## Coexistence (Informative)

802.15.3c will liaise with 802.19 TAG regarding coexistence requirements.

# Simulation Scenarios

## Common Conditions

Table 1 defines conditions that are common to all simulation scenarios.

Table 7 - Common Simulation Conditions

| Condition | Description |
| --- | --- |
| Shadowing | In generating a channel realization, the shadowing term of the channel model shall be set to 0dB. |
| Channel Model Breakpoint | When creating a channel realization between two STA, the distance between the STA is used to select between LOS and NLOS models according to the breakpoint distance defined in the channel model [5]. |

## Scenario 1

Use channel model B for this scenario.

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 19.2 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA3 | 24 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA4 | 4 Mbps |  | Constant, UDP | 1500 |  | 200 |  | SDTV |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA7 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA8 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

## Scenario 2

Use channel model B for this scenario.

| STA Name: STA1 |  | Role: DV Audio/Video sink |  |  | Location: 4, -4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| Note: this STA is a sink only, no applications generate data from this STA. |  |  |  |  |  |  |  |  |

## Scenario 3 (Large Enterprise)

Use Channel model D for this scenario.

Note, items marked "**" are not strictly specified by the Usage Model, but are a reasonable extrapolation intended to achieve a range of offered loads.

| STA Name: STA1 |  | Role: |  |  | Location: 5, -9.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 0.256 Mbps |  | TCP | 64 |  |  |  | ** Clicking on web link? |

## Scenario 4 (Conference Room)

Use channel model C for this scenario.

| STA Name: STA1 – STA20 |  | Role: laptop (local file download) |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

## Scenario 6: Hot spot

Use channel model E for this scenario.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1-10 (replicated) | 2 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA11-14 (replicated) | 2 |  | UDP | 512 |  | 200ms |  | Mid quality streaming audio/video |
| STA15-17 (replicated) | 8 |  | UDP | 512 |  | 200ms |  | High quality streaming audio/video |
| STA18-19 (replicated) | 5 |  | UDP | 1500 |  | 200ms |  | SDTV broadcast |
| STA20-34 (replicated) | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

## Scenario 7 (Mixed-Mode BSS)

Use channel model D for this scenario

Note, legacy STA means .11g in the 2.4GHz band and .11a in the 5GHz band as appropriate.

# Simulation Scenarios related to Comparison Criteria

The simulation scenarios in this section are referenced from the comparison criteria document [6].

## Scenario 16 (Point-to-Point Throughput Test)

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

This simulation is to be repeated using channel models B and D.

The simulation is repeated with the STA at locations in the range (0,0 to 0,200).

The UDP delay is not relevant, it can be considered to be infinite.

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA | 400Mbps |  | UDP | 1500 |  | N/A |  |  |

| STA Name: STA |  | Role: HI Sink |  |  | Location: (varies, see text) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

## Scenario 17 (Point-to-Point HT Goodput Test)

Use channel model B for this scenario.

The channelization for this scenario is 20 MHz. Goodput is measured.

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this AP is a sink only, no apps generate data from this STA |  |  |  |  |  |  |  |  |

| STA Name: STA |  | Role: HT UDP Source |  |  | Location: (0, 10) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

# References

[1]	Add References here

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.

This condition is specified because the shadowing term is a slowly varying random variable.  It varies slowly compared to realistic simulation durations (i.e. 10s).  This simulation specification has two alternatives to incorporate this effect properly: perform a large number of simulation runs and average the results or incorporate a static shadowing term.  It is not feasible to multiple the number of simulations by a large number, so shadowing is viewed as a static property of the simulation scenario that is already incorporated into the topology of the scenario (i.e. randomized placement of the STA).