IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | doc.: IEEE-15-06-0055-02-003c (UMD), Draft |  |
| Date Submitted | [18Jan06] |  |
| Source | [Ali Sadri] [Intel Corporation, 13290 Evening Creek Drive ,San Diego , CA 92128-3419 ,USA ] | Voice: [+1 858-774-6202] FAX: [] E-Mail: [ali.s.sadri@intel.com] |
| Re: | Move reference to reference section |  |
| Abstract | [802.15.3c Usage Model Document] |  |
| Purpose | This document defines usage models for 802.15.3c, The UMD, or Usage Model Document, defines the standard’s features, and all other elements which must be defined to enable standard success in the marketplace. The UMD is the guide for the Technical Requirements, and to generate simulation results for specified well-defined simulation scenarios provided by the Selection Criteria and Channel Modeling documents.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Authors/Contributors:

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Ali Sadri | Intel Corporation | Intel Corporation, 13290 Evening Creek Drive ,San Diego , CA 92128-3419 ,USA | 858-774-6202 |  | ali.s.sadri@intel.com |
| Alireza Seyedi | Philips | Philips, 345 Scarborough Rd., Briarcliff Manor, NY, 10510 | 914-945-6318 |  | Alireza.seyedi@philips.com |
| Tony Pollock | National ICT Australia Limited | NICTA, Level 2, Nouvelle House, 216 Northbourne Ave, Braddon ACT 2612, Australia | +61-2-6125-3797 |  | tony.pollock@nicta.com |
| Kazuaki Takahashi | Panasonic | 4-12-4, Higashi-Shinagawa, Shinagawa-ku, Tokyo 140-8507, JAPAN | +81-6710-2029 |  | takahashi.kazu@jp.panasonic.com |
| Raymond Yu Zhan | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 |  |  | Raymond.Yuz@sg.panasonic.com |
| Ichihiko TOYODA | NTT Network Innovation Laboratories | 1-1 Hikarinooka, Yokosuka, Kanagawa 239-0847, Japan | Phone: +81-46-859-2366 |  | toyoda.ichihiko@lab.ntt.co.jp |
| Abbie Mathew | NewLANS | 43 Nagog Park Suite 200 Acton, MA 01720 | 617-283-1363 |  | abbie.mathew@verizon.net |
| Dr. E. Grass | IHP-GmbH | Im Technologiepark 25 D-15236 Frankfurt (Oder) Germany | Tel: +49 335 5625 731 | Fax: +49 335 5625 671 | Eckhard Grass [grass@ihp-microelectronics.com] |

Abstract

This document defines usage models for 802.15.3c, The UMD, or Usage Model Document, defines the standard’s features, and all other elements which must be defined to enable standard success in the marketplace. The UMD is the guide for the Technical Requirements, and to generate simulation results for specified well-defined simulation scenarios provided by the Selection Criteria and Channel Modeling documents.]

NEED TO UPDATE AS WE COMPLETE THE DOCUMENT

# Revision History of Document 15-06-0055

| ### Revision | ### Comments | ### Date | ### Author |
| --- | --- | --- | --- |
| R0 | Initial version of the Usage Model Document. Changed name from MRD to UMD IEEE 802.15-06-0055-00-003c | January 18, 2006 | Ali Sadri, Ian C. Gifford |
| R1 | Add Draft to Usage Model Document (UMD) to the name IEEE 802.15-06-0055-01-003c | January 18, 2006 | Ali Sadri |
| R2 | Add Contributors list | February 1, 2006 | Ali Sadri |
| R3 | Consolidate comments from A. Seyedi, T. Pollock and K. Takahashi | February 8, 2006 | Ali Sadri |
| R4 | Consolidate comments from T. Pollock and I. TOYODA | February 15, 2006 | Ali Sadri |
| R5 | Consolidate comments from CFA doeument in to usage models, Include comments from Abbie and Raymond | March 1, 2006 | Ali Sadri |
| R6 | Incorporated the applications tables and converged in to one table. Add new table to include the MAC systems attribute, expand the simulations scenarios, Updated applications parameter definition table. | March 9, 2006 | Ali Sadri |
| R7 | Incorporated comments from Eckhard Grass on the MAC attributes for Application #29 Revise table 4, applications, and combine several applications in to similar categories. The same category applications are color coded similarly | March 22, 2006 | Ali Sadri |
| R8 | Regroup Applications, Re-label applications to new applications name, add applications parameter definition, The new Application list is reduced to 7 applications, A1-A7. The description of the applications is the Use Cases for the applications. | March 29, 2006 | Ali Sadri |
| R9 | Agreed to keep the application as A1-A7, Created sample Use Case for HD streaming and Wireless Desktop | April 13, 2006 | Ali Sadri |
| R10 | Update Contributor list with Abbie Mathews contact information, Added use case for video streaming | April 26, 2006 | Ali Sadri |
| R11 | Reformat the Use Case table 6 and add new use cased. The new table includes the attributes of the use case, including separation, motion, TX and RX location and data content | May 10, 2006 | Ali Sadri |
| R12 | Updated applications table, use cases table | May 17, 2006 | Ali Sadri |
| R13 | Ranking Use case table, Ranking available in a separate table | May 17, 2006 | Ali Sadri |
| R14 | Remove “Multiple Stream” definition from applications table | May 18, 2006 | Ali Sadri |

# Introduction

To support the definition of the mmWave higher throughput WPAN standard (which will incorporate changes to PHY and MAC if needed) within the IEEE, this document attempts to define usage models based on various market-based use-cases. The usage models are intended to support the definitions of simulations that will allow 802.15 members to evaluate the performance of various proposals in terms of, for example, network throughput, delay, packet loss and other metrics. It is intended that the outputs of this document will aid in the subsequent development of the evaluation and selection criteria used by 802.15.3c.

Note - These usage models that the usage model committee develops here are subject to the following constraints:

C1:	They are relevant to the expected uses of the technology

C2:	They require higher throughput than can be achieved with existing 802.15 and similar technologies

C3:	They are capable of being turned into an unambiguous simulation scenario

# Process going forward

The 802.15.3c Usage Model Document committee has been given responsibility for maintaining this document.

# Definitions

This section defines some of the terms used in this document.

Application – A source or sink of wireless data that relates to a particular type of user activity.

Examples: Streaming video, Wireless Display, High Capacity disc drive synchronization.

Environment – The type of place where a WPAN, or short range wireless communications system, is deployed.  Examples: residential, office, desktop.

Use Case – A use case is a description of how an end user uses a system that exercises that system’s deployment of a WPAN.  A use case includes an Application in a deployment Environment with details regarding the user activity and both sides of the link and the range at which the application should operate.

Examples: Watching a television physically remote from the cable or set-top box within the home.

Usage Model – A specification of one or more applications and environments from which a simulation scenario can be created once the traffic patterns of the applications are known.  Usage Models are created to "cover" Use Cases.

Simulation Scenario – A simulation scenario is a description of a usage model that supports simulation.  A simulation scenario includes details needed for simulation.  Types of details to be included are descriptions that link the usage model to the simulation scenario:

- Environment linked to a channel model,

- Position of the transmitter and receiver,

- Data traffic for all applications,

- Number of users on the same WPAN channel.

A Simulation Scenario is created from a Usage Model by characterizing the traffic profile of the Applications and possibly of merging multiple Applications together to reduce simulation time.

# Mappings between Application, Environment, Channel Model, Use case, Usage Model and Simulation Scenario

Understanding and defining the Application, Environment, and channel model, Use Case, Usage Model and Simulation Scenario are all necessary to create comparative results from 802.15.3c proposals.

Channel models have been defined in [xyx], with YY channel models.

Each use case involves the use of one or more Applications and is defined for one or more Environments.  It represents a single type of use of a system using the technology.

Each Application reflects a source or sink of data.   They will eventually be characterized in terms of a traffic profile that allows a simulation of the Application to be created.

Each Usage Model contains a representative mixture of Applications and channel models designed to adequately cover the selected Use Cases.  There is a many to many mapping between Use Cases and Usage Models (i.e., the same Use Case may contribute to multiple Usage Models and the same Usage Model may include Applications from multiple Use Cases).

There will be a one-to-one mapping between Usage Models and Simulation Scenarios.   The usage model is a marketing-oriented description of a "reasonable mixture" covering the important use cases.  The simulation scenario fills in any technical details necessary to fully define the simulation inputs not present in the usage model.

# Environments

Following is the list of environment and its definitions and how it maps to the channel models.

Table 1 - Environment to Channel Model Mapping

| Channel Model | Scenario | Environment Name | Environment Definition |
| --- | --- | --- | --- |
| CM1 | LOS | Office | Typical office setup furnished with multiple chairs, desks, computers and work stations. Bookshelves, cupboards and whiteboards are also interspersed within the environment. The walls are made by metal or concrete covered by plasterboard or carpet with windows and door on at least one side of the office. Cubical, laboratory, open and closed office can be treated as a generic office. Typically these offices are linked by long corridors. |
| CM2 | NLOS |  |  |
| CM3 | LOS | Desktop | Typical office desktop and computer clutter. Partitioning surrounded this environment |
| CM4 | LOS | Residential | Typical home with multiple rooms and furnished with furniture, TV sets lounges, etc. The size is comparable to the small office room. The walls/floor are made of concrete or wood covered by wallpaper/carpet. There are also windows and wooden door in different rooms within the residential environment. |
| CM5 | NLOS |  |  |
| CM6 | LOS | Conference Room | Typical conference room with metal shelves, white board and office window. |
| CM7 | NLOS |  |  |
| CM8 | LOS | Corridor | Passage way that links different offices |
| CM9 | LOS | Library | Typical small size library with multiple desks, chairs and metal bookshelves. Bookshelves are filled with books, magazines, etc. Some tables and chairs were interspersed between the bookshelves. At least one side of room has windows and/or door. The walls are made of concrete. |
| CM10 | NLOS |  |  |

# Applications

Table 3 lists the applications that are referred to from the usage models, together with relevant traffic parameters.

The parameters used to define the application are defined in Table 2.

Table 2 - Application Parameter Definitions

| Parameter | Definition |
| --- | --- |
| MSDU size | Packet size at the top of the MAC |
| Maximum PER | Maximum packet error rate at the top of the MAC. This is defined by the error rate that can be tolerated by the application. |
| Maximum Delay | Maximum transport delay at the top of the MAC |
| Data Type | Indicates the data type of the applications. It takes one of two values: asynchronous or isochronous. These two types are intended to represent a request for a total amount of channel time and a request for channel time on a periodic basis that does not expire. |
| Asymmetry | Yes, NO. Asymmetric applications typically have significantly different offered loads in different direction. e.g. Wireless HDMI |
| Maximum Offered Load | Data rate required by applications. |

Table 3 - Application Definitions

| # | Application |  | MAX Load (Gbps) |  | Data Type |  | MSDU (B) | PER/ BER | Delay (ms) | Asymmetry | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1.0 |  | HD Video Streaming |  | 0.05-3.0 |  | I | ? | ?? BER 10-11 | ? | Yes |  |
| A1.1 |  |  |  | 0.05-2.5 |  |  | ? | ? | ? |  |  |
| A1.2 |  |  |  | 0.1-3.2 |  |  |  |  |  |  |  |
| A1.3 |  |  |  | 1 |  |  |  |  |  |  |  |
| A1.4 |  |  |  | 1 |  |  |  |  |  |  |  |
| A2.0 |  | File Transfer |  | 0.1-2.0 |  | A | 1500 Octets | BER 10-6 | N/A | No |  |
| A2.1 |  |  |  | 0.1-2.0 |  |  | ? | ? | ? |  |  |
| A2.2 |  |  |  | 0.1-3.2 |  |  |  |  |  |  |  |
| A3 | Wireless Docking station and Desktop P2MP |  | 0.05-4.0 |  | I/A |  | ? | For I same as A1 For A, same as A2 | For I same as A1 For A, same as A2 | Yes |  |
| A4 | Gaming |  | 0.05-1 |  | I |  | ? | ? | 25ms | No |  |
| A5 | Shortrange Backhaul |  | 0.156-2.5 |  | A, clarify |  | 1500 Octets | Same as A2 | Low need a # | No |  |
| A6 | Ad-hoc |  | 0.1-1.6 |  | A |  | 1500 Octets | Same as A2 | ?? | No |  |
| A7 | Wireless Gb Ethernet |  | 0.5-1.0 |  | A |  | 1518 Octets | Same as A2 | Same as A5 | No |  |

Table 4.1- MAC related Systems Attributes

| # | Usage | MAC Related Systems Attributes |
| --- | --- | --- |
| 1 | Uncompressed HDTV Video/Audio streaming [DVD players and other power-line operated devices] | - Isochronous - High throughput efficiency - Point-to-Point - Support for high gain antennas for Data Transmission - Device discovery - Moderate latency - Minimum reserved bandwidth |
| 2 | HDTV Video/Audio streaming [video camera/mobile devices and other battery operated devices] | - Isochronous - High throughput efficiency - Maintain link throughput while in motion jitter - Point-to-Point - Support for moderate gain antennas for Data Transmission - Automatic device discovery - Moderate latency - Minimum reserved bandwidth - Multiple nearby data transmissions |
| 3 | Internet bulky music and video downloading [computing devices] | - Asynchronous - High throughput efficiency - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) |
| 4 | Internet bulky music and video downloading [mobile devices] | - Asynchronous - High throughput efficiency - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) - Multiple nearby data transmissions - Power saving mode |
| 5 | Internet small size file transfer (email, web, chat) | - Asynchronous - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) - Multiple nearby data transmissions - Power saving mode - Fast connect |
| 6 | Local file transfer for printing, document and small size file | - Asynchronous - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) - Multiple nearby data transmissions - Power saving mode |
| 7 | Local file transfer for bulky music and video, point-to-point connection (photo/video camera and photo/video handy phone, mp3 player) | - Asynchronous - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) - Multiple nearby data transmissions - Power saving mode |
| 8 | Wireless docking station | - Isochronous - High throughput efficiency - Maintain link throughput while in motion jitter - Point-to-Point - Support for moderate gain antennas for Data Transmission - Device discovery (Automatic preferred) - Moderate latency - Minimum reserved bandwidth - Multiple nearby data transmissions |
| 9 | Wireless display at projector and TV |  |
| 10 | Interactive Gaming [Console to Internet Access] *NOTE : Depends on Game Type |  |
| 11 | Interactive Gaming [between mobile devices] |  |
| 12 | Wireless display at projector long distance, range of display resolutions |  |
| 13 | 1Gigabit Ethernet link - LOS |  |
| 14 | Vertically connected wireless link Outdoor |  |
| 15 | Wireless IEEE1394 applications - LOS |  |
| 16 | Ad hoc information distribution system |  |
| 17 | Multimedia, information distribution system |  |
| 18 | - Outdoor: Fixed wireless access, distribution in stadiums, inter-vehicle communication, etc. - Indoor: Connecting multimedia devices (wireless home link), ad-hoc meeting, heavy content download, distribution system |  |
| 19 | Small office/meeting scenario, general office applications |  |
| 20 | Outdoor, Distribution links in apartments, stadium, etc. LOS Point-To-Point |  |
| 21 | Ad hoc network |  |
| 22 | Wireless home video server connected to HDTV, PC and other video devices |  |
| 23 | Wireless IEEE1394 applications |  |
| 24 | - Outdoor: Distribution links in apartments, stadium, etc. Indoor: Ad-hoc network |  |
| 25 | PowerPoint and such applications |  |
| 26 | Two way vertical wireless link |  |
| 27 | Wireless Gigabit Ethernet applications |  |
| 28 | - Replacement for 1394 FireWire - Replacement for USB Military – future combat systems, secure communication |  |
| 29 | Video supply, Environment bus, train, airplane ( | Isochronous High throughput efficiency Broadcast, multicast and unicast capable Minimum reserved bandwidth QoS support Low delay jitter (<100 ms) Moderate latency (< 100 ms) Support of asymmetric data rates and gaming Cumulative acknowledgements and unacknowledged streaming Infrastructure mode (non ad-hoc) Secondary (fallback) PHY for 100% coverage and uplink |
| 30 | 7.1 Surround sound multiple end points |  |

# Use Cases

Table 5 contains a definition of the use cases used in this document.

Table 5 - Use Case Definitions

| U# | Application | Covered by Channel Model # | Separation (m) | LOS/NLOS | TX Position | RX Position | Motion | Content | Score 3= high 2=mid 1=Low |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U1 | A1 Uncompressed Video Streaming | Residential | 5-10 | There could be a wall between the TX and RX | The transmitter sits on a shelf near or under the STB or AV receiver There may be two units at each end spaced about 1m apart for diversity | The receiver sits somewhere near the TV There may be two units at each end spaced about 1m apart for diversity | Fixed, Link may be temporarily shadowed by Human Body | Uncompressed Video and Audio | 2.82 |
| U17 | A7 Gigabit Ethernet | Office Conference Room Library | 3-5m | LOS | Transmitter/receiver is and access point located near the ceiling | Transmitter/receiver is a portable device (e.g. laptop) located on a desk | Fixed | Ethernet traffic | 2.81 |
| U16 | A6 Ad-hoc | Office Conference Room Library Desktop | 3-5m | LOS | Transmitter/receiver is a portable device (e.g. laptop) located on a desk | Transmitter/receiver is a portable device (e.g. laptop) located on a desk | Fixed | Ethernet traffic | 2.70 |
| U5 | A2 File Transfer and synch | Office Desktop | 1-3 | NLOS / LOS | Transmitter next or close to the receiver 2 fixed device | Transmitter next or close to the receiver 2 fixed device | Fixed | Email, news, chat, printing, small and Large music and video contents and files downloading and synchronization | 2.64 |
| U6 | A2 File Transfer and synch | Desktop office | 1-3m | LOS/NLOS | The transmitter/receiver is a portable device (PDA or PMP) and is placed on the table in front of or next to the computing device | The transmitter/receiver is a PC placed on the desk. | Fixed | Email, news, chat, printing, small and Large music and video contents and files downloading and synchronization | 2.61 |
| U2 | A1 Uncompressed Video Streaming | Conference room | 10-20 | LOS | The transmitter is located on or near the presenter’s podium, It is connected to a laptop or a multimedia server | The receiver is mounted on or close to the ceiling and it (the receiver) is connected to a HD projector, which illuminates a large screen | Fixed, | Uncompressed Video and Audio | 2.57 |
| U3 | A1 Uncompressed Video Streaming | Residential | 3-5m | LOS/NLOS | The transmitter is a portable device (PDA or PMP) and is placed on the table in front of or next to the TV. | The receiver sits somewhere near the TV There may be two units at each end spaced about 1m apart for diversity | Fixed, Link may be temporarily shadowed by Human Body | Uncompressed Video and Audio | 2.52 |
| U11 | A3 Wireless Desktop | Desktop Office | 1-3 | NLOS | Transmitter in the drawer or in a cabinet beneath the monitor Receiver on the desk or next to the laptop | Same | Device is jittered during transmission such as handheld in hand or laptop on lap | Wide range of Data rates from BT rate to HD Streaming video | 2.52 |
| U7 | A2 File Transfer and synch | Office Conference Room * | 1-3m | LOS | The transmitter/receiver is a kiosk in a store | The receiver is a portable device (PDA, PMP, digital camera) placed next to the kiosk | Fixed | Large music and video contents and files | 2.50 |
| U9 | A2 File Transfer | Residential Office | 0.1-1 | LOS | TX/RX near AV recorder, Kiosk or associated with portable device. | TX/RX is associated with portable device | Fixed during transferring | Large/Small video contents and files downloading and uploading | 2.43 |
| U14 | A5 Short range Backhaul | Corridor | 20-30m | LOS | The transmitter and receiver are connected to the wall or ceiling in a corridor | The transmitter and receiver are connected to the wall or ceiling in a corridor | Fixed | Gig E or similar traffic | 2.42 |
| U10 | A3 Wireless Docking station | Office Desktop | 1-3 | NLOS | Transmitter on the desk | Receiver in the drawer | Fixed | Single high throughput connection from Laptop to the docking station | 2.31 |
| U4 | A1 Video Streaming | Residential | 0.1-1 | LOS | Transmitter near AV player or associated with portable device | Receiver is associated with portable device | Fixed during streaming | Video and Audio | 2.25 |
| U13 | A4 Mobile Gaming | Residential | 1-3m | LOS | Transmitter is a hand-held device (e.g. mobile phone) | The receiver is placed near a TV | Jittered (hand movement) | Interactive Video | 2.10 |
| U8 | A2 File Transfer | Office | 3-5m | LOS/NLOS | The transmitter is a PC or a portable device placed on a desk | The receiver is a printer located elsewhere in the room | Fixed | Files, “byte stream” | 2.07 |
| U15 | A5 Short range Backhaul | AWGN | 200-300 | LOS | Transmitter on Pole top or on a tower or outdoor on the wall Receiver on Pole top or on a tower or outdoor on the wall | Transmitter on Pole top or on a tower or outdoor on the wall Receiver on Pole top or on a tower or outdoor on the wall | Fixed, jitter due to wind | Gig E equivalent data Professionally installed | 2.00 |
| U12 | A4 Mobile Gaming | Residential Office | 1-3 | LOS | Transmit and receive are held closely to each other | Transmit and receive are held closely to each other | Jitter | low throughput Data with low latency | 1.61 |
| Use Case; High; Mid; Low; High; Mid; Low; Total; Average. 1; 10; 4; 0; 13; 1; 0; 28; 2.82. 17; 10; 3; 0; 11; 2; 0; 26; 2.81. 16; 11; 2; 0; 9; 4; 1; 27; 2.70. 5; 10; 4; 0; 9; 4; 1; 28; 2.64. 6; 10; 6; 0; 9; 6; 0; 31; 2.61. 2; 8; 6; 0; 9; 4; 1; 28; 2.57. 3; 8; 8; 0; 7; 6; 0; 29; 2.52. 11; 7; 7; 0; 9; 5; 1; 29; 2.52. 7; 7; 7; 1; 10; 4; 1; 30; 2.50. 9; 9; 5; 1; 6; 8; 1; 30; 2.43. 14; 8; 3; 2; 7; 4; 2; 26; 2.42. 10; 6; 6; 2; 8; 4; 3; 29; 2.31. 4; 9; 4; 1; 2; 9; 3; 28; 2.25. 13; 6; 6; 3; 4; 7; 4; 30; 2.10. 8; 4; 8; 2; 5; 5; 5; 29; 2.07. 15; 5; 2; 5; 5; 4; 5; 26; 2.00. 12; 0; 6; 8; 1; 9; 4; 28; 1.61. | Ranked Data |

# Usage Models

Table 6 defines the usage models defined by this document.

The purpose of these models is to merge representative use cases to create a small number of credible mixtures of applications.  The usage models have to be realistic in terms that they are covered by the use cases listed above and different from each other.

The number of usage models needs to be limited because each usage model adds simulation time to the preparation of results for submission against the 802.15.3c selection criteria.

Table 6 - Usage Model Definitions

| Model # | Usage Model | Covers Use Cases | Score (ave/devn) | Application mix | Comments |
| --- | --- | --- | --- | --- | --- |
| UM1 | Residential |  | 2.8/0.3 | 1 x AP STA 1: 1 Gbps near lossless compressed HDTV , OLOS STA 2: 1 Gbps near lossless compressed HDTV, OLOS Up to 5 Stations with different use case | This scenario should be room to room or indoor/outdoor. The exact spatial distribution and mobility as well as the desired number of simultaneous connections can be referred to [2]. 20m range. |
| UM2 | Residential IBSS | 4, 33, 11, 34, 15, 21 | 2.3/0.6 | 0 AP STA 1: peer-peer DV Audio/Video (A1) sink STA 2,3: local file transfer (A15) sink, local file transfer (A15) source STA 4,5,6,7: Video Phone (A23) sink, Video Phone (A23) source STA 8: 4 x Video gaming, controller to console (A16), sink STA 9: content download (A13) sink STA 10: peer-peer DV Audio/Video (A1) (rate, range combination > current technology) source STA 11,12,13,14: Video gaming, controller to console (A16) source | Note, all these devices are operating on the same channel. |

## Coexistence (Informative)

802.15.3c will liaise with 802.19 TAG regarding coexistence requirements.

# Simulation Scenarios

## Common Conditions

Table 1 defines conditions that are common to all simulation scenarios.

Table 7 - Common Simulation Conditions

| Condition | Description |
| --- | --- |
| Shadowing | In generating a channel realization, the shadowing term of the channel model shall be set to 0dB. |
| Channel Model Breakpoint | When creating a channel realization between two STA, the distance between the STA is used to select between LOS and NLOS models according to the breakpoint distance defined in the channel model [5]. |

## Scenario 1

Use channel model B for this scenario.

| STA Name: AP |  | Role: Access Point |  |  | Location: 0, 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1 | 19.2 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA3 | 24 Mbps |  | Constant, UDP | 1500 |  | 200 |  | HDTV |
| STA4 | 4 Mbps |  | Constant, UDP | 1500 |  | 200 |  | SDTV |
| STA4 | 1 Mbps |  | TCP | 300 |  |  |  | Internet file |
| STA7 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |
| STA8 | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

## Scenario 2

Use channel model B for this scenario.

| STA Name: STA1 |  | Role: DV Audio/Video sink |  |  | Location: 4, -4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| Note: this STA is a sink only, no applications generate data from this STA. |  |  |  |  |  |  |  |  |

## Scenario 3 (Large Enterprise)

Use Channel model D for this scenario.

Note, items marked "**" are not strictly specified by the Usage Model, but are a reasonable extrapolation intended to achieve a range of offered loads.

| STA Name: STA1 |  | Role: |  |  | Location: 5, -9.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  | Notes |
| AP1 | 0.256 Mbps |  | TCP | 64 |  |  |  | ** Clicking on web link? |

## Scenario 4 (Conference Room)

Use channel model C for this scenario.

| STA Name: STA1 – STA20 |  | Role: laptop (local file download) |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

## Scenario 6: Hot spot

Use channel model E for this scenario.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1-10 (replicated) | 2 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA11-14 (replicated) | 2 |  | UDP | 512 |  | 200ms |  | Mid quality streaming audio/video |
| STA15-17 (replicated) | 8 |  | UDP | 512 |  | 200ms |  | High quality streaming audio/video |
| STA18-19 (replicated) | 5 |  | UDP | 1500 |  | 200ms |  | SDTV broadcast |
| STA20-34 (replicated) | 0.096 Mbps |  | Constant, UDP | 120 |  | 30 |  | VoIP |

## Scenario 7 (Mixed-Mode BSS)

Use channel model D for this scenario

Note, legacy STA means .11g in the 2.4GHz band and .11a in the 5GHz band as appropriate.

# Simulation Scenarios related to Comparison Criteria

The simulation scenarios in this section are referenced from the comparison criteria document [6].

## Scenario 16 (Point-to-Point Throughput Test)

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

This simulation is to be repeated using channel models B and D.

The simulation is repeated with the STA at locations in the range (0,0 to 0,200).

The UDP delay is not relevant, it can be considered to be infinite.

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA | 400Mbps |  | UDP | 1500 |  | N/A |  |  |

| STA Name: STA |  | Role: HI Sink |  |  | Location: (varies, see text) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources – none, this STA acts as a sink only |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |

## Scenario 17 (Point-to-Point HT Goodput Test)

Use channel model B for this scenario.

The channelization for this scenario is 20 MHz. Goodput is measured.

Unlike other UDP sources, these UDP sources have no timeout value specified (it can be considered to be infinite).

| STA Name: AP |  | Role: HT AP |  |  | Location: (0, 0) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| Note: this AP is a sink only, no apps generate data from this STA |  |  |  |  |  |  |  |  |

| STA Name: STA |  | Role: HT UDP Source |  |  | Location: (0, 10) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP | 100 Mbps |  | UDP | 1500 |  |  |  |  |

# References

[1]	IEEE P802.15-05-0353-07-003c

Appendix:

|  |  | Data Rate With TMDS (Gbps) | Applications |
| --- | --- | --- | --- |
| 1 | 1280 x 720 p, 24 Hz | 0.7810 | US HDTV |
| 2 | 1280 x 720 p, 30 Hz | 0.9763 | US HDTV |
| 3 | 2880 x 576 i, 50 Hz | 1.5457 | Game consoles |
| 4 | 2880 x 288 p, 50 Hz | 1.6165 | Game consoles |
| 5 | 1440 x 480 p, 59.94 Hz | 1.6200 | High end DVD players |
| 6 | 1440 x 576 p, 50 Hz | 1.6200 | High end DVD players |
| 7 | 1440 x 480 p, 60 Hz | 1.6216 | High end DVD players |
| 8 | 2880 x 240 p, 59.94 Hz | 1.6231 | Game consoles |
| 9 | 1920 x 1080 i, 50 Hz | 2.1830 | EU HDTV |
| 10 | 1920 x 1080 i, 60 Hz | 2.1830 | US HDTV, EU HDTV |
| 11 | 1280 x 720 p, 50 Hz | 2.2275 | EU HDTV |
| 12 | 1280 x 720 p, 60 Hz | 2.2275 | US HDTV, EU HDTV |
| 13 | 1920 x 1080 p, 24 Hz | 2.2275 | US HDTV |
| 14 | 1920 x 1080 p, 30 Hz | 2.2275 | US HDTV |
| 15 | 1920 x 1080 p, 50 Hz | 4.4550 | NEXT GEN |
| 16 | 1920 x 1080 p, 60 Hz | 4.4550 | NEXT GEN |

This condition is specified because the shadowing term is a slowly varying random variable.  It varies slowly compared to realistic simulation durations (i.e. 10s).  This simulation specification has two alternatives to incorporate this effect properly: perform a large number of simulation runs and average the results or incorporate a static shadowing term.  It is not feasible to multiple the number of simulations by a large number, so shadowing is viewed as a static property of the simulation scenario that is already incorporated into the topology of the scenario (i.e. randomized placement of the STA).