IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | A simplified test for Doppler and Angular Velocity |  |
| Date Submitted | March 2006 |  |
| Source | Richard Roberts Intel Corporation | Voice: 503-712-5012 FAX: [] E-Mail: richard.d.roberts@intel.com |
| Re: |  |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

A simplified Doppler test

We graphically show the general Doppler scenario below.

-Do+vt

L

R

-Do

Do

v

R

source

mobile

0

We notice that as the mobile moves from point -Do to Do, the angular slant range R between the mobile device and the source is changing in a nonlinear manner where

v = velocity of the observer device

R = slant range between the mobile and source

-Do = the starting point

t = time

L = standoff distance.

The Doppler shifted frequency for a moving mobile and a stationary source is given by

f=f01+vRc

where

vR = relative slant range velocity between the mobile and the source

c = speed of light

f0 = nominal center frequency

f = Doppler frequency

We can derive vR as

- R=L2+−D0+vt212

- vR=dRdt=v−D0+vtL2+−D0+vt2−12

where 0≤t≤2⋅D0v.

The Doppler shift is then given as

- Δf=f01+vRc−f0=f0vRc

- Δf=f0v−D0+vtcL2+D0−vt2−12

Numerical Examples

Example 1:  1 meter standoff

Determine the Doppler shift for the case where

D0 = 10 meters

v = 2 m/s

L = 1 meters

f0 = 60 GHz

Example 2:  3 meter standoff

Determine the Doppler shift for the case where

D0 = 10 meters

v = 2 m/s

L = 3 meters

f0 = 60 GHz

Suggested Simplified Simulation Test for Doppler

While this test does not reflect any particular real physical deployment, it does provide a simulation test environment for relative comparison of PHY proposals.

1. Assume an AWGN channel with an Eb/No of TBD.

2. Establish a continuous packet exchange between the source and the mobile

3. In the simulation environment, mathematically vary the source carrier frequency according to the equation

Δf=f0v−D0+vtcL2+D0−vt2−12, 0≤t≤2⋅D0v

with the parameters

D0 = TBD meters

v = TBD m/s

L = TBD meters

f0 = 60 GHz

4. Record the impact on the performance (either BER or PER … TBD).

A simplified angular velocity test

In conjunction with the Doppler test, we can also simultaneously show that the antenna pointing algorithms are tracking the time varying angle of arrival.

-Do+vt

L

R

-Do

Do

v

R

source

α

α

The angle of arrival at the mobile is given as

α=arctanL−D0+vt

and the angular velocity (rads/sec) is given as

dαdt=11+−D0+vt2, 0≤t≤2⋅D0v.

Example 3 – 1 meter standoff

Determine the time varying angle and the angular velocity for the case where

D0 = 10 meters

v = 2 m/s

L = 1 meters

f0 = 60 GHz

Suggested Simplified Simulation Test for Angular Velocity

While this test does not reflect any particular real physical deployment, it does provide a simulation test environment for relative comparison of PHY proposals.

1. Assume an AWGN channel with an Eb/No of TBD.

2. Establish a continuous packet exchange between the source and the mobile

3. In the simulation environment, mathematically vary the angle of arrival according to the equation

α=arctanL−D0+vt, 0≤t≤2⋅D0v

with the parameters

D0 = TBD meters

v = TBD m/s

L = TBD meters

4. Record the impact on the performance (either BER or PER … TBD).