IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3c Channel Modeling Sub-committee Report (Draft) |  |
| Date Submitted | [20 March 2006] |  |
| Source | [Su-Khiong Yong] [Samsung Advanced Institute of Technology] [P.O. Box 111, Suwon 440-600, Korea.] | Voice: [+82-31-280-9581] Fax: [+82-31-280-9555] E-mail: [su.khiong.yong@samsung.com] |
| Re: | [IEEE 802.15.3c Channel Model] |  |
| Abstract | [This is a discussion document for the IEEE document of the IEEE 802.15.3c channel modeling subgroup. It provides models for the following frequency ranges and environments: for 60GHz channels covering the frequency range from 57 to 66 GHz, it covers indoor residential, indoor office and library environments (usually with a distinction between LOS and NLOS properties). The document also provides MATLAB programs and numerical values for 100 impulse response realizations in each environment] |  |
| Purpose | [The purpose of this report is to summarize the work of the channel modeling sub-committee and provide some final recommendations on how the channel model can be used to help evaluate PHY submissions to IEEE 802.15.3c.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE 802.15.3c Channel Modeling Sub-committee Report (Draft)

Date: 20 Macrh, 2006

Revision History of Final Recommendations

| Revision Number | Date | Comments |
| --- | --- | --- |
| 0.0 | 20/03/2006 | Creation of first version of Draft recommendations |

Table of Contents

1.	Introduction	4

2.	Environments	5

3.	Large Scale Channel Characterization	6

3.1	Path Loss	6

3.2	Shadowing	7

4.	Small Scale Channel Characterization	7

4.1	Generic Channel Model	7

4.2	Power Delay Profile	8

4.3	Power Azimuth Profile	8

4.4	Small Scale Fading Statistics	8

4.5	Polarization	8

5.	60GHz Model Parameterization	8

6.	Summary and Conclusion	8

7.	List of Contributors	8

Appendix	9

A	Matlab Program for Generation of Channel Impulse Response	9

B	Measurement Setups and Procedures	9

C	Data Post-Processing and Analysis	9

D	Extraction of SV Parameter	9

8.	References	9

# Introduction

This document summarizes the activities and recommendations of the channel modeling subgroup of IEEE 802.15.3c. The Task Group 802.15.3c (TG3c) is aimed to develop a millimeter-wave based alternative physical layer for the existing 802.15.3 Wireless Personal Area Network (WPAN) Standard 802.15.3-2003.

In order to evaluate the performance of different physical (PHY) layer proposals, a commonly agreed upon channel models is a must. However, there is no well-known millimeter-wave channel model available at the time the sub-group was formed, that could benefit the use of antenna arrays as well as fit firmly into the environments defined in response to the Call for Applications (CFA) and Usage Model Document (UMD) [1]. The main goal of the newly developed channel models is to allow a fair comparison of different proposals submitted to TG3c in response to the Call for Proposals.

Since the sub-group was formed, a numerous of channel modeling related documents has been presented and discussed at the IEEE 802 meetings and weekly teleconference calls. During the establishment of the channel model, the sub-committee encountered a number of challenges such as time constraint and limited resources. Despite of significant efforts have been carried out to make models as realistic as possible, the number of available measurements on which the model can be based in the 57-64 GHz range as well as the number of available measurement data, are insufficient to fully characterize the underlying environments. Therefore, it was inevitable to do some (over) simplifications that affect the absolute performance, but not the relative behavior of the different proposals.

All the models presented and submitted as recommendation in this document are based on measurements conducted in several environments [2]-[6]. The generic structures of these millimeter-wave models are derived based on the clustering model that characterizes both the large and small scale fading (attenuation and dispersion). The large scale fading includes path loss and shadowing while the small scale fading describes the power delay profile, power azimuth spectrum and amplitude fading statistics.

All the models are continuous in time while the temporal discretization (which is required for any simulation) is left to the implementer. To facilitate the use of the model, this document also includes a MATLAB program for the generation of channel impulse responses (CIR). A set of stored CIR in the form of MATLAB format (.mat) and Excel tables (.xls) for each channel model is provided. The use of these stored CIRs is mandatory for the simulations to ensure consistent and fair comparison of systems submitted to 802.15.3c.

The remainder of the document is organized as follows: Section 2 gives an overview of the considered environments; Section 3 presents a generic channel model as well as the definitions of the channel parameters that will be used in later sections. Section 4 lists all the parameterizations for the considered channel models. A summary concludes the report. Appendix A contain a summary of all measurement documents and proposals presented to the group; a MATLAB program for the generation of impulse responses, can be found in Appendix B, and general procedures for the measurement and the evaluation of the data, as recommended by the modeling subgroup are contained in Appendix C.

# Environments

From the CFA and UMD [1], a list of environments can be identified in which IEEE 802.15.3c devices should be operating. Due to the resources constraint, only xx environments will be characterized in this report by the sub-committee. Table 1 summarizes the considered environments with their respective typical layouts, settings and descriptions.

| Environment | Layout | Description |
| --- | --- | --- |
| Office - Desktop - Corridor - Closed - Open - Cubical |  | - Typical office desktop and computer clutter - Corridor - Single office with desk and bookshelf clutter - Large office with multiple desk/work stations and typical office clutter (bookshelves, filing cabinets - Large office with typical cubicle workstations |
| Conference Room |  | Large office with large table and multiple chairs |
| Library |  | Typical library environment with tables, chairs and metal bookshelves with books |
| Residential |  | Typical family room with TV, lounges, cabinets, coffee tables, bookshelves, etc. |

Table 1: List of the environments under consideration by the channel modeling sub-committee.

The environments listed in Table 1 are not comprehensive given that the broad applications envisaged by the millimeter-wave technology.

Discussion Point: We need to fix the number of environments given that the measurement data we have. The pros and cons of each of the measurement such as bandwidth used, measurements points taken, AoA information, resolution etc can be taken into account. Based on what we have now, we can have 14-16 channel models for both LOS and NLOS cases.

IMST

Library Environment (12.59 m x 5.12 m x 2.6 m)

The measurements have been performed within the IMST premises in a library room of size. The room was equipped with metal bookshelves of 2 m height and 2.5 m to 5 m length, which were positioned along the walls or standing freely in the room. Bookshelves were almost completely filled with books, magazines, etc. Some tables and chairs were interspersed within the shelves. One long side of the room was made of concrete and had windows.

NICTA

Office Environnent (12.5m x 22m x 3.5m)

Office with steel wall, steel ceiling and steel floor covered. The floor and the ceiling are covered with carpet and plaster board, respectively. Plate glass windows are attached on one side of the wall. The office is filled with multiple desks and chairs, computers and cupboards.

Home Environnent (6.85m x 3.57m x 2.47m)

Empty room with surface of the wall and ceiling are covered with wallpaper. Window is plane glass, wooden door and the floor is made by lumber.

NICTA

Still waiting detail from Tony

FRANCE TELECOM

- Residential environment (100 m² apartment)

- The apartment had a rectangular shape, with a L-shaped corridor separating different rooms. Internal walls of this residential flat were made of 7 cm thick plasterboard with 3 cm thick wooden doors. Outer walls were made of 44 cm thick concrete with double-glazing. Ceiling and floor were made of concrete. Most of the rooms in the apartment were equipped with wooden furniture (tables, chairs, bed, shelves, etc.)

- Office environments

Consists three types of rooms: an open-plan office, a corridor and a conference room.  Pascal will update later.

UMASS

Still waiting detail from Abbie

IBM

Office, library/lab environments

The walls were made of metal, plasterboard or brick.

Residential home

The walls were made of wood/plasterboard.

| Data Source | Environment | scenario | Number of Local Measurement points per environment | Number of Spatial Measurement points per Local point | Center Freq | BW | Angular info. (Rotation/ Virtual Array) | TX Antenna Type | RX Antenna Type | Polarization |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NICT | Office (open), Residential Room (Empty) |  | 1 (Res) 1 (Office) | NA | 62.5 GHz | 3 GHz | Receiver Rotation (5° per rotation) | Omni, Horn (10 dBi, 16 dBi, 22 dBi) | Horn (22 dBi) | Vertical |
| NICTA | Desktop, Corridor, different-sized offices, labs, cubicles |  | 7 (Desktop) 4 (Corridor) 28 (Indoor: different-sized offices, labs, cubicles) | NA | 60 GHz | 10 GHz | Receiver Rotation (4º per rotation) | Omni | Direc (21 dBi) | vertical |
| IMST | Library |  | 9 (Library) | 501, 1001, 301x51; 1mm step | 59.5 GHz | 960 MHz | Virtual Array | Omni Lens (8 dBi at 760 from vertical) | Horn (20 dBi), Planar (22 dBi), Biconical (9dBi) | co- and crosspolar orientations |
| France Telecom | Residential (Cluttered), Office (closed, Corridor, Conference room) | LOS, NLOS | 26 (Res. LOS) 49 (Res. NLOS) 34 (Off. LOS) 12 (Off. NLOS) | 76 (Res.) 60 (Off.) | 60 GHz | 1024MHz, 512 MHz (some Off.) | Virtual Array (76 steps Res., 60 steps Off.) | Horns (72° 8 dBi, 10° 24.6 dBi) | Horns (60° 13 dBi, 10° 24.6 dBi) | Vertical |
| France Telecom (AoA) | Office | LOS, NLOS | 66 | 100 | 61 GHz | 1024MHz | Virtual Array (10x10, 0.4 lambda spacing)) | Omni (5.5 dBi), Horn (100°, 7.3 dBi) | Omni (5.5 dBi), Horn (100°, 7.3 dBi) | Vertical |
| IBM | Office, home, Library/lab |  | 513, 136, 117 |  | 61.5 GHz | 5GHz | No | Omni | Omni | Vertical |
| UMASS | Conference room Corridor Residential Office – cublicle type office |  | Measurement still on going – at least two per environment |  | 60 GHz | 1GHz | 182 rotational points | Directional, HPBW of 12º | Directional, Polarization, HPBW of 12º? | Circular, right hand |

# Large Scale Channel Characterization

## Path Loss

The path loss is defined as the ratio of the received signal power to the transmit signal power and it is very important for link budget analysis. Unlike narrowband system, the path loss for a wideband system such as UWB [7][9] or millimeter-wave system, is both distance and frequency dependent. In order to simplify the models, it is assumed that the frequency dependence path loss is negligible and only distance dependence path loss is modeled in this report. The path loss as a function of distance is given by

where (dB) is the average path loss and Xs is the shadowing fading, which will be described in Section 3.2 As summarized in [10], several distance dependence path loss modeling approaches were reported. The channel sub-group adapted the conventional way to model the average path loss as given by

where d0,  and d denote the reference distance, wavelength and distance, respectively. The path loss exponent n for millimeter-wave based measurements ranges from 1.2-2.0 for LOS and from 1.97-10 for NLOS, in various different indoor environments [10]. In the presence of wave-guiding effect, n can be smaller than 2. Table 2 summarizes the values of n for different environments and scenarios, obtained based on our measurement data. The PL exponent is obtained by performing least squares linear regression on the logarithmic scatter plot of averaged received powers versus distance to . The data was segmented into LOS and NLOS scenarios, respectively. The value of d0=1 is used in all of the cases as reference distance as listed in Table 2 while the value of  is computed using the mid band frequency point.

| Environment | Scenario | n |  | Comment | Reference |
| --- | --- | --- | --- | --- | --- |
| Office Desktop | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Office Closed | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Office Open | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Office Cubical | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Conference Room | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Corridor |  |  |  |  |  |
| Library | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |
| Residential | LOS |  |  |  |  |
|  | NLOS |  |  |  |  |

Table 2: The path loss exponent, n and standard deviation for shadowing, .

## Shadowing

Due to the variation in the surrounding environments, the received power will be different from the mean value for a given distance. This phenomenal is called shadowing which causes the path loss variation about the mean value given in . Many measurement results reported in the millimeter-wave range have shown that the shadowing fading is log-normal distributed i.e.

Xs[dB]=N(0, sL) where Xs denotes zero mean, Gaussian random variable in unit decibels with standard deviation sL. The value of sL is site specific as listed in Table 2 for different environments.

Discussion Point: So far we have only PL model from P. Pagani (15-06-0041-00-003c) and S. Emami (15-05-0601-00-003c and 15-06-0191-00-003c), can the others confirm me that whether a large scale characterization (including shadowing) will be performed or not, or if I have missed someone here?

# Small Scale Channel Characterization

## Generic Channel Model

Based on the clustering of phenomenon in both the temporal and spatial domains as observed in our measurement data [][][], a generic millimeter-wave channel model which takes clustering into account is proposed since it can always be reduced to conventional single cluster channel model as observed in [6][15]. The proposed cluster model is based on the extension of Saleh-Valenzuela model [11] to the angular domain by Spencer [12]. The channel impulse response (CIR) in complex baseband is given by

where is the dirac delta function, L is the total the total number of clusters and K is total number of rays in kth cluster. The scalars ,  and  and denote the complex amplitude, delay and azimuth of the kth ray of the lth cluster. Similarly, the scalars  and  represent the delay and mean AOA (MAOA) of the of the lth cluster. The key assumption used in arriving to equation  is that the spatial and temporal domains are independent and thus uncorrelated. However, measurement results in [13] have otherwise shown that there was a correlation between these two domains and was modeled using two joint probability density functions (pdf). It is also important to note that each of the multipath in  will experience distortions due to the frequency dependency of the scatterers [14] but this is not accounted in our model due to lack of information.

## Power Delay Profile

Will be updated when more analysis are available

## Power Azimuth Profile

Will be updated when more analysis are available

## Small Scale Fading Statistics

Will be updated when more analysis are available

## Polarization

# 60GHz Model Parameterization

# Summary and Conclusion

# List of Contributors

The following have participated in the 802.15.3c Channel Model Sub-Committee activities.

Chang-Soon Choi (presenter)

Chia-Chin Chong

Shahriar Emami (presenter)

Reed Fisher

Brian Gaucher (IBM measurement data)

Tian-Wei Huang (presenter)

Jurgen Kunisch (IMST measurement data)

Zhi-Guo Lai

Wei Li (presenter)

Abbie Mathew (presenter)

Hiroyo Ogawa (presenter)

Pascal Pagani (presenter)

Tony Pollock (presenter)

Rick Roberts (presenter)

Ali Sadri (presenter)

Hirokazu Sawada (NICT measurement data)

Alireza Seyedi

Yozo Shoji

Stan Skafidas (presenter)

Kai Siwiak (presenter)

Su-Khiong Yong (presenter)

# Appendix

## Matlab Program for Generation of Channel Impulse Response

## Measurement Setups and Procedures

## Data Post-Processing and Analysis

## Extraction of SV Parameter

# References

- A. Sadri, “802.15.3c Usage Model Document,” IEEE 802.15-06-0055-xx-003c, xxx 2006.

- P. Pagani et al., “Characterization and Modeling of the 60 GHz Indoor Channel in Office and Residential Environments,” IEEE 802.15-06-0041-01-003c, Hawaii, USA, Jan 2006.

- J. Kunisch et al., “MEDIAN 60 GHz wideband indoor radio channel measurements and model,” IEEE VTC’99, pp. 2393-2397, 1997.

- C. Liu et al., “NICTA Indoor 60 GHz Channel Measurements and Analysis update,” IEEE 802.15-06-0112-00-003c, Denver, USA, Mar 2006.

- H. Sawada et al., “NICT Propagation Data,” IEEE 802.15.06-0012-01-003c, Hawaii, USA, Jan 2006.

- T. Zwick et al., “Wideband Channel Sounder With Measurements and Model for the 60 GHz Indoor Radio Channel,” IEEE Trans. Vech. Technol. vol. 54, pp. 1266-1277, July 2005.

- A. Mathew et al., “Observations of the UMass Measurements,” IEEE 802.15.06-0137-00-003c, Denver, Mar 2006.

- A. F. Molisch et al., “IEEE 802.15.4a Channel Model Final Report,” IEEE 802.15-04-0662-01-004a, 2005.

- C. C. Chong et al., “Statistical characterization of the UWB propagation channel in indoor residential environment,” Wiley J. Wireless Commun. Mobile Comput. (Special Issue on UWB Communications), vol. 5 no. 5, pp. 503-512, Aug. 2005.

- S. K. Yong et al., “Generalization and Parameterization of the mmWave Channel Models,” IEEE 802.15-05-0261-01-003c, Cairns, Australia, May. 2005.

- A. Saleh and R. Valenzuela, “A Statistical Model for Indoor Multipath Propagation,”   IEEE J. Select. Areas Commun., Vol. SAC-5, No. 2, pp. 128-137, Feb. 1987.

- Q. Spencer et al., “Modeling the Statistical Time and Angle of Arrival Characteristics,”   IEEE J. Sel. Areas Commun., Vol. 18, No. 3, pp. 347-360, Mar. 2000.

- C. C. Chong et al, “A new statistical wideband spatio-temporal channel model for 5-GHz band WLAN systems,” IEEE J. Sel. Areas Commun., vol. 21, no. 2, pp. 139–150, Feb. 2003.

- R. C. Qiu et al., “Ultra-wideband for multiple access communications,” IEEE Commun. Mag., vol. 43, no. 2, pp. 80–87, Feb 2005.

- S. Emami et al., “Channel Model Based on IBM Measured Data,” IEEE 802.15-06-0191-00-003c.

- A. Sadri et al., “IMST Time Angular Characteristics Analysis,” IEEE 802.15-06-0141-01-003c, Denver, USA, Mar 2006.