# IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Task Group 4a for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | # Calculating PHY and MAC PIB Attributes for the CSS PHY |  |
| Date Submitted | 28 June 2006 (rev 0) |  |
| Source | [Lars Menzer] | Voice: [+49 30 399954-0] E-mail: [l.menzer@nanotron.com] |
| Re: | 802.15.4a Draft 2 Comment Resolution |  |
| Abstract | Formula for calculating attributes phySHRDuration, phyMaxFrameDuration, macAckWaitDuration depending the mandatory and the optional data rate for the CSS PHY. |  |
| Purpose | Resolve comments on Draft 2 regarding missing definition of macAckWaitDuration |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Calculating PHY and MAC PIB Attributes for the CSS PHY

6.4.2 Change rows in table Table 23—PHY PIB attributes

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
| phyMaxFrameDuration† | 0x05 | List of Integer | 53, 206, 184, 258, 712, 1032 | The maximum number of symbols in a frame. For all non-UWB and non-CSS PHY types a single value: = phySHRDuration + ceiling([aMaxPHYPacketSize + 1] * phySymbolsPerOctet) For UWB PHY types, see subclause 6.4.2.1 For CSS PHY list of two values. First value corresponds to the mandatory data rate (1Mbit/s), second to the optional data rate (250kbit/s). See subclause 6.4.2.2. |
| phySHRDuration† | 0x06 | List of Integer | 3, 7, 10, 12, 24, 40 | The duration of the synchronization header (SHR). For all non-UWB and non-CCS PHY types, a single value in symbols for the current PHY. For UWB PHY types For CSS PHY list of two values {12, 24} in symbols. First value corresponds to the mandatory data rate (1Mbit/s), second to the optional data rate (250kbit/s). |
| phySymbolsPerOctet† | 0x07 | List of Float | 0.4, 4/3, 1.6, 2, 32/6, 8 | The number of symbols per octet for the current PHY. For all non-CSS and non-UWB PHY types a single value. For UWB PHY types .. For CSS PHY list of two values {4/3, 32/6}. First value corresponds to the mandatory data rate (1Mbit/s), second to the optional data rate (250kbit/s). |

Add sub-clause to 6.4.2

6.4.2.1 ....UWB

      - PIB values phyMaxFrameDuration for CSS PHY

For the CSS PHY type, the values of this read-only attribute depend on the selected data rate of the PSDU.

For the mandatory data rate (1Mbit/s)  phyMaxFrameDuration is calculated as follows:

phyMaxFrameDuration1M =

phySHRDuration1M + [1.5 + 3/4*ceiling(4/3*aMaxPHYPacketSize)]* phySymbolsPerOctet1M

For the optional data rate (250kbit/s)  phyMaxFrameDuration is calculated as follows:

phyMaxFrameDuration250k = phySHRDuration250k + 3*ceiling(1/3*[1.5+aMaxPHYPacketSize])* phySymbolsPerOctet250k

Add to clause 7.4.2 MAC PIB attributes

For the CSS PHY type, the values of the read-only attribute macAckWaitDuration depend on the selected data rate of the PSDU.

For the mandatory data rate (1Mbit/s)  macAckWaitDuration is calculated as follows:

macAckWaitDuration1M = aUnitBackoffPeriod + aTurnaroundTime +

phySHRDuration1M + [1.5 + 3/4*ceiling(4/3*5)]* phySymbolsPerOctet1M

(13a1)

For the optional data rate (250kbit/s)  macAckWaitDuration is calculated as follows:

macAckWaitDuration250k = aUnitBackoffPeriod + aTurnaroundTime +

phySHRDuration250k + 3*ceiling(1/3*[1.5+5])* phySymbolsPerOctet250k

(13a2)

6.4.2 Change Table 86—MAC PIB attributes

| Attribute | Identifier | Type | Range | Description |
| --- | --- | --- | --- | --- |
| macAckWaitDuration† | 0x40 | List of Integer | See Equations (13, 13a1, 13a2) | The maximum number of symbols to wait for an acknowledgment frame to arrive following a transmitted data frame. This value is dependent on the supported PHY, which determines both the selected logical channel and channel page. The calculated value is the time to commence transmitting the ACK plus the length of the ACK frame. The commencement time is described in 7.5.6.4.2. For all non-UWB and non-CSS PHY types a single value. For UWB PHY ... For CSS PHY list of two values in symbols. First value corresponds to the mandatory data rate (1Mbit/s), second to the optional data rate (250kbist/s). |