IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE P802.15 WPAN SG4b Draft 5C |  |
| Date Submitted | [18 September, 2006] |  |
| Source | [Phil Beecher, SG4d Chair] [Integration UK Ltd] [16 West Street, Reigate, Surrey, UK] | Voice: [+44 1737 227728] Fax: [] E-mail:[pbeecher@integration.com] |
| Re: | [Document 15-06-0334-00-wng0-Japanese-950MHz.ppt from Shigeru Fukunaga.] |  |
| Abstract | [During the July 2006 IEEE 802 Plenary the IEEE P802.15 working group formed the IEEE 802.15 4d study group with the goal to create a Project Authorization Request for enhancements to the IEEE 802.15.4-2006 standard. This document contains the 5 criteria.] |  |
| Purpose | [This document is supporting the submission of the PAR to the P802.15 Working Group] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE P802.15 Low Rate Wireless Personal Area Networks Study Group Functional Requirements Standards Development Criteria

The IEEE P802.15 4d Study Group for Wireless Personal Area Networks (WPANs) reviewed and completed the required IEEE Project 802 Functional Requirements, Standards Development Criteria (a.k.a. the Five Criteria). The IEEE P802.15 WPAN Five Criteria response is in Italics below.

1. BROAD MARKET POTENTIAL

a) Broad sets of applicability

There is increasing interest for Low Rate Wireless Personal Area Network (WPAN-LR). Especially in Japan, sub Giga band is required for the many applications of WPAN-LR.

Examples of applications include Home Automation, Meter Reading, Medical Monitoring, Precision Agriculture and Environmental Networks, Industrial Controls, and Access/Authorization.  Examples of devices include Smart Tags & Badges, Auto Location ID’s, Stick-on and Security Sensors, Interactive Toys, Human Interface Devices ( HIDs), Portable bar code readers, and Remote controls.  With an effective wireless standard, geared to this class of applications, the Japanese market potential is huge.

The wireless capability will make these devices easier to use and provide additional functionality and efficiency.

b) Multiple vendors and numerous users

The breadth of membership of this WPAN Low Rate Study Group demonstrates the interest in this class of WPANs. Members include international wireless industry leaders, academic researchers, semiconductor manufacturers, system integrators, and end users. Already, there are industry consortiums, such as ZigBee and WINA actively addressing the requirements of ultra low power, low data rate wireless PAN class networks and are promoting the current standard .There are currently at least 3 semiconductor manufacturers providing semiconductor solutions for sub 1GHz 80215.4.

The target user base will be large as indicated by the growing demand for wireless connectivity in almost all devices.

c) Balanced costs (LAN versus attached stations)

The proposed amendment to 802.15.4-2006 will be developed with the aim that the connectivity costs will be a reasonably small fraction of the cost of the target devices such as sensors, tags, HIDs, and bar code readers as previously mentioned.

2. COMPATIBILITY

IEEE 802 defines a family of standards. All standards shall be in conformance with IEEE 802.1 Architecture, Management and Interworking. All LLC and MAC standards shall be compatible with ISO 10039, MAC Service Definition1, at the LLC/MAC boundary. Within the LLC Working Group there shall be one LLC standard, including one or more LLC protocols with a common LLC/MAC interface. Within a MAC Working Group there shall be one MAC standard and one or more Physical Layer standards with a common MAC/Physical layer interface. Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with OSI systems management standards.

Note: This requirement is subject to final resolution of corrections and revision to current ISO 10039, currently inconsistent with ISO 8802 series standards.

The MAC (Medium Access Control) Layer of the Wireless Personal Area Network (WPAN) Standard will be compatible with the IEEE 802 requirements for architecture, management, and inter-networking.

3. DISTINCT IDENTITY

a) Substantially different from other IEEE 802 standards.

802.15.4-2006 uniquely supports wireless sensor and control application.  Without amendment, 802.15.4-2006 will not support the new frequency allocation 950MHz -956MHz, (forthwith referred to as 950MHz) proposed by the Japanese Ministerial Ordinance.

b) One unique solution per problem (not two solutions to a problem).

The proposed amendment to 802.15.4-2006 will provide a unique solution for the Japanese 950 MHz as WPAN-LR standard.

c) Easy for the document reader to select the relevant specification.

The proposed amendment to 802.15.4-2006 for Japanese 950 MHz will be a clearly distinguishable specification.

4. TECHNICAL FEASIBILITY

a) Demonstrated system feasibility

Physical layer implementations in the 900MHz band are well known and well characterized

b) Proven technology, reasonable testing

There are examples of technology that exist today, which will allow design and fabrication of these radio systems.

c) Confidence in reliability

The air interface protocol will be designed to meet commercial reliability standards. Existing products provide confidence in the reliability of the proposed project.

Although not strictly required as there are no other IEEE standard radios operating in the specified band, a Coexistence Assurance document will be created to describe coexistence with other systems operating in the band.

5. ECONOMIC FEASIBILITY

a) Known cost factors, reliable data

High volume applications in Japanese 950 MHz band provide a low cost source of components. Existing products indicate cost targets are easily met..

b) Reasonable cost for performance

Based on test results, prototype, and production solutions, the estimates meet expected size, cost, and power requirements.

c) Consideration of installation costs

One of the 802.15.4-2006  standard objectives includes low cost installation with minimal to no operator intervention.