IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | SG BAN Draft 5C |  |
| Date Submitted | [November, 2006] |  |
| Source | [Arthur W. Astrin#1, Huan-Bang Li#2, and Ryuji Kohno#2] [#1=Astrin Radio, USA #2=National Institute of Information and Communications Technology (NICT), Japan] [#1=1051 Greenwood Ave.Palo Alto, CA 94301 USA #2=3-4 Hikarino-oka, Yokosuka, Kanagawa, Japan] | Voice: [+1-650-704-2517] E-mail: [art@astrinradio.com ] |
| Re: | [In response to the IEEE 802.15 motion to form IG BAN] |  |
| Abstract | [Since May 2006, IG BAN has investigated and studied the market requirements, industry needs, and technology feasibility for a body area network alternative for PHY /MAC for IEEE Standard 802.15.4-2006, with the goal to create a Project Authorization Request. This document contains the supporting 5 criteria.] |  |
| Purpose | [This document is supporting the submission of the PAR to the P802.15 Working Group] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

IEEE P802.15 Body Area Networks (BAN) Study Group

Functional Requirements Standards Development Criteria

The IEEE P802.15-BAN study Group for Body Area Networks (BAN) reviewed and completed the required IEEE Project 802 Functional Requirements, Standards Development Criteria (a. k. a. the Five Criteria). The IEEE P802.15 BAN Five Criteria response is in Italics below.

1. BROAD MARKET POTENTIAL

- Broad sets of applicability

There is increasing interest in Body Area Networks (BAN).

Examples of applications include Body Automation, Medical Monitoring, evolutionary set of health care services, medical implants, and assistance to people with disability, body interaction, entertainment and Access/Authorization.

Examples of devices include Smart Implants (Cochlea, Artificial heart, Diabetes monitor and control, Auto Location for inductive, in-body battery recharging, Capsule Endoscope) Stick-on EKG/ECG, temperature and blood pressure Sensors, and other vital signal sensors. Interactive Medication Dispensers, Human Interface Devices (HIDs), Wireless headset, and Remote controls. With an effective wireless standard, geared to this class of applications, the BAN market potential is huge.

The wireless capability will make these devices easier to use and provide additional functionality and efficiency.

- Multiple vendors and numerous users

The breadth of membership of this BAN study group demonstrates the interest in BAN. Members include international wireless industry leaders, academic researchers, semiconductor manufacturers, system integrators, and end users. Already, there are industry consortiums, such as MAGNET in EU, Continua in USA, and MICT Consortium in Japan, actively addressing the requirements of ultra low power, reliable data, multi-rate wireless BAN class networks and are promoting the current standard.  FCC, EU, Japan and New Zealand as well as many other countries/areas have assigned Medical Telemetry frequency bands and medical implant frequency bands. There are currently at least 3 semiconductor manufacturers providing semiconductor solutions.

By establishing a globally harmonized standard, the target user base will be large as indicated by the growing demand for wireless connectivity in almost all devices.

- Balanced costs

The proposed project will be developed with the aim that the connectivity costs will be a reasonably small fraction of the cost of the target devices such as sensors, tags, HIDs, as previously mentioned.

2. COMPATIBILITY

IEEE 802 defines a family of standards. All standards shall be in conformance with IEEE 802.1 Architecture, Management and Interworking. All LLC and MAC standards shall be compatible with ISO 10039, MAC Service Definition, at the LLC/MAC boundary. Within the LLC Working Group there shall be one LLC standard, including one or more LLC protocols with a common LLC/MAC interface. Within a MAC Working Group there shall be one MAC standard and one or more Physical Layer standards with a common MAC/Physical layer interface. Each standard in the IEEE 802 family of standards shall include a definition of managed objects, which are compatible with OSI systems management standards.

Note: This requirement is subject to final resolution of corrections and revision to current ISO 10039, currently inconsistent with ISO 8802 series standards.

The MAC (Medium Access Control) Layer of the Wireless Personal Area Network (WPAN) Standard will be compatible with the IEEE 802 requirements for architecture, management, and inter-networking.

3. DISTINCT IDENTITY

a) Substantially different from other IEEE 802 standards.

Transmit Power will be substantially lower due to proximity to or within human body.

|  | Other 802 standards | BAN |
| --- | --- | --- |
| Configuration | 15.3, 15.4 MAC | Single scalable MAC with reliable delivery |
| Power consumption | Low power consumption | Extremely low power while communicating to protect human tissue |
| Power source | Conventional power source | Possible scavenge operation |
| Requirements | Low latency | Guaranteed response to external stimuli |
| Frequency band | ISM | Medical authorities approved bands for in and around human body |
| Channel | Air | Air in and around human body |

b) One unique solution per problem (not two solutions to a problem).

The proposed standard will address a unique solution for body area networks that provide short-range communications, in and around human body, addressing emerging markets such as continuous vital signal monitoring, assistance to people with disabilities, and entertainment oriented applications, with consideration for human body safety.

c) Easy for the document reader to select the relevant specification.

The proposed project will be a clearly distinguishable specification so that users can easily distinguish and select the specifications.

4. TECHNICAL FEASIBILITY

a) Demonstrated system feasibility

Physical layer implementations in the approved frequency bands are well known and well characterized. This provides confidence that this amendment to meet the requirements of BAN is feasible.

b) Proven technology, reasonable testing

There are examples of technology that exist and are tested today, which will allow design and fabrication of BAN radio systems

c) Confidence in reliability

The Project will standardize a new PHY that meets the national medical and regulatory authorities standards. Existing products provide confidence in the reliability of the proposed project.

5. ECONOMIC FEASIBILITY

a) Known cost factors, reliable data

High volume applications in health care services, assistance to people with disability, body interaction and entertainment will provide a low cost source of components. Development efforts for BAN will ensure a cost that is consistent with reasonable business strategy. Existing products indicate cost targets will be easily met.

b) Reasonable cost for performance

Based on test results, prototype, and production solutions, the estimates meet expected size, cost, and power requirements.

c) Consideration of installation costs

One of the project objectives includes low cost installation with minimal to no operator intervention.