IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Annex giving example transmission |  |
| Date Submitted | 16 November 2006 |  |
| Source | [Michael Mc Laughlin] [Decawave Ltd.] | Voice: [ +352-87-688-2514 ] Fax: [ ] E-mail: [ michael@decawave.com] |
| Re: | 802.15.4a |  |
| Abstract | An example of a short message converted to actual chips on the air using the 15.4a draft specification. |  |
| Purpose | To be incorporated as an annex into draft 6 of the 802.15.4a standard. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Annex M.

(informative)

## Example  UWB PHY Transmit Data Frame Encoding

This informative annex provides supplemental material to the normative clauses of the standard.

### M.1.1

### Channel used in the example

This annex provides one example of how the PHY would encode a short sample PSDU received from the MAC. In the example, the PHY transmits at 850kbps on channel 3 using preamble code index 6. The annex shows how the data gets changed by the PHY encoding steps that eventually lead to bursts of pulses for transmission.

### M.2.1

#### Transmit PSDU

In this example the MAC presents a sample PSDU to the PHY via the PHY SAP. The sample that will be encoded is the following 17 octet PSDU.

UWB welcomes IEEE

converted from ASCII to decimal as follows

85    87    66    32   119   101   108    99   111   109   101   115    32    73    69    69    69

which is in hex as follows:

55    57    42    20    77    65    6C    63    6F    6D    65    73    20    49    45    45    45

Note that the MAC would not usually present such a PSDU to the PHY because it does not contain a valid CRC. The purpose of this annex is to provide an example to help the reader to understand the PHY encoding process, not the MAC.

M.2.2

#### PSDU bits

For the rest of this annex, the ternary +, −, 0 notation will be used where + represents a one, 0 represents a zero and − represents a -1.

The sample PSDU is converted to binary, LSB first and starting with the first bit in time first as follows:

+0+0+0+0+++0+0+00+0000+000000+00+++0+++0+0+00++000++0++0++000++0++++ 0++0+0++0++0+0+00++0++00+++000000+00+00+00+0+0+000+0+0+000+0+0+000+0

#### M.2.3

#### RS Encoded bits

Reed Solomon encoding is then applied to these bits as described in 6.8a.9.1 to give the following bits:

+0+0+0+0+++0+0+00+0000+000000+00+++0+++0+0+00++000++0++0++000++0++++0++0+0++0++0+0+00++0++00+++000000+00+00+00+0+0+000+0+0+000+0+0+000+000++0++000+++00+0++++0+00+++++++00+++0+++00+0+00

#### M.2.4

#### Convolutional encoder input bits

The next step is to prepend the PHY header to this RS encoded data for input to the convolutional coder. Because of the data rate and length of PSDU, the PHY header is as follows:

Rt1 Rt0 L6 L5 L4 L3 L2 L1 L0 Rng Ext Pr1 Pr0 C5 C4 C3 C2 C1 C0

0 1 0 0 1 0 0 0 1 0 0 0 1 1 1 0 0 1 1

Prepending this to the PSDU gives the following bit stream:

0+00+000+000+++00+++0+0+0+0+++0+0+00+0000+000000+00+++0+++0+0+00++000++0++0++000++0++++0++0+0++0++0+0+00++0++00+++000000+00+00+00+0+0+000+0+0+000+0+0+000+000++0++000+++00+0++++0+00+++++++00+++0+++00+0+00

#### M.2.5

#### Convolutional encoder output bits

Two tail bits are appended to the input bits and they are input to the convolutional encoder. The convolutional encoder produces two types of output bit. Position encoding bits, denoted g0, and sign or parity bits denoted g1.

For this example the position bits, g0, will be simply a delayed version of the above with a zero tail bit or:

00+00+000+000+++00+++0+0+0+0+++0+0+00+0000+000000+00+++0+++0+0+00++000++0++0++000++0++++0++0+0++0++0+0+00++0++00+++000000+00+00+00+0+0+000+0+0+000+0+0+000+000++0++000+++00+0++++0+00+++++++00+++0+++00+0+000

The sign bits, g1, will be:

0+0++0+0+0+0++0++++0+0000000+0+0000++0+00+0+0000+0+++0+0+0+0000+++++0+++0++0+++0+++0+00+0++000++0++0000++++0+++++0++0000+0++0++0++00000+0+00000+0+00000+0+0+0+++0+++0++0+++00+00+00+++00000++++0+0+0+++000+00

#### M.2.6

#### Scrambler output bits

The scrambler is now enlisted to help with the encoding process. For this complex channel the scrambler initializer, starting at s-15 and going to s-1 is:

1 1 1 0 0 0 1 0 1 1 0 1 1 0 1

Since there are 205 of each type of convolutional bits, there will be 205 symbols. Each symbol produces a burst of length 16 so the scrambler provides 3280 bits which are:

00+00+++0++0+++0++0+00++0++00++0+++0+0++0+0+0++00++++0++++++0+0+000++00000+++++00+0+0000+0000+0++++000++000+++000+00+0+00+00+00++0++++0++0++0+0++000++0++0++++0+00+0++0++000+++0+++0++0+00+00++00++0+++0++0+0+0+0++00++0++++++++0+0+0++0000000++++++0+000000+00000+++00000++0000+00+0000+0+000++0++000++++00+0++0+00+000+0+++0+++0++00+++00++00++0+0+00+0+0+0+0+++++0+++++++++0000++00000000+000+0+0000000++00++++000000+0+0+000+00000++++++00++0000+00000+0+0+000++0000++++++00+0+000+00000+0++++00++0000+++000+0+0+000+00+00++++++00++0++0+00000+0+0++0+++0000+++++0++00+000+0000++0+0++00++000+0++++0+0+0+00+++000+++++++0+00+00+000000+++0++0++00000+00++0++0+0000++0+0++0+++000+0++++0++00+00+++000++0+0++0+00+00+0++++0+++0++0+++000++00++0++00+00+0+0+0++0+0++0+++++++0++++0++000000++000++0+00000+0+00+0+++0000++++0+++00+000+000++00+0++00++00+0+0+++0+0+0+0+++++00++++++++0000+0+0000000+000++++000000++00+000+00000+0+0++00++0000+++++0+0+0+000+0000+++++++00++000+000000+0+0+00++00000++++++0+0+0000+00000+++++000++0000+0000+00+0+000++000++0++++00+0+00+0++000+0++++0+++0+00+++000++00+++0+00+00+0+0+00+++0++0++++++0+00++0++00000+++0+0++0+0000+00++++0+++000++0+000++00+00+0+++00+0+0++0+++00+0+++++0++00+0+++0000++0+0+++00+000+0++++00+0++00+++000+0+++0+0+00+00+++00+++++0++0+00+0+0000++0+++0++++000+0++00++000+00+++0+0+0+00++0+00+++++++0+0+++0+000000++++00+++00000+000+0+00+0000++00++++0++000+0+0+000++0+00++++++00+0+++0+00000+0+++00+++0000+++00+0+00+000+00+0++++0++00++0+++000++0+0+0++00+00+0++++++0+0++0+++00000++++0++00+0000+000++0+0++000++00+0++++0+00+0+0+++000+++0+++++00+00+00++0000+0++0++0+0+000+++0++0+++++00+00++0++0000+0++0+0++0+000+++0++++0+++00+00++000++00+0++0+0+00+0+0+++0+++++0+++++00++0000++0000+0+0+000+0+000++++++00++++00+00000+0+000+0++0000++++00+++0+000+000+0+00+++00++00++++0+00+0+0+0+000+++0++++++++00+00++0000000+0++0+0+000000+++0+++++00000+00++0000+0000++0+0+000++000+0+++++00+0+00+++0000+0++++0+00+000+++000+++0++00+00+00+00++0+0++0++0++0+0++++0++0++0++++000++0++0++000+00+0++0++0+00++0+++0++0+++0+0++00++0++00++++0+0+0++0+0+000++++++0+++++00+00000++0000+0++0000+0+000+++0+000++++00+00+++00+000+0++0+00+0++00+++0+++0+++0+0+00++00++00+++++0+0+0+0+0+0000+++++++++++000+0000000000+00++000000000++0+0+00000000+0+++++0000000+++0000+000000+00+000++00000++0++00+0+0000+0++0+0++++000+++0++++000+00+00++000+00++0++0+0+00++0+0++0+++++0+0++++0++0000++++000++0+000+000+00+0+++00++00++0+++00+0+0+0+0++00+0+++++++++0+0+++00000000++++00+0000000+000+0++000000++00+++0+00000+0+0+00+++0000++++++0+00+000+00000+++0++00++0000+00++0+0+0+000++0+0+++++++00+0++++000000+0+++000+00000+++00+00++0000+00+0++0+0+000++0+++0+++++00+0++00++0000+0+++0+0+0+000+++00+++++++00+00+0+000000+0++0++++00000+++0++000+0000+00++0+00++000++0+0+++0+0+00+0++++00+++++0+++000+0+0000++00+00++++000+0+0++0+000+00+++++0+++00++0+0000++00+0+0+++000+0+0+++++00+00+++++0000+0++0+0000+000+++0+++000++00+00++00+00+0+0++0+0+0++0+++++0++++++0++0000++00000++0+000+0+0000+0+++00++++000+++00+0+000+00+00+0++++00++0++0+++000+0+0++0++00+00+++++0++0+0++0+0000++0++++0+++000+0++000++00+00+++0+00+0+0++0+00+++0+++++0+++0+00++0000++00+++0+0+000+0+0+00+++++00++++++0+0000+0+00000+++000++++0000+00+00+000+000++0++0++00++00+0++0++0+0+0+0+++0++0++++++++00++0++0000000+0+0++0+000000+++++0+++00000+0000++00+0000++000+0+0++000+0+00+++++0+00++++0+0000+++0

#### M.2.6

#### Ternary output symbols

These convolutional encoder and scrambler outputs are used to generate the ternary pulses. In this example, each symbol consists of 512 chips. During each symbol the 512 chips are silent except for a train of 16 pulses. The following tables give the chip position of the first pulse and show the signs of the burst of pulses, starting with the first in time. The chip positions in this example can range from 0, the first chip position, to 511, the final chip position. Each symbol is also numbered from 0 to 204.