IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft TG3c System Requirements |  |
| Date Submitted | [18 January, 2007, Revised on 12 March, 2007(by H. Ikeda)] |  |
| Source | [Alireza Seyedi] [Philips] [345 Scarborough Rd, Briarcliff Manor, NY, 10510, USA] | Voice: [+1-914-945-6318] Fax: [+1-914-945-6580] E-mail: [alireza.seyedi@philips.com] |
| Re: | [This document is an updated version of IEEE 802.15-05-0353-08-003c, Revision due to change of Channel model document revision] |  |
| Abstract | [TG3c System Requirements] |  |
| Purpose | [TG3c System Requirements] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 	Introduction	3

2.0 	Physical Layer Technical Characteristics	3

3.0 	Bit Rate and Range	3

4.0	Delay	5

5.0 	Channelization	5

6.0 	Coexistence and Interference Resistance	5

7.0 	Channel Models	5

8.0 	Power Consumption	6

9.0 	Form Factor	6

10.0 	Complexity	7

11.0 	Supplements to Standard 802.15.3-2003 functionality	7

12.0	Regulatory	7

Annex A (informative) Profile of Applications	7

# 1.0 	Introduction

This document and the Selection Criteria (15-05-0493-27-003c) provide the technical content for the project to develop an alternative physical layer (alternative PHY). This alternative PHY shall be a supplement to the Standard IEEE 802.15.3-2003.

This document serves two purposes. It is first a summary of the application presentations from the Study Group 3c Call for Applications (CFA) (15-04-0204-00-003c). Second, it defines the fundamental requirements implied by applications but not necessarily stated explicitly. The selection criteria document, (15-05-0493-27-003c) will guide proposers on how to respond to a Call for Proposals. It also provides amplification on key areas not addressed in this document. The group may update this document as discoveries are made while working on the Selection Criteria document.

# 2.0 	Physical Layer Technical Characteristics

Anticipated high-level characteristics of the alternative physical layer are:

- High bit rates

- Long ranges

- Low power consumption

- Provide coexistence mechanisms

- Multiple channels

Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 3.0 	Bit Rate

At least one mandatory mode with a PHY-SAP Payload Bit Rate of 2Gbps or more is required. Also, at least one optional mode with a PHY-SAP Payload Bit Rate of 3Gbps or more is desired. Figure 1 shows the reference partitioning.

Figure 1 – Reference partitioning

# 4.0	Delay

The proposed system shall provide at least one mode with sufficiently low latency in order to satisfy the applications.

# 5.0 	Channelization

The PHY shall provide a minimum of 3 channels, each being able to support the data rates as specified in section 3.0, with similar performance. This requirement is based on the bandwidth available in the US, Canada and Japan regulatory domains. The proposers should demonstrate the number of channels that their proposal provides in other regulatory domains, where possible.

# 6.0 	Coexistence and Interference Resistance

The proposed PHY should address coexistence with other wireless devices that may be in close proximity to IEEE 802.15.3c devices. This includes interference from 802.15.3c devices to other wireless systems as well as interference from other wireless systems or radiators to 802.15.3c devices.

Coexistence and interference resistance shall, at a minimum, be considered for the following wireless devices: IEEE 802.11, IEEE 802.15.1, IEEE 802.15.3, IEEE 802.15.4, IEEE 802.16, ISM devices, mm wave analog broadcasting systems (that may exist as optional modes in 802.15.3c) and mm wave collision avoidance radars. Interference resistance from other radiators, like microwave ovens, shall also be considered.

# 7.0 	Channel Models

Performance in a multipath environment is a critical feature for the TG3c alternative PHY. Document (15-07-0584-01-003c) provides a reference alternative PHY channel model. The individual proposers shall make use of the TG3c models as they become available as a basis for evaluating the performance of their proposals. Individuals submitting proposals that are not addressed by the RF channel models selected by the group shall provide specific details of the channel models used with references to literature, used in other standards, and/or research.

# 8.0 	Power Consumption

The proposer shall estimate DC power consumption in both the transmit state and the receive state.

The proposer should indicate the possible power management actions that can be initiated by Standard 802.15.3-2003 "PHY-PWRMGT.request". Furthermore, the proposers should provide text including a table that indicates the number of power save levels offered and how much power each level consumes.

# 9.0 	Form Factor

The size and form factor of the proposed PHY (including the antenna) should be compatible with the application.

# 10.0 	Complexity

The proposed PHY should lend itself to highly integrated implementation. The complexity of the PHY should not prevent mass commercial adoption of a variety of cost sensitive products.

# 11.0 	Supplements to Standard 802.15.3-2003 functionality

It is required that the alternative PHY should operate with the 802.15.3 MAC without fundamental change in operation.  There may be MAC modifications necessary to support the mm wave PHY.

# 12.0	Regulatory

The alternative PHY standard should comply with necessary geopolitical or regional regulations.

# Annex A: List of Contributors

- Bruce Bosco

- Reed Fisher

- Brian Gaucher

- James Gilb

- Kiyoshi Hamaguchi

- Hideto Ikeda

- Nobujiko Kuribayashi

- Tadahiko Maeda

- Abbie Mathew

- Hiroyo Ogawa

- Tony Pollock

- Khusro Saleem

- Alireza Seyedi

- Yozo Shoji

- Stan Skafidas

- Su-Khiong Yong

# Annex B: (informative) Profile of Applications

Table below summarizes potential applications of millimeter wavelength systems as submitted in response to the CFA. The proposals should illustrate support for some of the applications listed. The applications have been arranged in the numeric order of the document number (last column).

| # | Description Of Applications | Outdoor | Indoor | CFA (IEEE) Document Number |
| --- | --- | --- | --- | --- |
| 1 | Vertically connected wireless link | - LOS - Bandwidth:300 to 2500 MHz - Range: ≤ 33 m | - | 04-0006 |
| 2 | Gigabit Ethernet link, wireless IEEE1394 applications | - | - LOS - Data Rate: ≤1 Gbps duplex - Range: ≤ 17 m | 04-0019 |
| 3 | Vertical wireless link | - LOS - Bandwidth:300 to 2500 MHz - Range: ≤ 30 m | - | 04-0092 |
| 4 | Ad hoc information distribution system | - | - LOS - Data Rate: 622Mbps - Range: ≥ 20 m (AP-AP) ≥ 3 m (AP-MT) | 04-0097 |
| 5 | Multimedia, information distribution system | - | - LOS - Data Rate: ≥ 1 Gbps - Range: ≤ 10 m | 04-0098 |
| 6 | - Outdoor: Fixed wireless access, distribution in stadiums, inter-vehicle communication, etc. - Indoor: Connecting multimedia devices (wireless home link), ad-hoc meeting, heavy content download, distribution system | - LOS - P2P, P2MP - Data Rate: 156 Mbps to 1.5 Gbps - Range: 400 m to 1 Km | - LOS - Data Rate: 100 Mbps to 1.6 Gbps - Range: ~ 10 m | 04-0118 |
| 7 | Small office/meeting scenario, general office applications | - | - NLOS - OFDM - Data Rate: ≤ 200 Mbps - Range: 2 to 4 m | 04-0141 |
| 8 | Distribution links in apartments, stadium, etc. | - LOS - P2P - Bandwidth:>300 MHz - Range: ≤ 220 m | - | 04-0153 |
| 9 | Ad hoc network | - | - LOS - Data Rate: 622 Mbps - Range: ≥ 20 m (AP-AP) ≥ 3 m(AP-MT) | 04-0155 |
| 10 | Wireless home video server connected to HDTV, PC and other video devices | - | - LOS - Data rate: 300 Mbps, 400 Mbps and 1.5 Gbps uncompressed HDTV data - Range: ≤ 10 m | 04-0348 |
| 11 | Wireless IEEE1394 applications | - | - LOS - Data Rate: 100, 200, 400, 800, 1600, 3200 Mbps - Range: 12 m LOS | 04-0351 |
| 12 | - Outdoor: Distribution links in apartments, stadium, etc. - Indoor: Ad-hoc network | - LOS - P2P and P2MP - Bandwidth:>300 MHz - Range: ≤ 220 m | - LOS - Data Rate: ≥ 1 Gbps and ≥ 622 Mbps - Range: ≥ 20 m and ≥ 3 m | 04-0352 |
| 13 | PowerPoint and such applications | - | - LOS and NLOS - Data Rate: ≥ 1 Gbps - Range: ≤ 3 m - Space diversity | 04-0514 |
| 14 | Two way vertical wireless link | - LOS - P2P - Bandwidth:500 to 2500 MHz - Range: ≤ 30 m | - | 04-0649 |
| 15 | Wireless Gigabit Ethernet applications | - | - LOS - Data Rate:1.25 Gbps duplex - Range: ~ 6 m | 04-0653 |
| 16 | - Replacement for 1394 FireWire - Replacement for USB - Military – future combat systems, secure communication | - | - LOS and NLOS (people) - 100 to 500 Mbps link, 1 Gbps in 2007 - Short range | 04-0665 |

PHY-SAP Payload Bit Rate is defined in Section 6.2.1 of the TG3c Selection Criteria document (15-05-0493-26-003c).