IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PHY System Parameters |  |
| Date Submitted | 29 August 2007 |  |
| Source | [Clinton Powell; Kuor-Hsin Chang; Michael Schmidt] (Freescale Semiconductor, Inc; Atmel) | Voice: [1.480.413.5413; 1.480.413.4441; +49 351 6523436] Fax: [] E-mail: [clinton.powell@freescale.com kuor-hsin.chang@freescale.com michael.schmidt@hno.atmel.com] |
| Re: | PHY System Parameters |  |
| Abstract | This document specifies the parameters for which the .4c proposed alternative sub 1-GHz center frequency PHY’s will be simulated and characterized in order to support the draft text that will be submitted for ballot approval. |  |
| Purpose | This document is provided in support of 802.15.4c PHY activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PHY SYSTEM PARAMETERS

For 779 - 787 MHz Band

PN Codes	Specify

Modulation Scheme	Crest Factor

EVM

P(e) - PER curve	AWGN, 20byte PSDU + hdr (SHR+

PHR) for 0ppm, +/-20ppm, +/-40ppm

Flat Fading	PER Curve for Delay spread = 0

20byte PSDU + hdr for 0ppm

Delay Spread RMS	PER Curve for Delay spread = 250nS

20byte PSDU + hdr for 0ppm

(per. doc 15-04-0585-00-004b)

Sync (preamble)	Codes & Length

780MHz PSD Mask	10dBm pk in-band w/ 16kHz res.

-36dBm pk out-band w/ 100kHz res.

Bit Rate, Chip Rate	Specify

Synchronization	State Method (ideal or simulated)

SFD	Specify

Modulation Scheme

There are two important parameters, characterizing the modulation scheme, the crest factor and the error vector magnitude (EVM).

Crest Factor:

The crest factor of the complex baseband signal x(t) is defined as

Ccrest=maxx(t)Ex(t)−Ex(t)2

i.e., a value characterizing the peak-to-average ration. Constant envelop signals have a crest factor close to 1.  A low crest factor usually simplifies the design of the power amplifier.

Error Vector Magnitude:

The EVM is a figure of merit characterizing the allowed deviation from a perfect transmitter.

The EVM is defined as

EVM=1Nj=1NδIj2+δQj2S×100%

where S is the magnitude of the vector of the ideal constellation point, (δIj,δQj)the error vector.

It should be noted, that a modulation scheme violating the first Nyquist criteria inherently obtains an EVM >0, even with a perfectly build transmitter. Also, as the number of constellation points increases, the maximum allowable EVM needs to be decreased.

780MHz band transmit PSD mask

The transmitted spectral products shall be less than the limits specified in Table 1. For the relative limit, average spectral power shall be measured using a 100 kHz resolution bandwidth. Also, the reference level shall be the highest average spectral power measured within ± 600 kHz of the carrier frequency.

Table 1 780 MHz band transmit PSD limit

| Frequency | Relative limit |
| --- | --- |
| f−fc>1.2 MHz | -20 dB |