IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PHY System Parameters |  |
| Date Submitted | 18 September 2007 |  |
| Source | [Clinton Powell; Kuor-Hsin Chang; Michael Schmidt] (Freescale Semiconductor, Inc; Atmel) | Voice: [1.480.413.5413; 1.408.904.2705; +49 351 6523436] Fax: [] E-mail: [clinton.powell@freescale.com kuor-hsin.chang@freescale.com michael.schmidt@hno.atmel.com] |
| Re: | PHY System Parameters |  |
| Abstract | This document specifies the minimum set of parameters for which all proposed .4c alternative sub 1-GHz center frequency PHY’s will be evaluated. It is expected that these parameters are included in the proposal. |  |
| Purpose | This document is provided in support of 802.15.4c PHY activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PHY SYSTEM PARAMETERS

For 779 - 787 MHz Band

PN Codes	Specify

Modulation Scheme	            Specify, if not constant envelop  
            needs to specify the Crest Factor

P(e) - PER curve	AWGN, 20byte PSDU + hdr (SHR+

PHR) for 0ppm, +/-20ppm, +/-40ppm

Flat Fading	PER Curve for Delay spread = 0

20byte PSDU + hdr for 0ppm

Delay Spread RMS	PER Curve for Delay spread = 250nS

20byte PSDU + hdr for 0ppm

(per. doc 15-04-0585-00-004b)

Sync (preamble)	Codes & Length

780MHz PSD Mask	10dBm pk in-band w/ 16kHz res.

-36dBm pk out-band w/ 100kHz res.

Bit Rate, Chip Rate	Specify

Synchronization	State Method (ideal or simulated)

SFD	Specify

Modulation Scheme

One important parameter to characterize the modulation scheme is the crest factor.

Crest Factor:

The crest factor of the complex baseband signal x(t) is defined as

Ccrest=maxx(t)Ex(t)−Ex(t)2

i.e., a value characterizing the peak-to-average ration. Constant envelop signals have a crest factor close to 1.  A low crest factor usually simplifies the design of the power amplifier.

779-787 MHz Band Transmit PSD Limits

The transmitted spectral products shall be within the limits specified in Table 1. For the relative/absolute limit, average spectral power shall be measured using a 100 kHz resolution bandwidth. Also, the reference level shall be the highest average spectral power measured within ± 600 kHz of the carrier frequency.

Table 1 779-787 MHz band transmit PSD limits

| Frequency | Relative limit | Absolute limit |
| --- | --- | --- |
| f−fc>1.2 MHz | -20 dB | -20 dBm |

Out of Band Spurious Emission Requirements

The spurious emission requirements are the maximum transmit emissions allowed outside [779MHz, 787MHz]. The spurious emission requirements for China are shown in Table 2.

Table 2  Spurious Emission Requirements outside 779-787MHz band

| Frequency Range | Limit during operation | Limit in standby |
| --- | --- | --- |
| 30 MHz - 1 GHz | -36 dBm/100 KHz | -47 dBm/100 kHz |