IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Transmit power spectral density (PSD) mask |  |
| Date Submitted | 29 August, 2007 |  |
| Source | [Kuor-Hsin Chang; Clint Powell] [Freescale Semiconductor, Inc.] [Tempe, AZ] | Voice: [1.480.413.4441; 1.480.413.5413] Fax: [] E-mail: [kuor-hsin.chang@freescale.com clinton.powell@freescale.com] |
| Re: | The transmit PSD mask for 779-787MHz band. |  |
| Abstract | This document proposes the transmit PSD mask requirements for 779-787MHz band in China. |  |
| Purpose | To document the transmit PSD mask requirements for 779-787MHz band. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Transmitter Limits Regarding Narrowband Spurious Emissions for 779-787 MHz

# Transmit PSD Mask

A transmitter spectrum mask for 779-787MHz is shown in Figure 1. In Figure 1, there is a relative limit of -20dB which indicates that the spectral power measured above fc+1.2MHz should be less than the measured maximum spectral power by 20dB. Where the average spectral power shall be measured using a 100 kHz resolution bandwidth. Also, the reference level shall be the highest average spectral power measured within ± 600 kHz of the carrier frequency.

Figure 1 Transmit Spectrum Mask

# Spurious Emission Requirements

Spurious emission requirements are the maximum transmit emissions allowed outside [779MHz, 787MHz]. The spurious emission requirements for China are shown in Table 1.

Table 1 Narrowband spurious emission requirements

| Frequency Range | Limit during operation | Limit in standby |
| --- | --- | --- |
| 30 MHz - 1 GHz | -36 dBm/100 KHz | -47 dBm/100 kHz |