IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Transmit power spectral density (PSD) limits for 779-787 MHz |  |
| Date Submitted | 14 September, 2007 |  |
| Source | [Kuor-Hsin Chang; Clint Powell] [Freescale Semiconductor, Inc.] [Tempe, AZ] | Voice: [1.480.413.4441; 1.480.413.5413] Fax: [] E-mail: [kuor-hsin.chang@freescale.com clinton.powell@freescale.com] |
| Re: | The transmit PSD mask for 779-787MHz band. |  |
| Abstract | This document proposes the transmit PSD mask requirements for 779-787MHz band in China. |  |
| Purpose | To document the transmit PSD mask requirements for 779-787MHz band. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Transmit PSD Limits for 779-787 MHz

The transmitter spectrum limits for 779-787MHz are shown in . In , when the maximum spectral power is 0dBm or less there is a relative limit of -20dB attenuation at fc+/-1.2MHz. When the maximum spectral power is greater than 0dBm there is an absolute attenuation limit of -20dBm at
fc+/-1.2MHz. The average spectral power shall be measured using a 100 kHz resolution bandwidth. Also, the maximum spectral reference level shall be the highest average spectral power measured within ± 600 kHz of the carrier frequency.

Figure 1 Transmit PSD Limits