IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Body Area Network (BAN) Technical Requirements |  |
| Date Submitted | [19 September, 2007] |  |
| Source | [Arthur Astrin] [Astrin Radio] [1051 Greenwood Ave. Palo Alto, CA 94301, USA] | Voice:[+1 650 704-2517 ] E-mail:[art@astrinradio.com] |
| Re: | [Body Area Network (BAN) Technical Requirements document] |  |
| Abstract | [BAN Technical Requirements] |  |
| Purpose | [This working document has been prepared to be a BAN Technical Requirements documentation |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

# 1.0 Introduction

This document and the Selection Criteria Document yy/nnn provide the technical content for the project to develop a Body Area Network.

This document serves two purposes. It is first a summary of the application presentations from the Study Group BAN Call for Applications. Second, it defines the fundamental requirements implied by applications but not necessarily stated explicitly. The Selection Criteria Document, yy/nnn will guide proposers on how to respond to a Call for Proposals. It also provides amplification on key areas not addressed in this document. The group may update this document as discoveries are made while working on the yy/nnn Selection Criteria document.

# 2.0 	Physical Layer Technical Characteristics

Anticipated high-level characteristics of the BAN physical layer are:

Tbd

Tbd

- Lower power consumption

- Improved coexistence

- Higher density piconets

Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 3.0 	Bit Rate and Range

Figure 1 shows the reference partitioning.

Figure 1 – Reference partitioning

The error rate criterion is equivalent to that specified in 11.6.1 of the Standard IEEE 802.15.3-2003. The proposer is encouraged to show signal processing techniques that will yield an ‘equivalent’ bit error rate of < 10-n　(TBD) at the PHY SAP. Data should be provided that shows the added complexity, power consumption, and latency due to this signal processing. In addition, estimates should be given to indicate the PHY overhead due to preambles and PHY headers.

| Requirement | Source |
| --- | --- |
| Non-delay (real-time) transmission at ____ (Application data rate) |  |
| Very-little-delay transmission at ___Gbps (Application data rate) |  |

| Requirement | Source |
| --- | --- |
| ____ Gbps at 3m distance (for point to point, application data rate) |  |
| Point to Multipoint |  |

# 4.0 	Channelization

The bit rates and ranges specified in section 3.0 shall be achievable on each piconet for operation with at least ___ other piconets operating in close proximity even with some degree of performance degradation.

Combination of analog/digital broadcasting channels and digital communication channels should be supported.

| Requirement | Source |
| --- | --- |
| ____ non-overlap channels |  |

# 5.0 	Coexistence and Interference Resistance

The PHY shall be able to coexist with other wireless devices that may be in close proximity to IEEE 802.15 devices. This includes interference from 802.15.BAN devices to other wireless systems as well as interference from other wireless systems or radiators to 802.15.BAN devices.

Coexistence and interference resistance shall, at a minimum, be considered for the following wireless devices: 802.15.1 (Bluetooth™), IEEE 802.11, IEEE 802.15 (3&4), IEEE 802.16, and ISM cordless phones. Interference resistance from radiators, like microwave ovens, shall also be considered.

| Requirement | Source |
| --- | --- |
| Coexistence with 802.15.1, IEEE 802.11, IEEE 802.15.3, IEEE 802.15.4, ISM cordless phones |  |
| Coexistence with IEEE 802.16 |  |
| Coexistence with mmWave collision avoidance radars |  |
| Interference resistance from microwave ovens |  |

# 6.0 	Channel Models

Performance in a multipath environment is a important feature for the BAN. Document (yy/nnn) provides a reference alternative PHY channel model. The individual proposers should make use of the BAN models as they become available as a basis for evaluating the performance of their proposals. Individuals submitting proposals that are not addressed by the RF channels selected by the group are encouraged to provide specific details of the channel models used with references to literature, use in other standards, and/or research.

| Requirement | Source |
| --- | --- |
| [Specification: Channel Model] | yy/nnnrn |

# 7.0 	Power Consumption

The proposer shall estimate DC power consumption in both the transmit state and the receive state.

The proposer should indicate the possible power management actions that can be initiated by Standard 802.15.3-2003 "PHY-PWRMGT.request". Furthermore, the proposers should provide text including a table that indicates the number of power save levels offered and how much power each level saves. A power save state is also required.

| Requirement | Source |
| --- | --- |
| Less than 1 W for mobile DEV | 04/352r0 |
| [Specification: Power consumption] | yy/nnnrn |

# 8.0 	Form Factor

The PHY components should be capable of fitting into a form factor consistent with a

A configuration shall be proposed for which the antenna size and form factor will not significantly increase the original device size.

| Requirement | Source |
| --- | --- |
| Consistent with a size of a pill to swallow |  |
| [Specification: Form Factor] | yy/nnnrn |

# 9.0 	Complexity

Complexity will be minimal to enable mass commercial adoption of a variety of cost sensitive products.

| Requirement | Source |
| --- | --- |
| [Specification: Complexity] | yy/nnnrn |

# 10.0 Supplements to Standard 802.15 functionality

# 11.0 Regulatory

The alternative PHY standard should comply with necessary geopolitical or regional regulations.

(Insert here the regulatory matrix)

# Annex A (informative) Profile of Applications

Table below summarizes applications submitted in response to the CFA. The applications have been arranged in the numeric order of the document number (last column).

| # | Description Of Applications | Outdoor | Indoor | CFA (IEEE) Document Number |
| --- | --- | --- | --- | --- |