IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Body Area Network (BAN) Technical Requirements |  |
| Date Submitted | [01 October, 2007] |  |
| Source | [Bin Zhen, NICT] [Maulin Patel, Philips] [SungHyup Lee, KORPA] [EunTae Won, Samsung] | E-mail: [zhen.bin@nict.go.jp] [maulin.patel@philips.com] [shlee@korpa.or.kr] [etwon@samsung.com] |
| Re: | [Body Area Network (BAN) Technical Requirements document] |  |
| Abstract | [BAN Technical Requirements] |  |
| Purpose | [This working document has been prepared to be a BAN Technical Requirements documentation |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Table of Contents

1.0 Introduction	3

2.0 BAN Technical Characteristics summary	3

3.0 Topology	5

4.0 Bit Rate	5

5.0 Range	6

6.0 Security	6

7.0 Quality of service (QoS)	6

8.0 Power Consumption	7

9.0 Coexistence and Interference Resistance	7

10.0 Form Factor	8

11.0 Antenna	8

12.0 Complexity	8

13. Mobility	8

14. Specific Absorption Rate (SAR)	8

15. Regulatory Matters	9

# 1.0 Introduction

This document provides the technical contents of the project to develop PHY and MAC protocols for Body Area Network. This document along with the corresponding selection criteria document (ref. 15-07-xxxx-0ban-SCD) will provide guidance on how to respond to a call for proposals. The reference model used for this standard is the following:

MAC_SAP: MAC Service Access Point

PHY_SAP: PHY Service Access Point

PMD: Physical Medium Dependent (radio)

PLCP: PHY Layer Convergence Protocol,

Figure 1, Reference partitioning

This document serves two purposes.  First, it summarizes the applications presented in response to BAN Study group call for Applications. Second, it defines the fundamental requirements implied by applications but not necessarily stated explicitly.

- ref. 15-07-0735-03-0ban-application-requirement-analysis.xls

- ref. 15-07-xxxx-00-0ban-usage-model.doc

# 2.0 BAN Technical Characteristics summary

The intended standard will define the PHY and MAC layers for short range, wireless communication in the body area. The standard aims to support a low complexity, low cost, ultra-low power and highly reliable wireless communication for use in close proximity to, or inside, a human body (but not limited to humans) to satisfy an evolutionary set of entertainment and healthcare products and  services. The project will address the requirements to support sensor, control and peripheral networks in multiple compliant co-located systems and also coexistence.

2.1 Applications and traffic analysis

The standard intends to address both medical/healthcare applications and other non-medical applications with different requirements. The medical applications cover continuous waveform sampling of biomedical signals and vital signal information, and low rate remote control. The non-medical applications include video and audio streaming, bulk data transfer, and small data transfer. Both application spaces typically desire a network of hundreds of sensors and actuators communicating with an access point, through which to be connected with the whole Internet.

Devices for the applications are usually highly constrained in terms of resource such as CPU, battery and memory and operate in unstable environments. At the same time, medical sensors and actuators have to physically small to be wearable or implantable. The wearable access points may also have some form of resource constraint.

Deployment of devices can be in the indoor, outdoor or even water environment in home, hospital, small clinic, fitness center etc. Depending on the philosophy of medical application, the major traffic tends to be point-to-multipoint and multipoint-to-point or a mix of them. Therefore the traffic flow is highly directed. Non-medical applications are generally point-to point.

Most of biomedical and vital signals are low frequency and period. Packet samples will be generated continuously, and the packet generation rates are from 1/ms to 1000/s. There are some sensors which are event/bursty (e.g. tumble sensors for elders). Most of traffic is upstream from sensors to access point. Almost all of the sensors may detect alarm conditions. Time crucial alarm packets are expected to have higher priority and low latency than senor data. The mean-time between missing of alarm packet should be over years.

Some sensors will transmit a log file every day, with typically Kbytes of data. There is very little “downstream” traffic (commands) coming from access point to a particular sensor. During diagnosis, may investigate a parameter in a command/response mode.

Low-rate remote control can be “open loop” or “close loop”. In the former case, sensor data makes its way through the access point to doctor who decide to take an action, and control information is sent out to the actuator in the network. It is envisioned that the “close loop” control is the future trend, which packet will flow over local loops without intervention from the doctor. Close loop control has a latency requirement that can be 100ms to seconds. In many of these applications, if the packets do not arrive within the specified interval, the system will enter an emergency alarm state, often with life or dead indication.

2.2 Overall requirements

Given the broad range of possible applications space, the key feature for this standard must be to address the issue of scalability, as it is recognized that in some cases the trade off for speed or security may be increased power consumption or indeed for improved quality of service other trade offs will need to be addressed in either or both the MAC or Physical layers. This standard shall formulate and propose the methods that this can be achieved, whilst demonstrating the key goals of improved lower power performance is indeed achieved against the peer standards in the field.

Anticipated high-level characteristics of the MAC and PHY layers are summarized as follows.

- The BAN should be self-forming, self-healing, secure, robust and reliable;

- Typical link throughput should be some tens of kb/s in most of the cases. However, raw data rate up to 10 Mbps is expected in some applications, and low data rate less than 10kb/s should be supported in some medical applications;

- The power consumption must allow for self-powered operating time without intervention from several hours to several years, depending on applications.

- QoS management, reliability, security and privacy, co-existence with other wireless technologies, environmental requirement (EMC/EMI), and power management are key features. Scalability is a key concern in terms of data rates, power consumption, network size, security;

- Minimization of SAR into the body to satisfy the local regulatory requirements;

Specific detailed requirements associated with these characteristics are described in the sections that follow.

# 3.0 Topology

The network components may be in close proximity to, or inside, a human body. Support for multihop communication is needed to enable connectivity between devices hidden by body. And these components should maintain bidirectional link, however, not necessarily symmetric.

The network configuration has to be highly dynamic. Thus the network must be workable without requiring complex static set up procedure and must comply with dynamic insertion and de-insertion of nodes into a network. No single point of failure should be guaranteed in life-critical applications.

The network configuration should be efficiently scalable (e.g. 256 nodes) to support some integrated medical applications.

Typical applications imply data collection by a unique or a set of coordinated data collectors. Thus the corresponding network component may have to sustain a much higher throughput than the ones of the other nodes. The data collectors should be able to transmit multicast messages to a set of nodes. For example, an ECG/EMG collector may issue a start (broadcast) command to all the electrodes asking them to start sampling and transmitting.

Topology awareness may be required for context-awareness applications. (?)

# 4.0 Bit Rate

The bit rate is categorized the following way:

- individual link bit rate. This is related to a peer to peer link, typically between a device and an information collector or between two devices.

- aggregated bit rate. This is typically the bit rate concentrated from many devices to a data collector during a short period of time (can be during specific situations when many devices need to update their information at the same time. For example, alarm or emergency situations, ECG/EMG devices have many electrodes and one aggregator etc). Typically, data collectors should gather data from 5 nodes, in the future the upper limit is expected to grow. The data collector should be capable of acquiring typically 1 Mb/s of effective data for configurations that require this feature.

Typical selected figures:

- Individual link bit rate: 1 kb/s (medical applications) and 10 kb/s (non-medical applications) at the low end, and 10 Mb/s at the high end at PHY-SAP.

- Aggregated bit rate (data collector only): 1 Mb/s or less (medical applications) and 10 Mb/s (non-medical applications) at PHY-SAP ?.

# 5.0 Range

The application analysis suggests that up to 3 meters of transmission range is sufficient for most applications. However, the range should be extendable up to 5 meters for some medical applications such as bed side monitoring in hospital, clinic, healthcare center or home. In this case, consideration should be given to how this can be achieved within the spectra masks that will apply.

# 6.0 Security

Medical BAN applications have substantial financial, privacy and human safety implications. Security is of consideration concern. During the transmission of encrypted physiological data, patient identifiers should be particularly protected. Deny of Service (DoS) occurs when network traffic is beyond the capacity if the systems. DoS is associated with the effects of both intentional act of malicious users and unintentional excessive peak network utilization.

Multi-level security is desired so that each application can chose a level that best suits its needs. For example camera pill may not need strong encryption and authentication process. However, pacemakers may need strong authentication process.  Devices need to be interrogated by authorized personal for the lifecycle of the product. The highest level of security should be equal to or stronger than those specified by IEEE 802.15.4_2006 standard.

Because security and privacy protection mechanism require a significant amount of computational resource and storage resource, the mechanism should be energy efficient and lightweight.

Novel security mechanisms are need for applications to account for

- The longevity of implanted devices.  (Doctor may need to calibrate the device once a year or so.  Patients may relocate to a different geographic region.)

- Average person cannot be expected to play the role of network administrator who can set up and manage authentication process.  Hence, limited user interaction during security configuration is desirable.

- Inability of the user to provide passkeys when needed.  (e.g. Devices should be accessible to paramedics/medics in a trauma condition.  However, in such situations, the user may not be able to provide authentication information)

Biometric identification based security mechanism could be useful for BAN.

# 7.0 Quality of Service (QoS)

The BAN applications fall into three large service categories:

- Published data: periodic or nearly periodic data which require constant bandwidth;

- Event data: alarms and aperiodic report with burst bandwidth requirements;

- Bulk transfer: blocks of data in multiple packets where temporary resources are assigned in a limited period of time.

QoS performance is an important part in the framework of risk management of medical applications. QoS support should be a major focus of PHY and MAC. The critical factor is the reliability of the transmission, meaning that appropriate error correction methods should be provided at PHY and MAC level.  QoS provisions should be flexible such that it can be tailored to suit application needs.

Other QoS parameters have a strong impact on MAC and PHY layers:

- Support of real time communication is required;

- Capability of providing fast and reliable reaction in emergency situations and alarm message which usually have higher priority than others;

- Support of congestion control and admission control are expected;

- Adaptive duty cycling and reservation to match the application load and latency requirement while maintaining low power operation;

- Link quality detection and channel migration should be considered when the channel gets crowded or noisy;

- The QoS should be considered in the case of abrupt system fall down in medical applications.

# 8.0 Power Consumption

In some applications, the device (complete communication system including PHY and MAC) should operate while supporting a battery life of months or years without intervention; whereas others may require a battery life of tens of hours due to the nature of the application and/or physical constraints on the size of the devices. For example, cardiac defibrillators and pace makers have a lifetime of more than 5 years whereas swallowable camera pills typically have lifetime of 10 hrs.

Ultra-low power operation is very crucial for longevity of implanted devices. In some applications energy scavenging techniques may be employed which may alleviate the need for a battery.  Also the standard should consider inductive recharging of the batteries if necessary for application. This may be practical only over a few inches from body surface to implanted device.

It is common for low duty cycle devices to shut down radio and CPU resource for most of the time. In a typical wireless system idle listening and overhearing consumes a significant amount of power.  Efficient and flexible duty cycling techniques are required to minimize idle listening, overhearing and control overhead.  And efficient power saving (sleep) mode is desirable, in particular for low duty cycle devices that transmit sporadically. In addition the coordination of nodes should not induce frequent wake up of nodes. These mechanisms should be supported by the MAC and PHY layers.

The maximization of whole network life time should consider the different power constraint of nodes in the piconet.

# 9.0 Coexistence and Interference Resistance

BAN devices shall co-exist with other BAN devices and legacy devices. The devices may need to operate in an interference environment by having attributes to deal with interference ingress (interference coming into the PHY layer) and interference egress (interference caused by the PHY layer). The attributes may be adjusted by higher layer (above PHY layer) management.

The devices must be able to operate in high noise, high multipath and dynamic environment. The PHY layer must be able to sustain an appropriate level of co-channel and out-of-band interference. Both medical application in hospital, small clinic, healthcare center and home have to be considered, along with wearable entertainment applications.

MAC should support simultaneous co-located operation of multiple BANs in crowded places such as subways, hospital ward, music concert etc.  A fair bandwidth sharing among collocated BANs and graceful degradation of service is highly desirable.  Medical applications can be given higher priority than entertainment applications when bandwidth is scarce.

# 10.0 Form Factor

The PHY components should be capable of fitting into a form factor consistent with wearable and implant applications. The critical point is that this generally includes the battery and the antenna parts.

# 11.0 Antenna

Non-directional antenna is the nominal requirement. The antenna(s) should be specifically designed to operate in a body centric environment and handle the issues resulting therein, such as propagation through and around the body.  Other factors such as body movement, body shadowing, size, MR safety etc should be taken into consideration.  In medical applications, the antennas (devices) may be put where is best for underlying application, which is not always the best for radio wave propagation. Besides, the antennas for implant applications are covered by tissue compatible material.

# 12.0 Complexity

Complexity should be minimal to enable mass commercial adoption for a variety of cost sensitive products. Complexity (gate count, die size) and BOM should be minimized. In a number of applications, the components are to be considered as throwaway after use.

# 13. Mobility

Nodes should be capable of reliable communication when in the move. It is admitted that limited communication performance (e.g. data rate) can be tolerated in such cases. The considered applications may involve body movement (e.g. twisting, turning, sitting, walking, running etc.) which induce channel fading and shadowing effect. Doppler effect is out of the scope.

# 14. Specific Absorption Rate (SAR)

At the frequencies of operation of most wireless devices, the known health effects centre around tissue is heating. BAN devices are quite low power and there is not enough power available for whole-body SAR to be concerned. Even with knowing that, since the device may be in close proximity to, or inside, a human body, the localized SAR could be quite large if all the available power deposited in a small volume. The localized SAR into the body must be minimized.  BAN devices shall comply with international or local SAR regulations.

# 15. Regulatory Matters

The transmit spectrum mask (measured by EIRP or ERP) at PHY layer shall comply with necessary geopolitical or regional regulations.

Devices shall comply with the regulatory requirements specified for the chosen frequency band by competent authorities.

# 16. References