IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Proposals at a High Level |  |
| Date Submitted | 15 November 2007 |  |
| Source | [Clinton Powell] (Freescale Semiconductor) | Voice: [1.480.413.5413] Fax: [] E-mail: [clinton.powell@freescale.com] |
| Re: | PHY System Parameters |  |
| Abstract | This document attempts to summarize at a high level the 2 proposals submitted for consideration in the 4c amendment. |  |
| Purpose | This document is provided in support of 802.15.4c PHY activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Summary of Proposals

Proposal 1	Proposal 2

(MPSK)	(O-QPSK)

Modulation	MPSK (16 pt)	O-QPSK

Data Rate	250 kbps	250 kbps

Coding

Chip Rate	1 Mcps	1 Mcps

PN Code Length	16	16

PN Codes	Non-Integer	Integer

Sym2Sym Cyc. Shift	1 Chip	2 Chips

PER (AWGN)	Same	Same

Crest Factor	~2.87 dB	~1 dB

Meets -36 dBm	Yes	Yes
(Out-of-band Rejection)

Flat Fading Perf.	Same	Same

Freq. Selective Fading	Nearly Identical	Nearly Identical

PPM Perf.	No Error Floor	No Error Floor

Meets Chinese Reg.	Yes	Yes

Provides Common Sync,
Modulation & Chiping Seq.	No	Yes
Across 700, 800 & 900 MHz

Scope from PAR:
This amendment defines alternate PHY and modifications to the MAC needed to support the PHY that complies with the applicable Chinese regulations, Radio Management of P. R. of China doc. # 6326360786867187500 or current document, for one or more of the 314-316 MHz, 430-434 MHz, and 779-787 MHz frequency bands.