IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | PHY System Parameters |  |
| Date Submitted | 15 November 2007 |  |
| Source | [Clinton Powell; Kuor-Hsin Chang; Michael Schmidt, Phil Beecher] (Freescale Semiconductor, Inc; Atmel, Integration) | Voice: [] Fax: [] E-mail: [clinton.powell@freescale.com kuor-hsin.chang@freescale.com michael.schmidt@hno.atmel.com] |
| Re: | PHY System Parameters |  |
| Abstract | This document specifies the minimum set of parameters for which all proposed .4d alternative sub 1-GHz center frequency PHY’s will be evaluated. It is expected that these parameters are included in the proposal. |  |
| Purpose | This document is provided in support of 802.15.4d PHY activities. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

PHY SYSTEM PARAMETERS

| Spreading Scheme | Specify if used and characteristics (chip rate, pn codes, hop rate etc |
| --- | --- |
| Modulation Scheme | Specify, if not constant envelop |
|  | needs to specify the Crest Factor |
| P(e) - PER curve | AWGN, 20byte PSDU + hdr (SHR+ PHR) for 0ppm, +/-20ppm, +/-40ppm |
| Flat Fading | PER Curve for Delay spread = 0 20byte PSDU + hdr for 0ppm |
| Delay Spread RMS | PER Curve for Delay spread = 250nS 20byte PSDU + hdr for 0ppm, (per. doc 15-04-0585-00-004b) |
| Sync Header | (preamble, SFD), Codes and Length |
| PSD Mask | Show PSD relative to in and out of band mask1pk. |
| Bit Rate, Chip Rate | Specify |
| Synchronization | State Method (ideal or simulated) |
| Transmit Power | Maximum power still meeting requirement of PSD mask |
| Susceptibility to RFID Interference (TBD) | Show device’s ability to tolerate interference from other devices operating in the band e.g. RFID reader/writer. Show a figure a merit to show ability to reject interference from coexistent devices |

Modulation Scheme

One important parameter to characterize the modulation scheme is the crest factor.

Crest Factor:

The crest factor of the complex baseband signal x(t) is defined as

Ccrest=maxx(t)Ex(t)−Ex(t)2

i.e., a value characterizing the peak-to-average ration. Constant envelop signals have a crest factor close to 1.  A low crest factor usually simplifies the design of the power amplifier.

Transmit PSD Limits

The transmitted spectral products shall be within the limits specified in Table 1. For the relative/absolute limit, average spectral power shall be measured using a 100 kHz resolution bandwidth. Also, the reference level shall be the highest average spectral power measured within ± 600 kHz of the carrier frequency.

Table 1 PSD limits

| Channel aggregation | Frequency | Relative limit | Absolute limit |
| --- | --- | --- | --- |
|  | TBD | TBD | TBD |

Out of Band Spurious Emission Requirements

The spurious emission requirements are the maximum transmit emissions allowed outside the band. The spurious emission requirement is -39dBm