IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3c Call for Poposalss |  |
| Date Submitted | [18 January 2007,Revised on 12 March 2007] |  |
| Source | [Ismail Lakkis] [Tensorcom] [10875 Rancho Bernardo Rd., San Diego, 92127] | Voice: [+1-858-231-9753] Fax: [+1-858-678-0300] E-mail: [ilakkis@tensorcom.com] |
| Re: | [P802-15-3c-DF0-Draft-Ammendment] |  |
| Abstract | [Proposed changed to the MCS in the HSI PHY] |  |
| Purpose | [Notice of Request for formal CFP submissions to the P802.15.3c Millimeter-wave based Alternative PHY Task Group] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

### Data field rate-dependent parameters

The outer code is an RS(252,236) code (when used) and the inner code is an LDPC(672,K) code with coding rates of 1/2, 3/4 and 7/8.

The OFDM base and optional header rate-dependent modulation parameters are listed in Table 2. The base and optional frame headers are QPSK modulated. The base frame header is encoded with an outer RS(42,26) code and an inner LDPC(672,336) code. The optional frame header is encoded with an outer RS(98,82) code and an inner LDPC(672,336) code.

The base data rate shall be 59 Mbps, the base header rate shall be 39 Mbps and the base optional header rates shall be 46 Mbps.

Sampling rate = 2592 MHz only.

Table 1 – Data field rate-dependent parameters

| M C S | Data Rate (Mbps) | Modulation scheme | Coding mode | Outer FEC rate (RO) | Inner FEC rate (RI) |  | Spreading factor (Lf) | Spread & coded bits/sym. (NSCBPS) | Coded bits/sym. (NCBPS) | Data info* bits/sym. (NIBPS) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MSB 8b | LSB 8b |  |  |  | MSB 7:4 | LSB 3:0 |
| 0 | 59 | QPSK | EEP | 0.94 | 1/2 |  | 24 | 672 | 28 | 14 |  |
| 1 | 708 | QPSK |  | 0.94 | 1/2 |  | 2 | 672 | 336 | 168 |  |
| 2 | 1416 | QPSK |  | 0.94 | 1/2 |  | 1 | 672 | 672 | 336 |  |
| 3 | 2124 | QPSK |  | 0.94 | 3/4 |  | 1 | 672 | 672 | 504 |  |
| 4 | 2478 | QPSK |  | 0.94 | 7/8 |  | 1 | 672 | 672 | 588 |  |
| 5 | 2832 | 16-QAM |  | 0.94 | 1/2 |  | 1 | 1344 | 1344 | 672 |  |
| 6 | 4248 | 16-QAM |  | 0.94 | 3/4 |  | 1 | 1344 | 1344 | 1008 |  |
| 7 | 4956 | 16-QAM |  | 0.94 | 7/8 |  | 1 | 1344 | 1344 | 1176 |  |
| 8 | 6372 | 64-QAM |  | 0.94 | 3/4 |  | 1 | 2016 | 2016 | 1512 |  |
| 9 | 1512 | QPSK | EEP | 1 | 1/2 |  | 1 | 672 | 672 | 336 |  |
| 10 | 2664 | QPSK |  | 1 | 7/8 |  | 1 | 672 | 672 | 588 |  |
| 11 | 4536 | 16-QAM |  | 1 | 3/4 |  | 1 | 1344 | 1344 | 1008 |  |
| 12 | 1770 | QPSK | UEP | 0.94 | 1/2 | 3/4 | 1 | 672 | 672 | 336 | 504 |
| 13 | 2301 | QPSK |  | 0.94 | 3/4 | 7/8 | 1 | 672 | 672 | 504 | 588 |
| 14 | 3540 | 16QAM |  | 0.94 | 1/2 | 3/4 | 1 | 1344 | 1344 | 672 | 1008 |
| 15 | 4602 | 16QAM |  | 0.94 | 3/4 | 7/8 | 1 | 1344 | 1344 | 1008 | 1176 |

* Reed Solomon encoded bits when RS is used.

Table 2 – Header data-dependent parameters*

| Header Rate (Mbps) |  | Modulation scheme | Spreading factor | Coded bit/sym. | RS encoded info bits/sym. | Number of occupied OFDM symbols |  | Number of pad bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base header | Optional header |  |  |  |  | Base Header | Optional header |  |
| 39 | 46 | QPSK | 24 | 28 | 14 | 24 | 64 | 0 |
| NA** | 492 | QPSK | 2 | 336 | 168 | NA | 6 | 224 |
| 936 | 984 | QPSK | 1 | 672 | 336 | 1 | 1 | 224 |

** Base header encoded with an outer RS(42,26) and an inner LDPC(672,336), and the optional header is encoded with an outer RS(98,82) and an inner LDPC(672,336).

*Not applicable

Figure 1 – Data field encoding process

Figure 2– Data field forward error correction