IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Request for Interpretation Worksheet |  |
| Date Submitted | 16 January, 2008 |  |
| Source | [Pat Kinney] [Kinney Consulting LLC] [Lake Zurich, IL] | Voice: [+1 847 960 3715] E-mail: [pat.kinney@ieee.org] |
| Re: | [A committee has been formed, headed by Pat Kinney, vice chair of 8802.15, to respond to a Request to Interpretation as per IEEE rules.] |  |
| Abstract | [The following submission is a call for participation, applications and proposals from the IEEE, Industry, and Academia.] |  |
| Purpose | [Request for interpretation of the IEEE 802.15.4-2006 standard] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Request for Interpretations:

- Submitted by Phil Beecher, Integration UK Ltd; 12 November 2007

Section 7.5.8.2.3 Incoming security processing. Sections c) and f) appear to be contradictory. How should the receiver behave when a frame is received with Security Enabled field set to 1 but when the security level in the frame is set to 0? Is this covered in c)? If so, what does f) refer to?

- Submitted by Phil Beecher, Integration UK Ltd; 12 November 2007

We have been looking at the PHY section of 15.4-2006.  Section 6.1.2.2 Channel pages references phyPagesSupported in the PHY PIB, but I cannot find it in the PIB. Was this an omission? Did it ever get given an attribute ID value?

- Submitted by Phil Beecher, Integration UK Ltd; 8 January 2008

I think there is another problem with 15.4-2006, this time concerning association.  The spec describes the behaviour when a device disassociates, also when the coordinator refuses the association request, but it does not describe what should happen if a communication error occurs during the association process, causing the process to fail. I believe this was an oversight in the spec and that the behaviour should be described.

- Submitted by Phil Beecher, Integration UK Ltd; 9 April 2008

Please could you clarify the behaviour of the MAC in the following situation?

A device is participating in a beacon-enabled network, tracking the beacon of its parent and generating its own beacon.  It receives a beacon from its parent in which the Superframe structure is changed in such a way as to cause the device’s outgoing beacon characteristics to knowingly violate the parent Superframe.

1) Should the device stop sending its own beacon?

2) Inform the next higher layer (NHL) that it has stopped, and if so how?

I think the answers to both these questions should be "yes", and that the device should inform the NHL by generating an MLME-SYNC-LOSS.indication with status of SUPERFRAME_OVERLAP.  This status enumeration was added in 15.4-2006 as a status for the MLME-START.confirm message to cover the condition described when it occurs during the MLME-START.request.

Committee members:

- Patrick Kinney (Kinney Consulting LLC)

- Phil Beecher (Integration Associates)

- Robert Cragie (Jennic)

- Klaus Meyer (Atmel)

- Kuor-Hsin Chang/Clint Powell (Freescale)

- Monique Brown (Motorola)

- Art Astrin (Astrin Radio)

Interpretation Meeting

14:42 17 March

Item 2

Mistake was made in the drafting, inadvertently an editor didn’t update the PIB, should be the channels supported not attribute PhyPagesSupported.  Clint proposed that the resolution is to change PIB name to PhyChannelsSupported.

Chair recessed the meeting for 5 minutes to allow the group to research the issue of item 3

Item 3

Phil explained that the association request primitive

Meeting adjourned

Resolutions

Item 1)

No change required:

Step a) in the procedure identifies the case where the Security Enabled subfield is set to zero and the security level thus becomes zero. It is this security level which applies throughout the rest of 7.5.8.2.3 except in the case of c), i.e. the case where the Security Enabled subfield is set to one. In this case, the procedure ensures that the Security Level subfield of the Security Control field is not zero - if it is, it rejects the frame at that point. So the apparently contradictory case in f) only applies when the Security Enabled subfield is set to zero.

This text could perhaps be clearer but this is the result of trying to write a logical procedure in something resembling English whereas a language with proper control flow constructs would have been more appropriate

Step f accepts an unsecured, "non bastardized" frame, provided that the recipient device does no expect a frame with "real" protection applied.

Step c deals with filtering out (rejecting) the bastardized case where one supposedly switches on protection (by setting the Security Enabled bit), but then "protecting" the frame with the "null" security level, i.e.., no confidentiality, no data authenticity.

Thus, Step c and Step f are not contradictory: Steps b-d just deal with bastardized cases, while the "real" processing starts at step e.

Item 2)

Change required:

The PHY PIB name used on page 29 in the second line of Clause 6.1.2.2 should be corrected to read as "phyChannelsSupported". Both the pages and channels supported within that page can be obtained via this attribute and provides the information that a "phyPagesSupported" PHY PIB Attribute would.

Item 3)

Change required:

15.4-2006 Page 181 section 7.5.3.1 Replace: “If the Association Status field of the command indicates that the association was unsuccessful, the device shall set macPANId to the default value (0xffff).”

with

“If the Association Status field of the command indicates that the association was unsuccessful, or, if there is a communication failure during the association process due to a missed acknowledgement or the association response command frame is not received, the device shall set macPANId to the default value (0xffff).”

Item 4)

Change required:

The device should stop sending its own beacon and inform the next higher layer by generating an MLME-SYNC-LOSS.indication with status of SUPERFRAME_OVERLAP.

MLME-SYNC-LOSS.indication primitive definition must be expanded to include this case.

SUPERFRAME_OVERLAP must be added to the list of possible enumeration values for the LossReason parameter of the MLME-SYNC-LOSS.indication primitive.

SUPERFRAME_OVERLAP definition must be slightly modified.

Note:

This status enumeration was added in 15.4-2006 as a status for the MLME-START.confirm message to cover the condition described when it occurs during the MLME-START.request.