IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Routing issues for HR |  |
| Date Submitted | Wednesday, March 19, 2008 |  |
| Source | [Youngae Jeon, Seonghee Lee, and Sangsung Choi] [161 eajeong-dong,Yuseong-gu, Dajeon, 305-350, Korea] [Sung-Woo Park] [161 Kajeong-dong,Yuseong-gu, Dajeon, 305-350, Korea] | Voice: [+82 42-860-6497] E-mail: [yajeon, slee003, sschoi@etri.re.kr] Voice: [+82 42-860-6722] E-mail: [swpark@hnu.kr] |
| Re: | [] |  |
| Abstract | [This document contains proposed changes to the IEEE P802.15.5 Draft to address a routing algorithm for HR-Mesh.] |  |
| Purpose | [Reselution for WG LB38] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Figure A. Server routing with the aid of MC

Server routing without the aid of MC

Routing Issues

- Mesh header format

| Octets: 4 | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Routing assistant | TTL | Mesh seqnum | Destination DEVID | Destination PNID | Source DEVID | Source PNID | Mesh frame control |
| Mesh header |  |  |  |  |  |  |  |

Mesh header format (old)

| Octets: 1 | 2 | 1 | 2 | 1 | 2 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TTL | Mesh sequence number | Destination DEVID | Destination PNID | Source DEVID | Source PNID | MeshID | Mesh frame control |
| Mesh header |  |  |  |  |  |  |  |

Mesh header format (new)

| Bits: b7-b5 | b4-b3 | b2-b0 |
| --- | --- | --- |
| Reserved | Frame type | Transmission method |
| Mesh frame control |  |  |

Mesh frame control subfield format (old & new)

Valid values of frame type

| Type value b4 b3 | Frame type description | Subclause |
| --- | --- | --- |
| 00 | Data frame | Error: Reference source not found |
| 01 | Command frame | Error: Reference source not found |
| 10-11 | Reserved |  |

Valid values of transmission method and routing assistant field format (old)

| Value b2 b1 b0 | Transmission method description | Routing assistant field |  | Subclause |
| --- | --- | --- | --- | --- |
|  |  | Octet: 2 | 2 |  |
| 000 | Full broadcast | n/a | n/a |  |
| 001 | Tree broadcast | n/a | n/a |  |
| 010 | 2-hop routing | n/a | n/a |  |
| 011 | Tree routing | Source TREEID | Destination TREEID | Error: Reference source not found |
| 100 | Server routing | n/a | n/a | Error: Reference source not found |
| 101 - 111 | Reserved | Reserved |  |  |

Valid values of transmission method and routing assistant field format (new)

| Value b2 b1 b0 | Transmission method description |
| --- | --- |
| 000 | Full broadcast |
| 001 | Tree broadcast |
| 010 | Tree routing |
| 011 | Server routing |
| 100 - 111 | Reserved |

- Mac Header Change for IMM_ACK

| 1 | 3 | 1 | 2 | 1 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Stream index | Fragmentation control | SrcID | Source PNID | DestID | Destination PNID | Frame control |
| MAC Header |  |  |  |  |  |  |

Figure 6 - MAC header format (new)

- Mesh CAP

|  | CAP | MCTA | … | … |  | CAP | MCTA | … |  | CAP | CMCTA | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MC |  |  |  | … | B |  |  |  | C |  |  |  |

If the PNC is a mesh capable PNC, the CAP can be extended to the Mesh CAP.